FAX2You - Fax receive option

The FAX2You option might be archived into an archive called

FAX2U.EXE.  Go into the OS2You directory and enter the command

A:FAX2U to unpack this archive.

The FAX2You option is an add-on to the OS2You/PM2You product that

you may license at additional 300 SEK (see included order form).

It allows you to receive, print and view faxes with OS2You if you

have a faxmodem.

General description

With the Fax receive option OS2You can use a class 2 faxmodem both

to receive data (on-line) calls and fax calls from fax machines.

If a data call is detected OS2You will prompt for user ID and

password as usual.  If a fax call is detected OS2You will receive

the fax and print it.  Thus you can use the same modem and phone

line to receive both data and fax calls.

To use FAX2You fax receive option you need a class 2 faxmodem and

a HP LaserJet (PCL4 or PCL5), Epson FX or Epson 24-pin printer.

The fax receive option is always installed and just needs configu-

ration to work.  The fax option works with both class 1 and class 2

faxmodems, but only the class 2 specification defines a standard for

detection between data and fax calls which is necessary for OS2You

to work.  Thus a class 2 faxmodem is recommended, although some

class 1 faxmodems may have an option for detecting fax calls.

The fax receive option is configured by the standard OS2You SETUP

program and by editing the FAX.CMD file with an ordinary text

editor.  With the OS2You SETUP program you edit the modem profiles

to initialize the faxmodem for detecting both data and fax calls.

With class 2 faxmodems this is done by adding the following strings

to your modem initialization string:

~AT+FCLASS=2^

~AT+FCR=1^

~AT+FAA=1^

~AT+FDCC=1,5,0,2,0,0,0,0^

~AT+FLID="Your fax-ID"^

In addition to defining the above initialization string, you should

set the Fax connect string to +FCON and disable the Release fax

device setting.

As a reference, you might look at the ZyXEL 1496E+ modem profile,

which is already correctly configured for this option (like the

Supra and other ZyXEL modem profiles).

The initialization string above will set the modem to accept both

data and fax calls.  If a fax call is detected the command file

FAX.CMD will be started.  This command file should be edited to

correctly start the fax receive program (FAX2You).  The significant

line of the FAX.CMD file is:

FAX2You -l %%%1 -connected -print -pcl

This line will start the fax receive program FAX2You and print

received files files with HP LaserJet printer format.  OS2You can

work together with other fax receive programs from other vendors,

but we recommend using the FAX2You program.  The FAX.CMD file

contains some sample lines for invoking other fax receive programs

as well.

The FAX2You program can be started with the following parameters:

-l <COMport>    Specifies serial port to use.  If set to

%xx COM-handle xx (decimal) will be used.

If set to %AUTO a COM-handle will be scanned.

Default AUTO.

-b <baudrate>   Specifies serial port baudrate, default no

change

-connected      Will instruct FAX2You that the call was

answered by a separate program (like OS2You).

-pcx            Produce PCX-file output

-dcx            Produce DCX-file output

-class1         Use Class 1 commands, default Class 2

commands

-print          Print received faxes

-epson          Use Epson FX printer format

-epson24        Use Epson 24 pin printer format

-pcl            Use PCL printer format, default

-pdev <device>  Use <device> for printer output, default PRN

The FAX2You program will maintain a log file named FAX2You.LOG with

statusreports from fax connections.  All faxes will be saved in

separate files, which can later be printed or viewed on screen with

separate utilities (not available yet).  The fax files are created

with names derived from the date in the format MMDDnnnn.APF

(MM=Month, DD=Day, nnnn=counter).  Note that these files are never

deleted and can be quite large.  Therefore you should have some

routine to delete old fax files whene not needed any more.