This page covers notes for PrintGL  1.30  that are not included in the

v1.28 manual.

The A suboption of /Y turns on PrintGL's  ADI handler.  In this mode

Autodesk  ADI 4.0 vector plotter  binary  commands  are  interpreted

(along with HP-GL). This lets you use 15 pens with AutoCAD.

The /FU driver has been removed and replaced by /FL!.

/F@ supports the Canon BJC880.

/F& is a native mode driver for the Canon BJC800/820 and BJ200.

/FD+!  supports the  Canon  LBP 8 Mk 4 at 600 dpi.  The !  suboption

switches  from  blank  area  data   compression  to   CaPSL  4  data

compression.

/F4+!  supports the IBM 4029 LaserPrinter with 600 dpi PPDS upgrade.

Adding  the !  suboption  to  /F4  switches  from  blank  area  data

compression to IBM 4029 data compression.

HP-GL output allows up to 24 pens.

Six additional shading patterns are available, A is 6.2%, B is 3.1%,

C is 1.6%, D is 87.5%, E is 62.5%, and F is 37.5%.