SATVIEW

Environmental Satellite Image Viewing and

Processing Program

Copyright 1990 - 1994

Graphics Software Technologies

Orange, CA

TABLE OF CONTENTS

=================

1.   SATVIEW   Page

Introduction   4

Operating System Requirements 7

Program Installation     7

Configuration  8

Program Disclaimer  11

Program Registration     11

Ordering SATVIEW    12

For More Information about SATVIEW 13

Special Images 13

Capturing Your Own Images     13

General Program Notes    16

Menu Organization   21

2.   Appendices

A.   Supported Video Boards and Printers     22

B.   Parameter Configuration File: SATVIEW.CFG    24

C.   Enhancement Curve File: SATVIEW.ENH     27

D.   Editor    28

E.   Correction History  31

F.   Available Images on CD ROM    33

INTRODUCTION

============

SATVIEW was written for the purpose to being able to view large

HRPT NOAA satellite images. These image files are typically 40

megabytes in size for all five channels.

While in the process of developing the program we had a number of

helpful individuals test the program and propose many additional

features which have been added to the first release, taking its

development well beyond its original scope.

Here are a few of the capabilities:

Complete interactive menu driven commands

On-line documentation for all menu items

Mouse and keyboard input

View images up to 200 megabytes in size

View many different image formats:

HRPT five channels

HRPT single channel

Vegetation index

APT images from A&M board

APT images from Bartlett board

Any binary format up to 7500 pixels wide

Propriatary formats

Meteosat formats A and B and LX

HP scanner images

HF FAX images

RST images

DMSP images

SPOT Image Corporation, GIS format, 7-1/2  quad

Supports SVGA cards with the following resolutions:

640 x 400 x 256

640 x 480 x 256

800 x 600 x 256

1024 x 768 x 256

Supports over 50 printers including laser and color printer.

Ability to write GIF and PCX images and write GIF format images

Filtering using convolution matrices for:

softening

gentle sharpening

hard sharpening

Four user defined filters

On-screen histogram display

Printed output for histogram

Zooming in and out of image

Panning around image

Brightness control

Contrast control

Add text annotation to any image in any of eight colors and

three text sizes

Use of function keys for often used commands

Quickly remove menus for viewing the full screen image

Place menus on left or right side of screen

Ehancement curves:

Histogram equalization

NOAA enhancement curves

User defined enhancement curves

View file as normal or inverted (ascending or descending)

Offset APT and VGA images

Noise removal

Image negation

Cut a sub-image

Rotate the image file

Create a color vegetation index image

Geometric correction of HRPT images

Can shell out to other capture programs

Can shell out to DOS

Viewing the current palette

Interactive on-line help

View and save any size of GIF file

User defined palettes

Create HRPT image from other channels by an equation

Cut any size image from existing displayed image

It is our desire to promote the hobby and profession related to

satellite image capture, viewing and processing. Providing

SATVIEW to amatures, educators, professionals and government

agencies at no other cost, other than for shipping and printing of

manuals, is one way we can meet this goal.

SATVIEW is a professionally written program and is supported by

frequent updates. Ideas from all users are therefore necessary to

make SATVIEW the most comprehensive program available for

viewing and processing of satellite images.

The authors particularly wish to thank all those who have assisted

with testing, ideas, and hardware for capturing and processing

images. In particular we wish to thank John Williams, John DuBois,

Steven Padar, and Rick Fogle and Alan Bundens of Quorum

Communications. We are especially indebted to NOAA and its staff

for their willingness to share information with the user community.

For those who wish to see more HPRT images and a description of

weather types and patterns, we suggest that you purchase a copy of

the book "View From Low Orbit", by Tom Loebl. The books is

filled with photos and costs only $16.95 in the US. You can order a

copy by just writing to: Image Publishing, P.O. Box 66,

Hubbardston, MA  01452 and enclosing $16.95.

OPERATING SYSTEM REQUIREMENTS

=============================

The minimum system configuration which will give adequate

processing times and fast viewing are listed below:

386 or 386/SX compatible computer

1 Megabyte of main memory

Math coprocessor

80 Megabyte harddisk drive

Super VGA or VESA card with 256 colors

Color monitor

Microsoft or compatible mouse

SATVIEW will work better with the following additional items:

2 Megabyte of memory

Laser or color printer

200 Megabyte harddisk drive

PROGRAM INSTALLATION

====================

First create a sub-directory called SATVIEW. Change to that

directory and copy the entire diskette to this directory by typing the

following:

COPY A:*.*

Insert the second and then third diskettes and copy them as well by

typing:

COPY A:*.*

Then un-zip the program and utilities by typing:

MAKESV

You should have the following program files when you are finished:

Executable files

----------------

SV.EXE

SVCONFIG.EXE

Support files

-------------

SATVIEW.EDT

SATVIEW.CFG

SATVIEW.ENH

SATVIEW.EQN

SATVIEW.PAL

SATVIEW.HLP

Demonstration Image files

-------------------------

GRANDCYN.SAT

OREGON.SAT

HURRCANE.SAT

SALTLAKE.SAT

SANFRAN.SAT

CONFIGURATION

=============

SATVIEW can be configured for different graphic cards,

resolutions and printers. To configure SATVIEW use the

configuration program:

SVCONFIG.EXE

You will be able to choose the graphics card which is in your

computer system and set the desired resolution. Once configured

there is no need to change the setting unless you change graphics

adapters or printer.

You will be asked to specify you graphics card, the graphics

resolution desired, the printer type, printer resolution, printer port,

and printer orientation.

A list of the currently supported graphics cards and printers are

listed in Appendix "A".

When completed, the configuration program then displays the

current configuration:

Configuration complete...

Selected devices are:

-------------------------------------------------

Display   =    Orchid Pro Designer II

Resolution     =    640 x 480

-------------------------------------------------

Printer   =    HP LaserJet +/II/III/IV

Printer Port   =    LPT1

Resolution     =    300 dpi

Orientation    =    Landscape ==

-------------------------------------------------

Restart the SATVIEW program with new configuration

SATVIEW can be run from a directory. In order to do this you

will need to do two things. First add the directory in which you

keep SV.EXE, to your AUTOEXEC.BAT path command. For

example, if you put SV.EXE in directory C:\SATVIEW, your

PATH may look like this:

PATH=C:\;C\:DOS;C:\SATVIEW

Secondly, you will need to add a SET command to your

AUTOEXEC.BAT file which tells SATVIEW where to find its

support files. The SET command will be of the form:

SET SATVIEW=C:\SATVIEW

where C:\SATVIEW is the directory containing the following files:

SV.EXE

SATVIEW.CFG

SATVIEW.ENH

SATVIEW.GEO

SATVIEW.LIB

SATVIEW.EQN

SATVIEW.PAL

SATVIEW.HLP

SVEDIT.OVL

GRANDCYN.SAT

OREGON-V.SAT

The image directory, defined in SATVIEW.CFG and decscribed in

Appendix "B", contains images with the following file name

extensions:

SATVIEW        .SAT Special SATVIEW and generic 8 bit binary format.

Binary         .BIN Any binary file.

APT            .APT An APT image

Meteosat       .MET Image from ASD board.

HRPT single    .CHn Single channel image.

HRPT complete  .HRP Combined 5 channel images.

Veg Index      .VEG Vegetation index image.

GIF            .GIF A GIF image.

Geometric corr .GEO Geometrically corrected HRPT.

SPOT GIS       .BIL A SPOT Image Corp GIS format image

(also *.HDR and *.CLR)

This histogram output file, HISTOGRAM.DAT, will also be located

in the image directory.

PROGRAM DISCLAIMER

==================

Graphic Software Technologies makes no warranty or guarantee,

expressed or implied, as to the accuracy, reliability, functionality,

suitability for a particular purpose, or results derived therefrom.

Nor shall distribution or licensing of this program constitute any

such warranty or guarantee. No responsibility or liability is assumed

by Graphic Software Technologies or its authors in connection

therewith.

PROGRAM LICENSING AND REGISTRATION

==================================

SATVIEW is a copyrighted program and is licensed to an

individual or a company. It is intended to only be used on a single

computer. Additional copies can be obtained from Graphic

Software Technologies or their agents. The licensing fee is $35 for

non-commercial users. Commercial users must contact the authors

or their distribution agents for a commercial license form.

SATVIEW is available on 5 1/4, or 3 1/2 inch high density

diskettes.

ORDERING SATVIEW

================

Orders for SATVIEW can be placed with:

Graphics Software Technologies          Quorum Communications

P.O. Box 3064                           8304 Esters Rd., Suite 850

Orange, CA  92665                       Irving, TX  75063

(714) 998-2900                          (800) 982-9614

(714) 998-6790 FAX                      (VISA or MasterCard)

Vanguard Labs                           R. Myers Communications

196-23 Jamaica Ave.                     P.O. Box 17108

Hollis, NY  11423                       Fountain Hills, AZ 85269-7108

(718) 468-2720                          (602) 837-6492

(VISA or MasterCard)                    (VISA or MasterCard)

FOR MORE INFORMATION OR HELP WITH SATVIEW

=========================================

Any questions can be directed to the authors via:

Mail (also for SATVIEW orders):

SATVIEW

P.O. Box 3064

Orange, CA  92665

Compuserve:         73417,1055

Voice telephone:    (714) 998-2900

FAX:                (714) 998-6790

SPECIAL IMAGES

==============

If there are images which you would like to view, but do not have

capability to receive, then you may wish to consider obtaining them

from the authors.

The SATVIEW CD ROM, described in Appendix "F", contains

nearly 400 GIF images, forty HRPT images, Meteosat images, a

SPOT panchromatic image and other images. The CD is available

from Graphics Software Technologies or any vendor selling the

SATVIEW program for $45 plus $5 shipping.

MORE INFORMATION ON CAPTURING YOUR OWN IMAGES

=============================================

Anyone thinking of capturing HRPT images must consider the two

board set from Quorum Communications, (817) 488-4861. For

APT images consider Quorum, Vanguard Labs, the Bartlett board

available from David Schwittek, or the board from Jerry Dahl. Both

Quorum and Vanguard carry excellent receivers for APT reception.

In addition, there is a non-board system consisting of a device

which plugs into a serial port on any IBM PC or clone that allow

receiving APT, WEFAX and HFFAX (shortwave) images. This low

cost, entry level system is available from Software Systems

Consulting.

Quorum Communications                     R. Myers Communications

8304 Esters Rd., Suite 850                P.O. Box 17108

Irving, TX  75063                         Fountain Hills, AZ 85269-7108

(800) 982-9614                            (602) 837-6492

(VISA or MasterCard)                      (VISA or MasterCard)

Vanguard Labs                             David Schwittek

196-23 Jamaica Ave.                       1659 Waterford Road

Hollis, NY  11423                         Walworth, NY  14568

(718) 468-2720                            (315) 986-2719

(VISA or MasterCard)                      (VISA or MasterCard)

Software Systems Consulting

615 S. El Camino Real

San Clemente, CA  92672

(714) 498-5784

(VISA or MasterCard)

Processed images, programs and satellite elements can be

downloaded from several BBS's including the following:

DRIG      (214) 394-7438 (19,200 baud)

NA/RIG    (909) 590-4382 (2400 baud)

Celestial (513) 427-0674

Quorum    (817) 421-0228

SSC       (619) 259-5554

Vanguard  (718) 740-3911

GENERAL PROGRAM NOTES

=====================

SATVIEW was written to view and process images. The integrity

of the original image is always maintained. SATVIEW uses and

object oriented database approach to image processing. The image

is loaded into memory and displayed on the screen. Certain actions

take place on a copy of the original image creating a new image.

Examples include, cutting an image, rotating, and spliting an HRPT

image. Other actions take place only in the temporary image

displayed on the screen. Examples include brightness, contrast,

filtering and noise removal. These temporary changes only become

permanent when you save the screen image to a SAT, GIF or PCX

file.

Because of this object database concept, there is an order of

processing which works best when enhancing an image. This order

can be generally outlined as follows:

View the full image

Zoom to the desired area and zoom level

Cut or rotate image

Adjust brightness and/or contrast

Apply filter or other enhancements

Add annotation

Save image

You cannot rotate all 5 channels of an HRPT image.  You should

cut out the channel you want and then rotate it if necessary.

Loading an Image

================

SATVIEW images are easily loaded onto the screen for viewing

and processing. The file loadind is done through the FILES +

LOAD IMAGE selection. Catagories of image types can be

selected and the corresponding file names will be displayed. Simply

click on the desired file name and then click on Load Image or hit

the Enter key. You may enter a file name with an extension other

than the default image types by clicking on the File Name button

and then entering the complete filename with extension.

On-line Documentation

=====================

SATVIEW makes extensive use of on-line help documentation

which is available at any time either the main or secondary menus

are displayed on the screen. HELP is made active by hitting the F1

key. At this point an initial HELP message is displayed. Click on

any button that you would like to know more about. The HELP

window will display information about that command. You can

scroll up or down by using the UP or DOWN arrow keys or by

clicking on the up or down arrows on the HELP screen. Exit the

current HELP screen by clicking on the CANCEL button, by

pressing Q or F1. Press F1 onece again to bet back to SATVIEW

processing mode.

Memory Requirements

===================

SATVIEW will either use extended, expanded or lower memory. It

first looks to see if there is extended or expanded memory.

Optimum extended or expanded memory is 2 Mb. If there is no

high memory available it will load and run in lower memory. It

needs about 540 Kb in lower memory to run. If there is not enough

memory for the program you may have to remove existing TSR's to

allow SATVIEW to run correctly.

Function Keys

=============

F1  - Toggles the HELP screen

F2  - Removes first and second level menus for viewing the full

screen image

F3  - Repaints the image. This is useful for restoring an incorrectly

filtered image

F4  - Resets image palette. Useful for restoring image after incorrect

brightness or contrast

F5  - Switches menus from left to right. Only works at level one menu

(first menu)

Exiting an Option

=================

Any command or menu can be exited by typing the ESC key. This is

equivalent to pressing the second mouse button. To get out the the

program you will need to answer Y to the question " Do you want

to quit?".

Hot Keys

========

Each button has a "hot key"  which is a single keystroke that will

select the corresponding button. This is an alternate way to select a

button using the keyboard.

Version 1.00 VGA Image Compatibility

====================================

Version 2.00 of SATVIEW users a different palette for the image

and annotation capabilities. If you view a *.VGA image with Version

2.00 there will be areas displayed incorrectly. Use the CONV-100

program to convert a Version 1.00 VGA image into a Version 2.00

image.

Version 2.00 VGA Image Compatibility

====================================

You must rename and 2.x SATVIEW images from *.VGA to

*.SAT. SATVIEW 3.x will recognize the older file type

automatically.

Mouse Usage

===========

SATVIEW now figures out whether your mouse is on COM1,

COM2, or a bus mouse automatically. Nearly all mice are now

supported.

Error Messages

==============

Error messages are displayed in understandable terms.

APPENDIX A

==========

Supported video boards and RESOLUTIONS

SATVIEW automatically figures out what graphics card you have installed. If

you are using a board which support the VESA standard you must load the

VESA driver before you configure or run SATVIEW.

AST VGA Plus   ATI VGA Wonder ATI VGA Wonder Plus

ATI VGA Integra     ATI VGA Charger     Cardinal VGA 700

Genoa SuperVGA 5300 Genoa SuperVGA 5400 Genoa SuperVGA 6000

Micro Labs VGA Micro Labs Ultimate Orchid Designer VGA

Orchid Designer 800 Orchid Pro Designer IIs  Orchid Pro Design Plus

Paradise 1024  Paradise Professional    Paradise VGA Plus 16

Perfect View   STB VGA Extra  Tseng Labs ET-3000

Tseng Labs ET-4000  Trident TVGA 8800   Trident TVGA 8900

Video-7 VRAM VGA    Video-7 V-RAM VGA   Video-7 VRAM II

Western Digital 1024     Western Digital Prof     Western Digital Plus 16

VESA - Any Board

The current supported resolutions are 640x400, 640x480, 800x600, and

1024,680

Supported Printers

CalComp ColorMaster+     Canon LBP-8/8III       Canon BJ 800/830/20

Canon BJ BJC-800/830     Canon BJ-130 Inkjet    Epson LQ/SQ/Action

Epson LQ/SQ Color        Epson EPL-6000         Epson EPL-7000/7500

Epson GQ 3500            Epson ActionLaser      Fujitsu 24C   B&W

Fujitsu 24C  Color       Fujitsu 24D   B&W      Fujitsu RX 7200/7300

Fujitsu PrintPartner     Fujitsu 7100 PS        HP Laser +/II/III/IV

HP LaserJet IV (600)     HP Laser Postscript    HP DeskJet (all)

HP DeskJet 500C/550C     HP PaintJet XL300      HP PaintJet B&W

HP PaintJet Color        HP QuietJet            HP DesignJet 7600

HP DesignJet 7600 C      Kodak Ektaplus 7008    Kodak Color 4 B&W

Kodak Color 4 Color      IBM Personal 2390      IBM ExecJet

IBM Laser 6p/10p         Kyocera a-Si FS-1500   Lasermaster 1000/1200

Lasermaster Truetech     Mitsubishi 300PS       Mitsubishi CHC-S446i

Okidata Laserline        Okidata OL 810 LED     Okidata OL 830 PS

Okidata 2410/2350        Okidata 2410/2350 C    Panasonic 24 Pin

Panasonic KX-P4410       Panasonic KX-P4430     Panasonic Postscript

QMS Postscript           Sharp JX-9500H         Sharp Postscript

Tandy PL 950             Toshiba 24 Pin  B&W    Toshiba 24 Pin Color

Toshiba GX200/GSX400

APPENDIX B

==========

Parameter Configuration File - SATVIEW.CFG

c:\sv     * pathname for image files

640 * binary file width

0 * length of binary file data header

2400 * APT default width (A&M=4800)(Bartlett=2400)

0 * low level noise cutoff

255 * high level noise cutoff

0  1  0  * \

1 -3  1  * -> processing mask    /// Mild smoothing

0  1  0  * /

0     *    mask mode (0 = no averaging,  1 = averaging)

-1 -1 -1  * \

-1  9 -1  * -> processing mask    /// High pass filter #1

-1 -1 -1  * /

0     *    mask mode (0 = no averaging,  1 = averaging)

0 -1  0  * \

-1  5 -1  * -> processing mask    /// High pass filter #2

0 -1  0  * /

0     *    mask mode (0 = no averaging,  1 = averaging)

1 -2  1  * \

-2  5 -2  * -> processing mask    /// High pass filter #3

1 -2  1  * /

0     *    mask mode (0 = no averaging,  1 = averaging)

Default Image File Pathname

This pathname is used as the default path, however it can be

overridden any time when reading or saving an image.

Binary File Width

This value defines the default binary image file width. The

user will be asked what width the image is each time a

binary file is loaded. By simply hitting ENTER this default

value will be used. Sample widths used by the program and

other binary files are:

RST files (T.S. Kelso)    767

FAX file (A&M board)     2400

SCANNER                  2550 for 300 dpi

VEG (vegetation index)   3867

DMSP                     1024

Length of the Binary File Data Header

Occasionally the binary file has a header containing some

information that is not a part of the image. By setting this

value to the size of the header SATVIEW will skip this

portion of the file.

APT Default Width

This sets the default value for the image width of the APT

image. For the A&M board the suggested value is 4800 and

for the Bartlett board the value is 2400.

Low Level Noise Cutoff

This is the low level (black) default value used when

removing noise. It can be easily overridded when removing

noise.

High Level Noise Cutoff

This is the high level (white) default value used when

removing noise. It can be easily overridded when removing

noise.

Filtering

There are two predefined and four user defined image

filters. Four user definable filters are included in the

configuration file. All six filters use a convolution matrix to

perform the filtering. The two predefined filters are for

smoothing and gentle sharpening.

The user may define their own additional filters. Each filter

consists of a 3x3 matrix and a mode. The center value of the

matrix represents the pixel to be modified. The mode

determines whether the applied value to the pixel will be

averaged or not. A mode value of 0 means there will be no

averaging and a value of 1 indicates that the value is to be

averaged by the total of all neighboring pixel values.

In the smoothing filter all the neighboring pixels will be

multiplied by 1, and the pixel in question will not even be

taken into account (multiplied by 0). The resulting value will

be averaged by dividing by 8 (the sum of the multiplier).

APPENDIX C

==========

Enhancement Curve File - SATVIEW.ENH

This file contains two user defined enhancement curves in addition

to the fifteen internally stored curves. The first three curves are the

default, linear and logarithmic curves. The next thirteen are the

NOAA curves.

In the user defined file each curve begins with a comment line,

beginning with an "*". The next line is the curve name for the pick

box. The maximum length of this name is three characters. The

remaining lines of data are the coordinate pairs defining the

substitute values which will be built into a lookup table and applied

to the image. Below is an example:

* User Defined Enhancement curve

U01

0   0

51 127

255 255

Enhancement curves defined in the GOES/SMS User's Guide:

Enhancement curve Ab     Hurricane curve

Enhancement curve Bb     Hurricane pattern recognition

Enhancement curve Ca     Cold cloud tops

Enhancement curve Cb     Terrain "hot spots"

Enhancement curve Db     Convective activity

Enhancement curve Eb     Middle clouds and convective activity

Enhancement curve Fb     Hydrology curve

Enhancement curve Hb     Lower cloud levels

Enhancement curve Mb     All seasonal clouds

Enhancement curve Pb     Pacific coastal upwelling

Enhancement curve Sa     Sea surface temperature

Enhancement curve Za     Lower and upper level clouds

APPENDIX D

==========

SATVIEW.CFG Editor Commands

____________________________________________

SATVIEW contains a built-in full screen editor. The purpose of the

editor is to quickly and easily edit the SATVIEW.CFG file.

When you have completed editing by either quitting or saving a file

and exiting, you will be returned to SATVIEW. When you save the

file before returning to the program, the editor automatically saves

a backup file with the extension ".BAK".

______________________________________________________

TEXT EDITOR COMMANDS

______________________________________________________

Key  Equivalent Key      Command

---  --------------      ----------------------

MOVE CURSOR:

Ctrl-S    Left-arrow     left one character

Ctrl-D    Right-arrow    right one character

Ctrl-E    Up-arrow       up one line

Ctrl-X    Down-arrow     down one line

Ctrl-A    None           left one word

Ctrl-F    None           right one word

Ctrl-Q C  Ctrl-PgDn      moves to end of file

Ctrl-Q R  Ctrl-PgUp      moves to beginning of file

Ctrl-Q S  Home           moves to beginning of line

Ctrl-Q D  End            moves to end of line

Ctrl-Q E  Ctrl-Home      moves to top of screen

Ctrl-Q X  Ctrl-End       moves to bottom of screen

Ctrl-J L  Alt-L          moves cursor to given line number

SCROLL:

Ctrl-Z    F7             down one line

Ctrl-W    F8             up one line

Ctrl-C    PgDn           down one screen

Ctrl-R    PgUp           up one screen

DELETE AND INSERT:

Ctrl-G    Del            deletes character at cursor

Ctrl-Y    F1             deletes current line

Ctrl-T    None           deletes word right

Ctrl-N    None           inserts a line before cursor

BLOCK:

Ctrl-K B  F3             marks beginning of block

Ctrl-K K  F4             marks end of block

Ctrl-K C  F5             copies block to cursor position

Ctrl-K V  F6             moves block to cursor position

Ctrl-K Y  Alt-F1         deletes block

Ctrl-K R  Alt-R          reads block from external file

Ctrl-K W  Alt-W          writes block to a file

Ctrl-Q B  None           moves cursor to top of block

Ctrl-Q K  None           moves cursor to bottom of block

FIND AND REPLACE:

Ctrl-Q F  F9             finds a string

Ctrl-L    F10            repeats a find

Ctrl-Q A  Alt-F9         finds and replaces a string

Options:

FIND and REPLACE:

U    ignore case, find upper and lower case

B    search backwards from the current position

W    whole words only

L    search block

REPLACE only:

G    replace all occurances in file

N    replace without confirmation

if confirming:

Y    yes, replace

N    no, leave unchanged

A    replace all from current position

Q    quit replacing

FILE:

Ctrl-K Q  Alt-Q          quits without saving

Ctrl-K X  Esc            saves file and exits editor

Ctrl-K S  None           saves file and remains in editor

MISCELLANEOUS:

Ctrl-Q L  F2             restores line to original contents

Ctrl-V    Ins            toggles insert/overstrike mode

Ctrl-U    None           abort command

Ctrl-Q U  None           undo last deletion

APPENDIX E

==========

Correction History

Version 1.00  -  first released on 6/1/91

Version 2.00  -  first release on 10/25/91

1. added GIF capabilities

2. Added VI to GIF

3. Can remove menus for viewing by hitting F2. Reinstated by F2 again.

4. Fixed random problem associated with file names on file selection screen.

5. Added cursor control keys for file menus.

6. Fixed image save one-key controls (VGA & GIF).

7. Create geometrically corrected images from single channel HRPT.

8. Added geometrically corrected file input.

9. Cleaned up code. Removed unused functions.

10. Added HRPT capture.

11. Added Meteosat capture.

12. Added Meteosat file conversion.

13. Added a user command/command-line option.

14. Added ability to edit SATVIEW.CFG.

15. Fixed contrast reset problem with BIN images.

16. Added warning if rotated file name is same as current image.

17. Fixed Enhance button toggle problem.

18. Added read GIF.

19. Restores existing image after Files/Load/Cancel.

20. Added text annotation (colors, justification, size)

^X = up arrow    ^Y = down arrow    ^Z = right arrow    ^[ = left arrow

21. Added normal/inverted image viewing to all image formats.

22. Fixed binary and APT image offset.

23. Added a button on file load for changing binary and APT image offset.

24. Fixed "Edit CFG" for letter E.

25. Changed the return after loading an image to main menu.

26. Removed the "files not found" messages.

27. Maximum image line increased to 8700 pixels.

28. Added SPOT GIS 7-1/2 quad sheet image viewing. Reads size and palette.

29. Maximum image line increased to 12,000 pixels in binary mode.

30. Fixed contrast divide-by-zero error.

31. Added ability to place menus on left or right side of screen.

32. Negate histogram problem corrected.

33. Sharpen filter fixed for occasional white pixel problem

34. CONV-100.EXE for converting Verson 1.00 VGA images to new format.

35. Changed annotation arrows: ^u = up, ^d = down, ^l = left and ^r = right.

36. Defined function keys:    F1 = (future)  F2 = View entire image

F3 = Reload image   F4 = Reload palette

F5 = Left/Right menus

37. Changed "fat-buttons" to regular buttons.

38. ESC now escapes or cancels from every menu and command.

39. Remove unnecessary CANCEL buttons so 640x400 image shows all menus.

Version 2.20  -  first release on 10/10/92

1. Added fifteen more display drivers.

2. Fixed Trident problem.

3. Added Mouse Port configuration for COM1 and COM2.

4. Minor changes to opening screen.

5. Fixed menu problem when printing.

Version 3.00  -  first release on 2/1/94

1. Completely rewritten

2. Interactive HELP system with F1 key

3. Ability to read GIF images of any size

4. NOAA enhancement curves are now built in

5. Automatic sensing for graphics adapter

6. Uses extended or expanded memory

7. Process AVHRR bands by equations

8. Separate simplified configuration program

9. Added the SATVIEW file format (*.VGA, *.BIN or binary)

10. Eliminated *.VGA file type

11. Added ability to read USGS DOQ file types

12. Updated Super VGA and VESA drivers

13. Added over 50 printers

14. Automatically detects mouse on COM or bus

15. Added user defined palettes

16. Mixed bugs from 2.x versions

17. Added PCX screen save format

18. Can cut an image into any horizontal or vertical size

19. Print screen is 5 times faster

20. Display screen is faster

22. Added undo button to annotation

APPENDIX F

==========

SATVIEW SATELLITE IMAGES

We have produced a CD ROM containing hundreds of images of

the earth. Many of these are the full data set and contain up to five

channels of different wavelengths. Images are from the NOAA

polar orbiters, GOES, Europe's Meteosat, Russia's Meteor, China's

Feng Yun 2 (no longer working) and the very high resolution SPOT

Image satellite.

These images are being made available to help promote interest in

weather and environmental satellite images. This large collection of

images of many different types and geographical areas will allow

you to better understand what types of images can be received on

relatively inexpensive home built or purchased equipment. The

SPOT images cannot be received by amateurs.

These images were for the most part were received by individuals

with interest in weather and environmental satellite images. Some

of these individuals are:

Tracy Lenocker   Ed Murashie    John Dubois

Ross Biggar      Roger Beale    Steve Padar

Tom Loebl

The SPOT image on the CD ROM titled PANCHRM2.BIL was

supplied by SPOT Image Corporation and is copyrighted by them.

Any use of this image, other than to be viewed by the SATVIEW

program requires the expressed written permission of SPOT Image

Corporation. The SPOT image may not be copied from this CD

ROM disk. For information on purchasing SPOT panchromatic or

multi-spectral images please contact SPOT Image Corporation at

1897 Preston White Drive, Reston, VA  22091-4368, (703) 620-

2200.

If you copy images off of the CD ROM (except for the SPOT

image which may NOT be copied) you may need to reset the file

attributes since the attributes for a CD ROM are read only.

The CD is $45.00 plus $5.00 for shipping in the US or to Canada.

International shipping is $10.00 US. GST only accepts checks or

money orders. You may purchase via Master Card of Visa from any

one of the companies listed below who carry all of our products:

Graphics Software Technologies     Quorum Communications

P.O. Box 3064                      8304 Esters Rd., Suite 850

Orange, CA  92665                  Irving, TX  75063

(714) 998-2900                     (800) 982-9614

Vanguard Labs                      R. Myers Communications

196-23 Jamaica Ave.                P.O. Box 17108

Hollis, NY  11423                  Fountain Hills, AZ  85269

(718) 468-2720                     (602) 837-6492