TCP to Serial

What it does...

TCPtoSer waits for a TCP connection on a specified port (for example the Telnet port 23). Or, it connects to any port of a host.

After that, any data from that connection is forwarded to the serial port and vice versa. If the data can´t be sent to the serial port immediately, it will be buffered dynamically.

What you can do with it...

There are probably many different things you can do with it.

I can only give some examples of use:

Remote modem connetion: You´ve two computers in your local TCP/IP network. One of them has got a modem, and you want to connect, for example, to a mailbox via that modem from the other computer.

Use TCPtoSer on the modem computer, and connect to it with Telnet. If you want to transfer files from and to the mailbox, you should use ZOC. ZOC (which is normally a terminal software) allows you to use for example the ZMODEM protocoll via a Telnet connection. It works, really! I´ve testet it.

Remote printing: You´ve got three computers in your local TCP/IP network, two Windows machines and one other machine (for example a Macintosh). One Windows computer is in another room, and a serial printer is attached to it. You want to print some documents (or pictures) created on your Macintosh.

Use TCPtoSer on both Windows machines. Connect the Macintosh to the Windows PC next to it using a serial cable. Make a TCP/IP connection between the two TCPtoSer, and use the standard printer driver for your printer on the Mac. The Mac gives the data to the first PC via the serial line, the first PC sends them to the second PC via TCP/IP, and the second PC gives them directly to the printer. It works, really! I´ve testet it.

The settings for a HP510 are: 57600,8n1,CTS/RTS.

Remote UNIX printing: You´ve got a UNIX machine and a Windows PC. A serial printer is attached to the Windows PC. You want to print some documents created on the UNIX-Machine.

Use a Perl script. There is an example script in TCP2Serial, the counterpart of my TCPtoSer for Macintosh computers. Please ask Peter Lewis, <peter.lewis@info.curtin.edu.au>.

What it doesn´t do...

Well, it doesn´t emulate a serial port. So, for example, you can´t take a simple modem program, and communicate with a modem remotly. That is, because the simple modem program doesn´t know how to send its data via TCP/IP. It tries to connect directly via the serial port. You can use another computer to get the data from that serial port and send it via TCP/IP (see Remote printing).

What it needs...

A free serial port and Winsock 1.1 is needed. TCPtoSer runs on Win95 and WinNT platforms.

What it costs...

The registration costs 20 Dollar (or 35,- DM in Germany). You may use this program without registration for a period of 30 days. After that, it will stop working and you MUST register. The file "Register.doc" describes how to register.

You may use the registration key for two own TCPtoSer installations, if you use both installations together. So you won´t have to buy two registration keys if you are interested in the "remote printing" example. You may not give the registration key to other people.

Shareware note...

All copyrights to TCPtoSer are exclusively owned by the author - Carsten Gottbehuet.

Anyone may use this software during a test period of 30 days. Following this test period of 30 days or less, if you wish to continue to use TCPtoSer, you MUST register.

The TCPtoSer unregistered shareware version, may be freely distributed, provided the distribution package is not modified. No person or company may charge a fee for the distribution of TCPtoSer without written permission from the copyright holder.

TCPTOSER IS DISTRIBUTED "AS IS".  NO WARRANTY OF ANY KIND IS EXPRESSED OR IMPLIED.  YOU USE AT YOUR OWN RISK. THE AUTHOR WILL NOT BE LIABLE FOR DATA LOSS, DAMAGES, LOSS OF PROFITS OR ANY OTHER KIND OF LOSS WHILE USING OR MISUSING THIS SOFTWARE.

You may not use, copy, emulate, clone, rent, lease, sell, modify, decompile, disassemble, otherwise reverse engineer, or transfer the licensed program, or any subset of the licensed program, except as provided for in this agreement.

All rights not expressly granted here are reserved by Carsten Gottbehuet.

If you do not agree with the terms of this shareware note you must remove TCPtoSer from your storage devices and cease to use the product.

How to use TCPtoSer...

TCPtoSer doesn´t need to be installed. You can just launch it. Use Deinstall.exe to remove TCPtoSer from the system.

In the main window, there are different fields and five buttons at the bottom. Use the preferences button to open the preferences window and check the settings concerning the serial port. You can also enter your registration code there, see "Register.doc" for more information. The program remembers all settings and the last entries in the main window the next time you launch it. Press the "OK"-button to save the settings and to close the preferences window.

Mode

In the main window, you have to decide which mode shall be used: The server mode or the client mode.

In the server mode, the application listens on the port you specify. Clients can connect to TCPtoSer after you´ve pressed the start button. If a client disconnects, TCPtoSer directly restarts the listening. You don´t have to press the start button again. So, you don´t have to supervise that PC.

In the client mode, the application connects to the specified host on the specified port after you´ve pressed the start button.

Status

The status field shows the current state: IDLE, LISTENING, CONNECTING, CONNECTED, RESETTING.

Buffer Size

During data exchange, you can see the current buffer size in the concerning field. Only data from TCP/IP to the serial port is buffered. The other data path doesn´t need to be buffered because of the handshaking.

Transfer Info

In the transfer info field you´ll find three LEDs: The first two ones for the "TCP/IP to serial port" path, the last one for the "serial port to TCP/IP" path. The LEDs will blink if data is exchanged.

STOP

The stop button shuts down the connection and clears the buffer. If you press this button, it´s an irreversible action. The application returns to its idle state.

PAUSE

The pause button will stop sending data to the serial port, and no data will be forwarded from the serial port to TCP/IP. But the receiving from TCP/IP doesn´t stop working, and the buffer isn´t cleared. You can press the pause button again to enable normal data exchange. Nothing should be lost. But some applications (which use, for example, ZMODEM) will probably run into a timeout if no data are received.

Homepage

http://www.aball.de/~gotti/tcptoser.html

EMail

carsten@gotti.aball.de

Additional Software

TCP2Serial is the counterpart of TCPtoSer for Macintosh computers. It works similar, but it just waits for connects and doesn´t have any windows.

Look at  ftp://nic.switch.ch/mac/software/peterlewis/

ZOC is a terminal software, which is normally used to connect to mailboxes. It also offers the capability to connect to hosts via TCP/IP.

Look at   http://www.emtec.com

Ports

Service    Port     Description

FTP          21     File Transfer Protocol - for transferring files

Telnet       23     For logging into an account on a Remote Host

SMTP         25     For sending mail

Gopher       70     Text menu based browser

HTTP         80     WWW protocol - Netscape, Mosaic

POP3        110     Downloading Mail

NNTP        119     Internet Newsgroups

IRC        6667     Internet Relay Chat

Compuserve 4144     Compuserve WinCIM communications

AOL        5190     America Online

MSN         569     Microsoft Network

<none>     1429     I suggest that you use this port for Remote                     Printing