WHAT'S NEW IN VERSION 2.41

Version  2.41  is  basically a maintenance release to correct some problems

discovered in version 2.40:

o  CHIPSET.EXE,  LASTBYTE.SYS:  Added  support  for  the  UNIchip

U4800VLX,  which  is functionally different from the (non-VLX)

U4800  predecessor.  CHIPSET used to identify both as a U4800,

but  unfortunately the low-level physical driver for the U4800

won't  work for the U4800VLX.  CHIPSET now distinguishes these

two  chipsets,  and  a  new low-level physical driver just for

the U4800VLX has been added to LASTBYTE.SYS.

o  HIGHUMM.SYS:  Wouldn't  allocate UMB's even though it reported

that   UMB   memory   was  available.   A  necessary  but  not

sufficient  condition  for  this to occur was that HIGHUMM had

"split"  (placing  most of its code in R/O memory, Bank-Switch

Memory,  or  the  HMA), and could be corrected in version 2.40

by  use  of  the "/NOSPLIT" command line option.  This bug has

been  corrected,  and  the  command  line  option is no longer

needed.

o  CHIPSET.EXE:  The Headland/G2 HT113 was incorrectly identified

as a HT103.  Corrected.

o  LASTBYTE.SYS:   Corrected   bugs  in  the  low-level  physical

drivers  for  the HINT CS8001 and CS8005, and the OPTi 82C463,

82C499,  82C801,  and  82C802.   The  bug  in  the 499/801/802

driver  was mismatched PUSH and POP instructions, which caused

the  driver  to  hang on initialization.  No problems had been

observed  with  the  other  chipsets  listed, but I found that

they  contained  code  that  should  have  been protected by a

PUSHF/CLI...POPF sequence; this protection has been added.