TN210_10.DOC

SYSOP COMMAND LIST PT II

****************************

LOCKING  ROUTES -- There may be times a NodeOp will want to modify the path

quality  value  of a route to a given node.  If  for network management reasons

one wants to isolate one  network from another as  far  as  preventing the flow

of  NODES  broadcasts  from  intermingling,  the  locked  routes  technique  is

preferable over the locked nodes procedure.   If one wants the existence of the

isolated  node  to  be known,  then a combination  of  nodes and routes locking

could be used.  The locked routes  command is:

R PORT [LOCKED NODECALLSIGN] + PATHQUALITY

To unlock the routes use:

R PORT [LOCKED NODECALLSIGN]  - PATHQUALITY

The  process  of  locking  routes or nodes as  explained  here   is  called

"perming."   Perming  is normally done when it is necessary to IMPROVE  network

throughput.  The most common example of perming is when reliable neighbor nodes

have their PATH QUALITY values set HIGHER than the value listed in Parameter 2.

The ROUTES locking command is used for this situation.     If the NODES locking

command  were to be used, then  the  neighbor would  remain in the NODES tables

and therefore be broadcast into the network  even if that neighbor should fail.

In the  early days of networking,  it was discovered many areas were subject to

short periods of enhanced VHF propagation.   When this occurred,  distant nodes

would suddenly be heard by local nodes.     The  dynamic  routing  mechanism of

TheNet  would be tricked into thinking these were new local nodes and broadcast

their existence accordingly.   A short while later when  conditions returned to

normal the network would be confused.   It would take  several  hours  for  the

original routing to be restored.   During this period, users would for the most

part be unsuccessful in connecting to distant nodes since the "correct" path no

longer existed.

Different  solutions to  this problem were proposed.   The technique originally

publicized in 1988 by Doug Everitt, N5DUB (then editor of The NODE) began to be

universally adopted.  Doug recommended the radio path quality parameter be  set

at a value lower than the standard  "192"  typically seen  for VHF radio paths.

He  recommended values for parameter 2 be in the range of 40 - 80.   The NodeOp

would then determine who his reliable neighbors were.    The neighbors are then

locked  to a "standard value" of "192"  by use  of  the  SYSOP  ROUTES  locking

command.

Known as "Reliable Neighbors" routing,  this technique solved network confusion

brought on  by periods  of temporarily  enhanced propagation.    Here is how it

works.    New nodes seen as  neighbors are  automatically assigned a radio path

quality  value  below the broadcast parameter cut-off value.    Since the newly

propagated  node is  not  rebroadcast  into  the  network,  circuit reliability

remains undisturbed.  While application of this technique applies mainly to VHF

simplex  networks,  under some   circumstances it also  may be used with  half,

full-duplex and, isolated LAN nodes.

A reliable neighbor routing

table may look like this:                        SYSOP action:

SONORA:XE2FE-1} Routes:

0 BISBEE:WA7KYT-1 192 24!                      R 0 WA7KYT-1 + 192

0 NOG:KA7TXS-2 192 16!                         R 0 KA7TXS-2 + 192

0 LMN:AK7Z-1 192 34!                           R 0 AK7Z-1 + 192

0 NOGAL:XE2SOC-5 192 5!                        R 0 XE2SOC-5 + 192

0 SVA:N7OO-1 100 1

Although  not a propagation node,  SVA is automatically  defaulted to the "100"

set in Parameter 2 because the path to it is better via BISBEE.    The reliable

neighbors  of BISBEE,  NOG,  LMN  and NOGAL are permed to "192" as shown by the

exclamation point locking symbol.    A by-product of this technique is it gives

users  assurance  the nodes with exclamation  points and  showing  destinations

greater  than  "0"  are solid routes.   The node  default value  of parameter 2

should be the same throughout the system.  The exact value can be determined by

observation  of the greatest number of hops a distant  node propagates in  as a

neighbor.   The default in the  SET210  utility is "100" which is a  reasonable

starting value.

Here is an example of a LOCKED ROUTE for a neighbor that has failed:

SVA5:N7OO-2} Routes:

1 AZ:N7OO-3 245 7

1 AZSE:N7OO-10 245 18

1 CONF:N7OO-5 245 1

1 SVA:N7OO-1 245 19

1 SVALAN:N7OO-4 245 8

1 #SVA6M:N7OO-6 245 4

0 N7DZH-5 192 0 !

In  this  example, HELIO has suffered a power failure and failed.   After a

period  of time  as determined  by the combination  of SVA5's  NODES  broadcast

interval,  parameter 6,  and Obsolescence counter,  parameter 4,  knowledge  of

HELIO  will  ordinarily  go away.  However in this case, HELIO:N7DZH-5 has been

permed  into SVA5's  ROUTES  as a reliable neighbor.   This can be readily seen

by  observing the path quality  value has been fixed at 192 as evidenced by the

exclamation  point  symbol.   By noting the  "0"  in the "number of destination

routes"  column,  we  know  this  is  a  useless  route.   If  and  when  HELIO

comes  back, it's  "destination routes column"  will  show a value greater than

"0."

The  HELIO node has not been  permed into the routes table of SVA5 with the

node locking  command and  thus will not be propagated  during  node broadcasts

until  it  comes  back  on the air again.     With TNPlus, the alias HELIO will

remain  on  SVA5's routes  list until decremented out of the routes table.   At

this  time  it  automatically reverts to the node call-sign,  N7DZH-5 as in the

example above.

ADDITIONAL LOCKING COMMANDS  -   With the  introduction  of  version  2.08,

greater network management and node security  has been  added  with the new R 2

and R 3 locking commands.  These commands apply to both user or node callsigns.

In  the  past,  networks had little  protection from those that would introduce

unwanted nodes into the system.   Neither was there recourse, short of shutting

down the node, of preventing unwanted users from accessing the system.  The R 2

and R 3 locking commands are designed to alleviate this situation.  R 2 locking

works with a specific callsign, or callsign + SSID.   Format is the same as the

R 0 and R 1 locking commands, but in this case:  R 2 CALLSIGN SSID + 0.

Examples for SPECIFIC node/user callsigns:

To lock:   R 2 W1XXX + 0  or, R 2 W1XXX-5 + 0.

To unlock: R 2 W1XXX - 0  or, R 2 W1XXX-5 - 0

An  R 2  application might be to  filter  out a certain SSID from a server with

multiple SSID's.   This would block the  designated callsign - SSID,  but allow

the same callsign with dissimilar  SSID's to pass.

The R 3 locking command controls a designated callsign plus ALL SSID's.

An example:

To lock W1XXX-5 and all SSID combinations:    R 3 W1XXX + 0

To unlock W1XXX-5 and all SSID combinations:  R 3 W1XXX - 0

When locked,  W1XXX is  barred from connecting TO the node,  regardless whether

or not ANY SSID is used.   Callsigns  designated in either of  R 2 or R 3 locks

will not appear in the node's Heard or ROUTES listings.  Attempts to connect to

a locked callsign by a user from the node will be ignored.  In the case where a

node is locked,  if presently in the routes table, it will  remain  connectable

until  decremented to  "0"  by the Obsolescence Count Initializer, parameter 4.

Should a CONNECTED  user be locked,  the lock will not be effective until after

he is disconnected.  Attempts by the user to reconnect will be ignored.

The  R 2  and  R 3   locking commands  are also  effective  on certain types of

aliases, some of which depends whether the Callsign Validation Parameter is set

or not.

RESET  - This is the big panic button.   When  a PC based node goes berserk

and contaminates  the  network  with "garbage nodes and symbols," one can clean

the node  table  by typing RESET.   This  not  only  wipes the NODES and ROUTES

tables,  it  also  wipes  the  softcoded  INFO section.   Stations connected or

networking through the node at the time of the RESET will be disconnected.   To

use, type "RESET."

Within  10 seconds  following  the RESET command,  the node  will broadcast its

existence.   Since knowledge of  previous neighbors and  permed routes has been

lost, this broadcast should not create confusion to the network.    All it does

is introduce itself to it's neighbors.   However, the NodeOp should take prompt

action to restore reliable neighbor routings and INFO messages.