XED version 1.0a heX EDitor

A Unique Hexadecimal/Binary file editor

(c) Copyright 1993-94 David L. Dight

All Rights Reserved.

P.O. Box 280, Kings Cross, N.S.W.  2011  AUSTRALIA

Fax: +61 2 233 2929,  On CompuServe:100033,2563

or the Internet davidd@interconnect.com.au

-+-

User's Guide

Revision 1.0.a.4 16-Jul-1994.

-+-

Contents

========

1. XED - an overview.

2. Features

a) Unlimited file size

b) Modify file length

c) Mouse support

d) EGA/VGA alternate screen modes

e) Undo and Undo all

f) Edit entire paragraphs as strings

g) Bitwise and binary

h) Create compiler readable data definitions

i) Online help, ASCII tables and Text filter

j) Multiple markers

k) Unassembler

3. Command line

4. Movement

5. Editing

6. Other functions and notes

a) Searching

b) Goto offset

c) Save

d) Insert paragraph

e) Copy paragraph

f) Delete paragraph

g) Seven/Eight bit ASCII

h) File information

i) DOS shell

j) Quit

k) Help menu

l) Bitwise operations

m) Binary display/edit

n) Assignment expressions

o) Fonts

p) C data definition creation

q) ASCII tables

r) Text filter

s) Using markers

t) Using the unassembler

7. Program Environment

8. Keyboard Summary

9. Notes, Disclaimer

-+-

1. XED - an overview.

=====================

XED is a simple to use hex file editor. XED permits binary files

to  be  edited in much the same way as normal text files.  Files

are  initially viewed as containing (bytes/16) paragraphs. Bytes

from  these paragraphs are presented on the screen as hex digits

to left, and as ASCII characters to the right. In this document,

the meaning of "line" and "paragraph" will be the same.

Typical  uses include repair  of backup file headers; editing of

string constants in executables; creation or expansion of binary

data  files or  any  other  situations which  require  low-level

modification to a file.

Unlike  many other hex editors, XED is not restricted to editing

the  file  as  it appears on the disk. The file is viewed  as  a

stream  of binary characters. This allows you to edit and change

any  binary file. The only restriction to the size of the edited

file is your hard disk capacity.

XED  starts  in  hex mode. To toggle over to the  ASCII  window,

press  the  TAB  key. The cursor may be moved  with  the  normal

cursor/page  keys.  A  simple reverse video  block  selects  the

current byte which may be edited on screen. You may also edit an

entire  paragraph by pressing the INS key. If you are  in  ASCII

mode,  the current paragraph can be edited as a string.  In  hex

mode, XED converts the paragraph into a hex string (2 hex digits

per  byte).  The  length of the paragraph can  be  increased  or

decreased.

To search, press Shift+F8. Depending on the mode you are in. XED

will  prompt for a hex or an ASCII string. F8 repeats  the  last

search.

To move to  specific  offset press F9. Offsets can be entered in

decimal, hex or octal.

F4  will save the current file to disk and by default, create  a

backup copy of your original file.

-+-

2. Features.

============

a) Unlimited file size

XED  can  edit files as big as your disk storage permits.  By

default,   XED  will  use  extended  memory.   If   that   is

unavailable or full, expanded memory will be used.  If  again

this  is  unavailable or full, XED will use the TMP  or  TEMP

environment  variable to locate the drive  and  directory  to

store  the file buffer on disk. For small files the hard-disk

will  never be required. However for larger files, this  will

be  used.  (A  note: if you use a ramdisk and have  your  TMP

variable  set  to  that  drive, make sure  you  set  the  TMP

variable to your hard disk. XED will need about two to  three

times  the  size of the file in bytes for buffer  storage  on

the disk).

b) Modify file length

XED  permits bytes to be inserted or deleted from any  binary

file.  This  means  that executable files, hidden  files  and

device  files  can be modified. Of course,  you  should  know

what  it  is you are editing before attempting to  do  so.  A

typical  example  may  be  to repair/insert  a  header  in  a

corrupted backup file. Blank paragraphs and paragraph  copies

can  also  be  inserted.  XED displays  a  '+'  next  to  any

paragraph  that is longer than 16 bytes, and a  '-'  next  to

paragraphs less than 16 bytes.

c) Mouse support

XED  can  be  used with a mouse. You can click  on  bytes  to

edit;  scroll the edit window; select paragraphs to be edited

and  select  commands from a menu. The  use  of  a  mouse  is

recommended.

d) EGA/VGA alternate screen modes

XED  will default to the 25 line display. If you have an EGA,

you can  also use the 43 line  display mode. If  you  have  a

VGA,  28, 43 or 50 line modes are also available. See the  -l

option.

e) Undo and Undo all

When  a paragraph has been modified, XED retains the original

contents.  The  paragraph may then be undone  at  anytime  by

pressing  ALT+U.  All  paragraphs that have  changed  can  be

undone as well by pressing ALT+A. XED displays a '!' next  to

any paragraph that has changed.

f) Edit entire paragraphs as strings

An  entire paragraph can be edited as an ASCII string  or  as

an  ASCII string of hex digits. This permits the insertion or

deletion   or  modification  of  any  paragraph,   with   any

character.  The  normal inline editing  functions  are  fully

supported.  Any  ASCII character that is used  by  XED  (e.g.

TAB) can be entered in hex mode.

g) Bitwise, binary and assignment expressions

XED supports a number of bitwise operations when in hex mode,

e.g. shift, rotate and 1's complement.   The current byte can

be viewed and edited in binary (see Editing/binary display).

Simple assignment expressions can be executed on  the current

byte. For example, the following expression can be entered to

switch off the first 3 bits:

[<offset>] &= ~0x7

XED supports the full set of 'C' assignment  operators plus a

few non 'C' operators. See Section 6.n.

h) Create compiler readable data definitions

It is often necessary to convert a binary file into a compiler

readable format. This usually requires a program to be created

to do the task. XED can  do this for  you by creating 'C' data

definitions from any file.

i) Online help, ASCII tables and Text filter

Various help screens are available within XED depending on the

situation.  A Dec/Hex/Oct/Bin/ASCII table is also provided. An

additional feature in these tables displays individual charac-

ter counts for the current file.

XED  allows  you  to  view  the  current  file  through  three

different text filters, which permits easier comprehension  of

data in particular editing situations.

j) Multiple markers

XED permits up to 39 file markers to be set within the current

file. A mark list window  is also  available for  quicker mark

retrieval.  Multiple  markers  can be  very useful  if you are

working on a number of different offsets within the file.  The

mark list permits easy viewing and retrieval of your marks.

k) Unassembler

XED permits data to unassembled for viewing. A feature of this

capability allows you to align starting byte offsets to decode

various  instructions correctly. The 80x86  family opcodes are

fully  supported including true  32 bit,  floating point 80x87

and emulated instructions.

-+-

3. Command line.

================

XED  accepts the name of the file to edit (no wildcards) plus

a number of switchable options.

An  optional response file can be specified. A response  file

is   simply  a  text  file  containing  a  list  of  switches

(including  a filename).  To specify a  response file, prefix

the response filename with the at '@' character, for example:

xed @myopts

You can debug your response file switches by appending the -h

switch to the parameter list.  XED will respond with the help

message, with  your correctly parsed switches  and or  values

listed in the respective switch 'default' values.

If no file or response file is given, XED will search for the

default response file "XED.RSP".  If this  file is present in

the current directory, it will be read.

Response files provide a way  of configuring  XED  yet  still

permit different files to be edited. For example, the default

response file could contain your display or backup options or

other switches you may need, but  not the filename.  XED will

then read the response file and prompt for the file to edit.

The switches are summarised below:

usage: XED [-abdefghijlmnprstuvx7?!] <[@]file>

-a start in ASCII mode, default False

-b make Backup before save, default True

-d force Disk based virtual memory, default False

-e Expert mode, default False

-f use system Font, default False

-g show system characters, default False

-h(help) print this Help message, default True

-i Ignore machine test, default False

-j enable mouse Jump ("snapto"), default True

-l <set screen Lines [25 28 43 50]>, default 25

-m Mono screen, default False

-n biNary display, default False

-p disable Pointing device, default False

-r Read only, default False

-s enable Sound, default True

-t <start in Text filter#>, default 1

-u Display hex bytes using Upper case, default False

-v <resident Virtual memory (Kb)>, default 128

-x <eXecute DOS command on startup>

-7 start with 7 bit ASCII display, default False

-? print Usage message, default False

-! about XED

Notes.

Switches that do not require  a  parameter  can  be  'stacked'

together on one switch (which can be a '-' or a '/'). The last

switch stacked can also be a switch that takes a parameter for

example:

xed -p -f -a -l28 -d myfile.bin

can be regrouped and passed as follows:

xed myfile.bin -pfadl28

The file name can appear any where on the command  line. If a

file  is not  specified, XED will  prompt  for  the filename.

Similarly, if a switch that  requires  a  parameter  is given

without  the parameter,  XED  will prompt you for it.  If the

filename is omitted, XED will prompt for it last.

Notes about each switch follows:

-a    XED  defaults to hex mode on startup. Use this switch  to

start in ASCII mode.

-b    By  default,  XED creates a backup of any file  that  has

been  saved,  in the directory where the  file   resides.

Use this switch to disable backup creation.

-d    Forces  XED to use the disk only for buffering. This  may

be  necessary  if your  extended/expanded memory  is  not

large  enough to hold the  file buffer. XED will normally

detect  this  situation anyway.  Use this switch  if  you

still experience problems.

-e    Disables yes/no prompts with non-trivial operations.

-f    Disable   internal   font.  XED  reprograms   the   video

controller  to  use  a  different text  font.   Use  this

switch  if characters appear corrupted. CGA  and  certain

EGA  adaptors will need this switch.  You may also select

this option if you are examining files  that use some  of

the redefined font characters.

-g    By default, XED uses 32 characters in the  font table for

system  purposes.  These are  not displayed.  This option

forces XED to display these characters.

-h    Prints a switch summary with switch descriptions.

-i    XED  will  only  run  on  PC/AT  or  greater  (386's  and

greater  are  of course ok).  XED will test your  machine

to  see  if it conforms.  On some early systems,  machine

ID's may not be recognised. If this is the case, and your

system is a 286 or greater, use this switch.

-j    By default, the mouse cursor will jump to the exit button

in most windows or to  the scroll characters in  the text

filter and ASCII tables. After leaving a window, XED will

move the cursor to the position it was prior to executing

the command. To disable this, use this switch.

-l<n> If  you  have an EGA, you can use 43 line mode (eg -l43).

With VGAs, you can choose from 28, 43 or 50 line modes.

-m    Force monochrome display attributes.

-n    When the binary display flag is set, the currently selec-

ted byte will be displayed in binary at  the far  left of

the screen (replacing the paragraph offset), see F5.

-p    XED will use the  mouse by default if present.  The mouse

can be disabled with this switch.

-r    The current file cannot be written (read only).

-s    XED  uses  bleeps  when issuing warning  messages.  These

can be disabled with this switch.

-t<n> This switch will force XED to enter the text filter wind-

ow number n (1-3) on starting.

-u    By default, XED displays hex digits (A-F) in lower case.

This  switch  forces upper case.  Hex  offsets are always

displayed using upper case.

-v<n> By  default,  XED will use 128Kb of DOS  memory  for  the

virtual heapspace.  This  can  be  adjusted  to suit your

memory availability (maximum value is 640).

-x<s> Execute DOS command on startup.  The  command  string can

contain  parameters  as  along  as  the  entire string is

enclosed in double quotes.

-7    By  default, XED displays the full  8 bit ASCII character

set. This switch selects 7 bit display.

-?    Print a short usage message.

-!    Print the About XED message.

Return value

------------

XED will return the following values to DOS:

0   -   normal exit, no error.

1   -   either the file was not found or the response

file was not found.

2   -   XED couldn't initialise the argument parser.

3   -   the executable XED.EXE is corrupt.

4   -   insufficient memory to execute.

5   -   XED couldn't initialise virtual memory.

+6   -   _vm_errors + 5 if any.

!!  if you are interested in the command line argument parsing

routine used in XED, please contact me on CompuServe or on

the Internet and I will upload it to you. The routines are

written in C++  and are a collection of  argument  parsing

objects. Compiles in DOS/UNIX.

-+-

4. Movement.

============

The  cursor keys will locate the software cursor anywhere  on

the  screen  in  either hex or ASCII modes. The  screen  will

scroll  one page at a time if up or down are pressed  on  the

first  or  last  lines respectively. HOME/END  moves  to  the

first and last byte on the current paragraph. PGUP/PGDN  move

one  page,  Crtl+PGUP/PGDN will move 10 pages. Crtl+HOME  and

Crtl+END will move to the first and last paragraph.

If  you  are using the mouse, the scroll bar to the right  of

the  screen supports mouse activated movement. If  you  click

on  the  top  or bottom arrows, XED will execute a PGUP/PGDN.

If  you  hold the button down, XED will continue to page.  If

you  click  anywhere  on  the  bar,  XED  will  move  to  the

approximate  position  relative to the  whole  file.  If  you

click  on the bar position just below the top arrow  or  just

above  the  bottom  arrow, XED will execute  a  Crtl+HOME  or

Crtl+END.

The relative offset block in the scroll bar can be picked  up

and  moved  within the scroll bar, moving the current  offset

relative to the whole file.

The UP/DOWN arrows will scroll the page on the window limits.

-+-

5. Editing.

===========

A  reverse  video  software  cursor  points  to  the  current

offset.  The offset is displayed in both hex and  decimal  at

the  top right of the screen. Moving the cursor or page  keys

will  relocate  the cursor. If you are using the  mouse,  you

can  click on any byte to relocate the cursor (if you are  in

hex  mode you can click on a byte in the ASCII window  -  and

vice-versa).  Press TAB  to toggle between  the ASCII and hex

windows.

In  hex mode, you can enter hex digits directly at the cursor

and  in  ASCII, normal ASCII characters can be  entered.  XED

will move  the cursor to  the right after a valid  entry.  To

save the changes you have made to the current paragraph,  you

must  press ENTER. XED will tag the current paragraph with  a

'!'.

If  the current  paragraph  has changed but  has not yet  been

committed (i.e. ENTER has not been pressed),  XED will display

a '' to left of the Hex display.

When in Hex mode, the following bitwise operations are recogn-

ized:

press   <   to shift left by 1 bit

"     L   "  rotate left by 1 bit

"     >   "  shift right by 1 bit

"     R   "  rotate right by 1 bit

"     +   "  increment

"     -   "  decrement

"     ~   "  bitwise NOT (1's complement)

"     `   "  negate (2's complement)

These keys operate on the currently selected byte.

As with any edit, you must press ENTER before moving off  the

line to keep your changes.

When in Hex mode, the current byte can be  edited  in  binary

mode by pressing Shift+F5.  (You  must  first  select  binary

display mode by pressing F5 if you are not in this mode).  If

you are using the mouse, you  can click on the binary display

at the left of the screen to edit in binary.

The cursor  will move to the far left of the screen where the

byte is displayed.  You  may  move the  cursor to any bit and

enter a 1 or a 0.  The bitwise operator  keys described above

are  also active. Mouse users can also toggle individual bits

on or off by  clicking on  any bit.  To save  the byte, press

ENTER (or click the right button). This will exit from binary

mode and take you back to Hex mode. To save any binary  edits

you must press  ENTER again before leaving the line. To abort

binary editing, press ESCAPE.

'C'  like  assignment expressions can be executed by pressing

F6.  After  entering  the  expression,  XED  will  update the

current byte with  the result.  Press  ENTER  to  accept  the

changes. See section 6.n below for more details.

To  edit  an entire paragraph, press INS. If you are  in  hex

mode,  the current paragraph will be translated into an ASCII

string  of  two  digit hex bytes.  This string  can  then  be

edited on the bottom  line of the screen. XED recognises  the

usual  in-line  editing keys (such as DEL,  INS,  HOME,  END,

LEFT,  RIGHT,  BACKSPACE,  and so on).  In  ASCII  mode,  the

string is presented for editing on the last line.  For inline

editing help, press F1 (or click on the '?').

Up  to  80  ASCII or 40 hex bytes can be inserted or  deleted

per  paragraph.  When  in hex mode, two hex digits are needed

for each byte - bytes that require one hex digit only must be

entered with a leading '0'.  To save changes you have made to

the  current paragraph, you must press ENTER. To abort, press

ESCAPE. XED displays  a  '+' if the paragraph is greater than

16 bytes, and '-' if it  is  less.  The  new  length  of  the

paragraph is printed on the bottom line.

If  you  are using the mouse, you can click on the  paragraph

offset at the left of the screen to edit that paragraph.  The

cursor can be located within the string using the mouse.

You  may  return  to  an  edited paragraph  and  restore  its

original   contents   by  pressing   ALT+U.   If   you   then

subsequently  re-edit  the paragraph,  XED  will  retain  the

previously  edited paragraph in the undo buffer.  This  means

you  can  still  lose the original contents! (of  course  you

don't have to save the file).

Inline  editing is available on any operation that takes  the

cursor  to  the  bottom line (this includes  whole  paragraph

edits, search specs and goto  offset). Apart from  the normal

cursor  keys,  backspace and INS/DEL,  XED  recognises  ALT+D

(delete to end of line) and F3 (get last edited string).

All  changes  can be undone by pressing ALT+A. Unfortunately,

this will NOT undo whole paragraph insertions or deletions.

Press  F1  for help or click on the '?' on the top line.  The

help  screen  can also be used as a menu.  Any command can be

entered or clicked on from the menu to execute.

If  you click on the filename, XED will execute F10 file info

function. If you click on the two shutter characters  at  the

top left. XED will exit.

When in any edit  mode, if  the mouse  location when the left

button is clicked is not valid, the  left button will execute

an ESCAPE.  The  mouse buttons default to ESCAPE for the left

button and ENTER for the right button.

-+-

6. Other functions and notes.

=============================

a) Searching

To  search  for  a string or byte, press Shift+F8.  In  ASCII

mode,  you can enter an ASCII string on the bottom  line.  In

hex  mode,  you can enter one or more hex digits.  To  search

again, press F8. You  can  interrupt  a  search  by  pressing

ESCAPE.

To automatically mark  the search find, press Crtl+F8 instead

of F8.  In this  way you  can record  search finds  for later

review.

b) Goto offset

To goto  an offset, press F9. XED will prompt you to enter an

offset. You can enter the value in Decimal, Hex or Octal, for

example:

285053      (dec)

0x4597D     (hex)

01054575    (oct)

You can also goto  a mark by entering the desired mark number

prefixed with a '%'.

c) Save

To  save the current file press F4. XED will create a  backup

of  the  current file by default. To disable this, start  XED

with the -b switch (see above).

d) Insert paragraph

F2  will  insert  a blank paragraph (16 bytes  -  all  nulls)

below  the  current paragraph. Use this to insert  data.  The

length  of the new paragraph can be modified by editing  (see

INS  above).  By default XED will prompt you to  confirm  any

editing  function  such  as insert paragraph.  XED  will  NOT

prompt in expert mode.

e) Copy paragraph

F3 will copy and insert the current paragraph below.

f) Delete paragraph

Shift+F2  will  delete  the  current  paragraph regardless of

whether it has been expanded or  contracted.   XED  will  NOT

prompt in expert mode.

g) Seven/Eight bit ASCII

By  default, XED will display ASCII characters (in the  ASCII

window) as 8 bit characters. To toggle between 7 and  8  bit,

press F7. Some of the low order ASCII characters and all  the

high bit characters will displayed as a period '.'

h) File information

To  obtain information about the current file, press F10. XED

will  display  the file size (both on disk  -  in  bytes  and

paragraphs,  and edit size in bytes), the time/date  of  last

modification,  any  attributes (A=archive  H=hidden  S=system

R=read-only) and the number of changes (if any). A change  is

any  change  to one paragraph. Also calculated and  displayed

is the file CRC32 and the free disk buffer space.

Mouse  users can  click on the  filename on the first line of

the screen to execute this command.

i) DOS shell

To  start  a DOS shell press ALT+X. XED will swap itself  out

to expanded/extended memory or to disk if necessary.

j) Quit

To  exit press ESCAPE. XED will prompt you to confirm. If you

are  in  expert mode, XED WILL NOT PROMPT, EVEN IF  THE  FILE

HAS  CHANGED.  Mouse users can click on the button    at  the

top  left  of the screen to execute this command.  Note:  the

right  mouse  button defaults to ENTER when in  XED  and  can

therefore be used for confirmations.

Shift+F4 aborts XED without  prompting.  You can  abort while

XED is loading a file by pressing 'q'.

k) Help menu

Press  F1 for the help screen. Mouse users can click  on  the

question mark on the top line of the screen. The help  screen

also doubles as a simple menu.  Any command  can  be  entered

from the menu. PGUP/PGDN move between the two help screens.

Mouse  users can  click on   any character  within a  command

to execute that command.

l) Bitwise operations

These are available only in  Hex and Binary edit  modes.  See

Section 5 for more details. To edit in binary, you must first

select binary display mode by pressing F5.

m) Binary display and edit

F5  toggles  the  binary  display.   When  on,  the currently

selected byte will be displayed in binary at the far  left of

the screen (replacing the current paragraph offset).  To edit

the  current byte in binary,  press  Shift+F5 or click on the

binary displayed byte.  Mouse users can  click on  individual

bits in the byte which will toggle the bit on or off.

n) Assignment expressions

To enter an assignment expression, press F6. XED will convert

the current byte to  a simple  assignment expression,  in the

form:

[<offset>] = 0x<hex byte>

for example:

[4CD0] = 0xe7

You  can  only  edit  the  operator  and  value  parts of the

expression, which is printed on  the bottom line for editing.

Use the cursor keys, backspace, etc, to edit  the expression.

You may enter any  of the standard 'C' assignment  operators.

The  right value (rval) must be  an integer  constant, in the

form:

0x??        hex digit(s) (e.g. 0xF6, 0x7)

0???        octal digit(s) (e.g. 045)

?????       decimal digit(s) (e.g. 45)

????????b   binary digit(s) (e.g. 10101010b, 1100b)

and no subexpressions or parentheses are recognised.

The following 'C' assignment operators are recognised:

a = b    simple assignment

a += b   plus equals          (a = a + b)

a -= b   minus equals         (a = a - b)

a *= b   times equals         (a = a * b)

a /= b   divide equals        (a = a / b)

a &= b   bitwise AND equals   (a = a & b)

a |= b   bitwise OR equals    (a = a | b)

a ^= b   bitwise XOR equals   (a = a ^ b)

a %= b   modulus equals       (a = a % b)

a <<= b  left shift equals    (a = a << b)

a >>= b  right shift equals   (a = a >> b)

a <op> ~<val>               1's complement

and the following non 'C' assignment operators:

a l= b    left rotate equals  (lrot (a, b))

a r= b    right rotate equals (rrot (a, b))

a <op> `<val>                2's complement

The 1's '~' complement operator is also  recognised. This can

precede any rval, to 'NOT' the value before assignment. A 2's

complement operator is also available, '`'.  This is used  in

the same way as  the 1's complement operator, and returns the

1's  complement  plus 1  (the same as the 'C' '-' unary minus

(sign change) operator).

When editing  an expression,  you can view a summary  of  the

expression types by pressing F1 (or click on the '?').

Examples:

[<offset>] |= 0x3           (switch on bits 0-1)

[<offset>] &= ~0x3          (switch off bits 0-1)

[<offset>] *= 10            multiply by 10 (dec)

[<offset>] += 1             increment

[<offset>] <<= 3            shift left 3 bits

[<offset>] l= 3             rotate left 3 bits

[<offset>] ^= 10011b        xor with 0x13

[<offset>] = `111b          assign 2's complement of 111b

(=0xF9)

[<offset>] >>= `0376        right shift by the 2's

complement of octal 376 (0xfe)

(=0x2)

o) Fonts

XED reprograms the video  controller to accommodate both  the

use of  special characters  and to emulate the graphics mouse

cursor.  Consequently, certain extended characters  (eg. some

ruling graphics) will not be displayed  in the  ASCII window.

In their place, XED will display a small 'x'. In order to see

these characters, use the -g switch (see section 3).

To force XED to display the default  character  set, use  the

following command line:

xed -pf <file>

which disables the mouse and uses the system font.

The following characters are used by the system:

0x8c 0x8d 0x8e 0x8f 0x90 0x91 0x92 0x93 0x94

0x95 0x9b 0x9f 0xc7 0xc9 0xcd 0xce 0xcf 0xd0

0xd1 0xd2 0xd3 0xd4 0xd5 0xd6 0xd7 0xd8 0xdc

0xf6 0xf7 0xf8 0xff 0x00

p) C data definition creation

To create a 'C' data definition using the current file, press

Crtl+F4. XED will create the definition to the current  file-

name with .HEX extension, in the following format:

typedef struct

{

char paragraph[16];

}

PARAGRAPH;

PARAGRAPH untitled[] =

{

{ "\x4d\x5a\xa3\x01\x11\x00\x05\x00\x20\x00\xe1\x02\xff\xff\x5b\x04" },

{ "\x00\x08\x00\x00\x14\x01\x00\x00\x1e\x00\x00\x00\x01\x00\x22\x01" },

.

.

.

{ "\x4d\x5a\xa3" },

};

For  example,  if the current  file is  MYFILE.BIN, XED  will

create the definition to the file  MYFILE.HEX.  The  file can

then be included in your program as a header file.

Note: the data generated  may cause an  overflow in the  DATA

segment if  the  data  definition  is  too large.  Check that

the  data created  will fit  your target  memory model.  As a

guide, the  generated file will be approx.  4 to 5 times  the

size of the binary file.

If the length  of  the  last  paragraph is  not 16 bytes, the

definition will  still compile to  16 bytes in length, padded

with nulls. You need to be aware of this.  Some compilers may

require pragmas to align the data on 1 byte boundaries.

To assist you to incorporate the  definitions generated,  XED

will generate the following #define statements:

#define FILESIZE <size of file in bytes>L

e.g. #define FILESIZE 12389L

and:

#define PARAGRAPHS (sizeof(untitled)/sizeof(PARAGRAPH))

which corresponds to the number of 16 byte paragraphs defined.

If  the  last  paragraph  is  not  16  bytes in  length,  the

following statement is also produced:

#define BYTES_ON_LAST <bytes on last paragraph>

e.g. #define BYTES_ON_LAST 11

See the file COLOURS.HEX which is an example data definition

that was generated by XED. The program file  MKBIN.C  is  an

example  that  uses  the  definition  file   to   create  an

executable file called COLOURS.COM

q) ASCII tables

To view the ASCII tables,  press Shift+F6.  XED  will display

two columns of 13 bytes. Each byte  is displayed  in decimal,

hexadecimal,  octal, binary and its ASCII character.  You can

page through the table with PGUP/PGDN. Mouse users can  click

on the up/down arrows at the top  right of the window.  Press

HOME/END to go to the start/end of the table.

Note: the ASCII characters displayed will contain some chara-

cters used by XED,  which are replaced with a small 'x'.  See

6.o above.

To see the standard 3 character names for the first 32 chara-

cters, press F1. Mouse users can click on the word  "Asc"  to

toggle the name display.

F2 toggles the  binary/character count fields.  The character

counts are calculated when you run the file info command F10.

Mouse users can click on the words  "Binary"  or  "Count"  to

toggle the display.

Character counts can be useful when analysing a  data or text

file. The counts shown will reflect changes made to the file.

To update the counters, press F10 then Shift+F6.

F3 will resort  the table  from  lowest to  highest character

frequencies.  If you press F3 again,  the table will resorted

back to the default ASCII order. Mouse users can click on the

words "Dec" to toggle the sort.

To exit, press ESCAPE.

r) Text filter

To view the current file through the text filter, press Shift

+F7. The current page is displayed as text lines in a window.

Extended and non-printable characters are  shown as a '.'  By

default, XED filters all characters below 0x20 (32 dec), only

displaying CR (0xd),  LF (0xa), EOF (0x1a) and ESC (0x1b)  by

their string names enclosed in angle brackets, e.g <ESC>.  To

view all the string names (for characters 1 - 32 only), press

F2. To view strictly text only press F2 again.  F2 will cycle

through these three types of filter. Mouse users can click on

the filter number in the window title to execute F2.

The 1 to 32  character  string  name  filter  is  useful  for

viewing raw data  packets or  other communications  data.  To

save  space, the name <NUL> (0) is not displayed  since  this

character is usually the most common in binary files.

XED displays the first character of  each paragraph in  bold,

allowing you to see the relative position  of the text in the

file.

XED will  display all  characters from  the current  page and

possibly some  or  all characters from the next page (to fill

the text window). Characters from the next page are displayed

in GREY attribute.

You can scroll  through  the file with PGUP/PGDN. Mouse users

can click on the arrows at the top right of the text window.

Mouse users can click on any character in the window to go to

the byte. For this to work properly, select filter number 3.

To exit, press ESCAPE.

s) Using markers

To set a mark at the current offset, press Crtl+F9.  XED will

assign the next available mark from a list  of 39.  To remove

this  mark or  any mark, go to the  marker and  press Crtl+F9

again.  To retrieve a mark, press F9 (goto offset)  and enter

the mark number prefixed with a '%', for example:

Enter offset: %3

will go to  marker number 3.  To view  the  mark list,  press

Shift+F9. XED will display each mark slot with the offset and

hex byte at that  offset in bold.  Unused slots  will display

'free'. Press ESCAPE to exit the list.  Mouse users can click

on a marker to retrieve (goto) that mark.

When in the mark list window, you can press DEL to remove all

marks.

To retrieve the next mark, press Alt+F10. XED  will go to the

next  logical  active mark  (the  next  mark after the   last

retrieved or  set).  To  retrieve  the previous  mark,  press

Crtl+F10. This feature is useful  when you wish to work at  a

few different offsets.  Set a  mark at  each offset then move

between them with these command keys.

s) Using the unassembler

The  unassembler  interprets  the 80x86  and   80x87   family

instruction set in both 16 and 32 bit modes.

To unassemble the current file commencing at the current file

offset, press Shift+F10. XED will disassemble as much data as

is  necessary to fill the window. PGUP and PGDN move the file

pointer one paragraph  which, depending  on the  instructions

will decode to on average 10 lines.  Crtl+PGDN/PGUP will move

the file pointer one page (16 * (screen lines - 1)) bytes.

XED will  display assembly  lines whose  offsets  begin  on a

paragraph  boundary in  bold attribute.  This  permits easier

reading of the mnemonics generated.

Clearly much of the data will not unassemble correctly. There

are a number of reasons for this:

a)  The data are not opcodes - they may be program data

or perhaps packed/compressed values;

b)  The unassembler has commenced disassembly at an odd

or incorrect offset;

c)  The opcodes are co-processor instructions, or

d)  The opcodes are invalid.

When the commencement  offset  is incorrect, the  unassembler

will not resolve  instruction sequences properly.  To correct

this problem, press LEFT or RIGHT  which will  move the  file

offset  back  or forward  one byte.   Alternatively, you  can

press ESCAPE to exit the window, reposition the cursor and re

-enter the unassembler.  You will often find the instructions

are now  correct.  This ability  will permit  disassembly  of

packed or  oddly aligned instructions.  Mouse users can click

on the left/right arrows on the title line to realign.

Crtl+LEFT  will set the offset  to the start of the paragraph

and Crtl+RIGHT will set the offset to the end (0xf).

The title line of the window  will display  the current  view

flags. These are:

toggle key            on                      off

F2       '32' assume 32 bit code       '16' assume 16 bit code

F3       'FP' use 80x87 instructions   'EM' use 80x87 emulator

F4       'NS' ignore operand sizes     'SZ' show operand sizes

e.g. mov [0456],0076          e.g. mov word ptr [0456],0076

F5       'ND' ignore jump distances    'UD' show jump distances

e.g. call 4560:0076           e.g.  call far 4560:0076

To view 80x87 co-processor instructions (and not their emula-

tions - the default), press F3.  To view 32 bit instructions,

press F2. To ignore byte/word pointer display,  press F4.  To

ignore the distance of calls/jumps, press F5. Mouse users can

click on the two letter flag tags on the title line to toggle

these flags.

Some instructions when expanded may not fit in the window.

This is usually due to the operand size/distance, e.g.

jmp    far word ptr es:[bx+56]

These lines will end in a small right hand  arrow. To see the

entire instruction sequence, toggle the 'SZ' or 'UD' flags.

In 16 bit mode, offsets are  displayed in  the segmented form

segment:offset, from the start of the  file.  In 32 bit mode,

single 32 bit values are displayed.

When the unassembler  cannot interpret  byte  sequences,  XED

will   display   the  sequence  followed  by  '<invalid>' and

then skip  to the  next byte.  On a few  rare occasions,  the

unassembler  may get confused and print rubbish, which should

be ignored.  The is usually  the result  of multbyte   opcode

sequences when   the second   or  subsequent   bytes  in  the

expected sequence are invalid.

-+-

7. Program Environment.

=======================

XED  stores  each  paragraph separately in a  dynamic  linked

list.  Paragraphs are stored in virtual memory - some in  DOS

memory,  maybe  some  in extended/expanded  memory  and  that

which  doesn't  fit in memory is buffered to disk.  XED  will

attempt  to  retain paragraphs that are 'near' the  currently

selected  paragraph by caching when necessary.  Consequently,

operations local  to  the  current  paragraph will be quicker

than those at distant offsets and overall performance will be

contingent on the capabilities of individual hard drives.

Operations  on  large  files (say 3Mb) that  require  XED  to

traverse the entire buffer may take longer depending  on  the

speed  of  your hard drive. Large files will also  take  some

time  to  read into the buffer as XED reads the  ENTIRE  file

before editing can begin.

XED will use the  TMP or TEMP environment variables to locate

the buffer file.  This  drive and directory  must be able  to

store at least 3 times the size of the file to edit. RAMDRIVE

users may need to reset this variable to their hard drive.

XED  has  been  tested on file sizes exceeding 20Mb  and  has

performed  with  tolerable speed. WINDOWS users  should  find

XED  will  operate  with  no  problems (see  the XED.ICO  and

XED.PIF files included with the release). The  only  possible

interference  may occur  with  file  share  permissions  when

writing to a file.

XED was written in MS C++ 7.00.

-+-

8. Keyboard Summary.

====================

Key                     Command

UP DOWN LEFT RIGHT      Position cursor

< > / L R               Shift/Rotate left/right

-/+/~/`                 Dec/Inc/NOT/NEG byte

ESC                     Quit

ENTER                   Commit changes

Alt+X                   DOS shell

TAB                     Toggle ASCII/HEX

Alt+U                   Undo paragraph

Alt+A                   Undo All

INS                     Edit paragraph

PGUP/DN                 Next/Previous page

Crtl+PGDN/UP            Move +-10 pages

F1                      Help/menu

Shift+F1                About XED

F2                      Insert paragraph

Shift+F2                Delete paragraph

F3                      Copy paragraph below

F4                      Save file

Shift+F4                Abort edit

Crtl+F4                 Save file as 'C' data

F5                      Binary display

Shift+F5                Binary edit

F6                      Assignment expression

Shift+F6                ASCII tables

F7                      Toggle 7/8 bit ASCII

Shift+F7                Text filter

F8                      Search (again)

Shift+F8                Search for...

Crtl+F8                 Search and Set marker

F9                      Goto mark or offset

Shift+F9                Mark list

Crtl+F9                 Set/Remove mark

F10                     File Information

Shift+F10               Unassemble

Alt+F10                 Goto Next mark

Crtl+F10                Goto Previous mark

Inline editing keys

-------------------

Key                     Command

RIGHT / LEFT            Next/Previous character

F3                      Get last command

HOME                    Start of line

INS                     Toggle insert

END                     End of line

DEL                     Delete character

Alt+D                   Erase to end of line

Alt+K                   Kill (erase) line

ENTER                   Accept edits and exit

ESCAPE                  Abort edit

-+-

9. Notes, Disclaimer.

=====================

Users of XED v1.0a must accept this disclaimer of warranty:

"XED  v1.0a IS  SUPPLIED  AS  IS. THE  AUTHOR  DISCLAIMS  ALL

WARRANTIES,   EXPRESSED   OR  IMPLIED,   INCLUDING,   WITHOUT

LIMITATION, THE WARRANTIES OF MERCHANTABILITY AND OF  FITNESS

FOR   ANY  PURPOSE.  THE  AUTHOR  ASSUMES  NO  LIABILITY  FOR

DAMAGES,  DIRECT OR CONSEQUENTIAL, WHICH MAY RESULT FROM  THE

USE  OF  XED  v1.0a. GOOD DATA PROCESSING PROCEDURE  DICTATES

THAT  ANY PROGRAM BE THOROUGHLY TESTED WITH NON-CRITICAL DATA

BEFORE  RELYING ON IT. THE USER MUST ASSUME THE  ENTIRE  RISK

OF  USING  THE PROGRAM. ANY LIABILITY OF THE AUTHOR  WILL  BE

LIMITED EXCLUSIVELY TO PRODUCT REPLACEMENT OR REFUND  OF  THE

REGISTRATION FEE TO REGISTERED USERS."

XED  v1.0a is  a  "shareware program" and is provided  at  no

charge  to  the user for evaluation. Feel free  to  share  it

with your friends, but please do not give it away altered  or

as  part  of another  system. The essence of "user-supported"

software  is to provide personal computer users with  quality

software  without  high prices, and yet to provide  incentive

for  programmers to continue to develop new products. If  you

find  this program  useful and  find that  you are  using XED

v1.0a and continue to use XED v1.0a after a reasonable  trial

period, PLEASE make a registration payment of $15 to DAVID L.

DIGHT.

Commercial users of XED v1.0a must register and pay for their

copies of XED v1.0a within 30 days of first use. Site-Licence

arrangements  may  be  made  by contacting  DAVID  L.  DIGHT.

Anyone  distributing  XED v1.0a for any kind  of remuneration

must  first contact DAVID L. DIGHT at the address  below  for

authorisation.  This  authorisation  will  be   automatically

granted  to  distributors recognised by  the  Association  of

Shareware  Professionals (ASP) as adhering to its  guidelines

for  shareware distributors, and such distributors may  begin

offering  XED v1.0a immediately (However DAVID L. DIGHT  must

still  be advised so that the distributor can be kept  up-to-

date with the latest version of XED v1.0a).

You  are encouraged to pass a copy of XED v1.0a along to your

friends  for  evaluation. Please encourage them  to  register

their  copy if they find that they can use it. All registered

users  will receive a copy of the latest version of  the  XED

v1.0a system. If you do pass on a copy please make  sure  you

include  all  the  system  files  with  documents.  The  file

XED10A.EXE  is  a  self-extracting  executable  archive   and

contains all files with this release.

-+-

The ASP provide  special  guidelines for  author  members  to

follow when they submit their programs for approval.  Some of

these provisions specify the ways in which the program may or

may not notify the user of their registration commitments.

Some methods  used  by  programmers  are not permitted. These

include   demo-crippling,  date-bombs, disabled  or  devolved

functionality. The ASP stipulates that all shareware releases

must be  fully functioning. XED  uses  the minimum  amount of

'coersion' in  the  understanding that for the software to be

evaluated fairly, the user must get a 'fair go'.

The registration fee  is not important.  The actual 'cost' to

develop shareware is very rarely recovered. What is important

is the message that the registation sends to the author.

_______

____|__     |                (R)

--|       |    |-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

-----|   |   |---------------------

|___|___|    MEMBER

-+-