BOXMODEL DOCUMENTATION

Version 4.0

BOXMODEL is a program for investigating  the  performance

of closed-box, vented-box  and  passive-radiator  loudspeaker

systems as  determined  by  Thiele-Small  models.   It  is  a

successor of the BOXRESPONSE program that is  easier  to  use

and offers additional capabilities.

This program is not intended to instruct  the  novice  in

designing or building loudspeakers.  The user is expected  to

have some familiarity with the Thiele-Small models and  their

associated notation and terminology.   If  you  know  nothing

about the models you should start by reading  about  them  in

books such as  "Bullock  on  Boxes"  or  "Loudspeaker  Design

Cookbook," both offered by Old Colony Sound Labs.

1. MACHINE REQUIREMENTS

BOXMODEL is designed for an IBM PC or compatible with  at

least 460K of available RAM,  DOS  2.x  or  higher  and  with

graphics capability.  The program will recognize and use CGA,

EGA, VGA or MCGA video adapters.  I believe it should also  be

usable with any adapter that is capable  of  emulating the high

resolution modes of these adapters, but I have not been  able

to verify this.  Color capability is not necessary, but it is

highly recommended.

Some of the data calculations in the program involve many

floating  point  operations,  so  a  coprocessor  is   highly

recommended.  If you run an 8088 machine with no coprocessor,

you can expect waits of up to three minutes  while  the  data

for four simultaneous graphs is computed.  With  an  8087  in

place this wait drops to about 15 seconds.  To  achieve  this

same time without a coprocessor, you need at least a  16  MHz

386 machine.  The 8087 is available for about $120.

2. PROGRAM OPERATION

The program starts with a title screen.  Hitting any  key

will take you to the main work screen.

You operate the program by a menu and data entry  system.

You can enter information  for up to four systems designated

A, B, C, D.  The current active system, i.e., the one you are

entering data for is  indicated in the lower right hand corner

of the screen.  To switch to data entry for a different system,

use the Files->Show selection to pick the system you want to make

active.  Once chosen, you are entering data for only that

system until you make another choice.  The performance of one

or more of these  systems  is examined by using the  PLOT menu

to  graph  various  system functions.

3. DATA ENTRY

The data entry system consists a  main  menu  from  which

submenus or data windows are accessed.  The main menu is free

form and consists of the items FILES, CONFIG, DATA, PLOT, MISC,

ALIGN, and HELP which  are always displayed.  Main menu select-

ions are made by pressing the RED capitalized letter of the

desired item, or by moving the highlight onto the desired item

with the LEFT and RIGHT ARROW keys and pressing the ENTER key.

Procedure for Data Entry

To enter data, first move the cursor to the data item you

want to change by using the UP ARROW  and  DOWN  ARROW  keys.

Then move into the data field with the RIGHT ARROW key.   The

cursor will move the start of the data item.  Then  overwrite

mode is active and you can simply write  in  what  you  want.

The LEFT and RIGHT ARROW keys  let  you  move  to  any  field

position.   The  SPACE  BAR  and  BACK SPACE   keys   delete

characters.  Once  you  have  edited  to  your  satisfaction,

striking the ENTER key will take you out of the field and you

can move on to another field with the UP or DOWN  ARROW  keys

or move back to the main menu by hitting the ENTER key again.

If you are in a data field and have modified it, but then

decide you want to keep the original value,  hit  ESC.   This

will move you out of the  field  but  preserve  the  original

value.  If you decide you really didn't mean to edit  any  of

the fields in a window, hit ESC rather  than  ENTER  and  the

cursor will move back to the main menu and all of the  fields

in the window will be returned to  the  values  they  had  on

entry.

3. HELP

Help can be summoned for most menu items by  highlighting

the item and then pressing F1.  If there is anything  I  felt

needed to be said about  the  item,  a  window  with  a  help

message will pop up.  On a color system the  window  will  be

green with black text.

4.  MAIN MENU DETAILS

The use of most main menu selections is  clear,  but  the

following items merit additional discussion.

DISPLAY

The current system (A,B,C or D) is always displayed.

Whether or not the current system is a maximally flat

alignment or not is shown.  The Boxf3 item in the right

side middle of the screen is the box cutoff frequency

f3B. This cutoff applies only to the box acting alone,

whether or not there is an equalizer in the  system.

The complete system -3 dB frequency is listed under

Sysf3 and includes the effect of any equalizer.

These cutoff frequencies are calculated by a numerical

procedure that can go awry.  If this happens then usually

the phrase 'Bad value' will appear rather than a number.

Nevertheless, you should be suspicious of any extremely

small number because it probably means an untrapped bad

value. Usually this happens only when you have specified

a  really wild set of parameters.  If you change to a more

reasonable set, a reasonable number should reappear in

this position.  The final item displays configuration of

the current system.

1. FILES

1.1  Show

The program  can  maintain  information  on  up  to  four

different systems in memory, designated A, B,  C  and  D  for

identification.  The particular system being displayed in the

data entry window at any time is indicated in the upper

center of the window frame.  To change to another, select

the Files->Show menu and pick the system who's information

you want to show.  You will then be working with that system

until you change it again using the Files->Show menu.

1.2 dBase

This selection will display a box with the current number of

records available in the database.  You can load any of the

drivers in the database by entering.  Those of you who have

a dBASE compatible database program, can look at the structure

and records.  Currently there are 1048 drivers in the FULL

version of this program's database.  There are drivers from

4.0" to 30.0". There are home hi-fi type drivers, PA type

drivers, and car stereo type drivers.  THE DEMO VERSION OF

THIS PRODRAM CONTAINS ONLY 4 DRIVERS IN THE DATABASE.

Make sure the file DEMO.DBF is in the same directory as

BOXDEMO.EXE otherwise you will get an error.

1.3 Move

This �allows �you �to move the �system �currently �being

displayed into any one of the four stored systems A, B, C or

D.  �Thus, for example,�you can enter driver parameters into

the �displayed system, �and quickly move them to any of �the

other systems.  This cuts down on data entry.

1.4 GPrint

THE DEMO VERSION OF BOXMODEL DOES NOT SUPPORT GRAPHICS

PRINTER DUMPS.

1.5 Print

This selection is provided for machines without a PRTSCR

key.  It will send the system data currently on display

to an attached printer if it is on line.  Otherwise, nothing

will happen.  The window borders may look somewhat strange

if your system doesn't support the IBM higher order chara-

cter set or if you have not run GRAFTABL.COM in certain DOS

versions, but all of the  data  should  be  sent  correctly.

This  PRint SCReen function is not capable of dumping graphics

screens to a printer.

1.6 Exit

Save current state and exit to DOS.

2. CONFIG

2.1 Speaker

2.1.1 Driver

Single - Single driver in current system.

Series Isobaric - Dual drivers wired in series in an

Isobaric Enclosure

Parallel Isobaric - Dual drivers wired in parallel in

an Isobaric Enc.

Series Compound - Dual drivers wired in series in a

Normal Enclosure

Parallel Compound - Dual drivers wired in parallel in a

Normal Enclosure.

2.1.2 Box

Closed - Select current system to be Closed Box Type

Vented - Select current system to be Vented Box Type

PR     - Select current system to be Passive Radiator

Box Type

2.1.3 EqOrder

0  - No active equalization on current system.

1  - 1st order active equalization on current system.

2  - 2nd order active equalization on current system.

3  - 3rd order active equalization on current system.

2.2 Program

2.2.1 Graphs

A     - Plot just System A.

AB    - Plot System A and B.

ABC   - Plot System A, B and C.

ABCD  - Plot System A, B, C, and D.

Enter in any order.

2.2.2 TempPr

The this selection is provided for those who prefer to

use their own values for ambient temperature and

barometric pressure.  The default values are in the

window when the program starts.

2.2.3 Units

Toggle between U.S. customary units and MKS units.

2.2.4 Load

Toggle between using the loudspeakers' voice coil

resistance or its' voice coil impedance.  i.e. When

looking at the maximum power graph, treat the amplif-

ier as a constant voltage source (Re) or as a constant

power source (Zvc).  Use of the Zvc toggle tends to

be a very conservative approach.

3.  DATA

3.1 Driver

Thiele-Small parameters are entered by hand into the fields.

3.2 Box

Vb - Volume of Box for current system

Fb - Vented Box Tuning frequency. (not displayed if Closed)

Vd - Vent Diameter.  (not displayed if Closed system)

Numb Vents - Current number of vents. (not displayed if Closed)

Vp - Acoustic equivalent volume of passive radiator.( if passive

radiator)

Sp - Surface area of passive radiator.(if passive radiator

system)

3.3 Loss

Qas - Absorption Losses of current system

Qls - Leakage Losses of current system

Qps - Port Losses of current system (not displayed if Closed)

3.4 Eq

The Eqdata item allows the entry  of parameters for a high-

pass filter of the chosen order.  Thus, you can examine fifth,

sixth, seventh and eigth order equalized alignments, or simply

decide whether a rumble filter will help performance.

Cr1 - Corner frequency of filter effect.

dmp - Damping Coefficient (reciprocal of Q) for 2nd order

section, dmp of 0.5 = +6.0 dB, dmp of 1.414 = -3.01 dB.

Cr2 - Second corner frequency of filter effect. (Third order

only)

3.5 Power In

Enter the Amplifier Power to be applied to each design in watts.

Files->Show A,B,C or D then Data->Power In menu and enter the

applied wattage.

4. PLOT

Freq Response -Small signal response of all systems selected.

Phase - Acoustic pahse response of all systems selected.

Impedance -Input Impedance for all systems selected.

Imp Phase -Phase of impedance for all systems selected.

Group Delay -Group delay for all systems selected.

Max Power -Maximum diplacement limited power for all selected systems.

Max SPL -Maximum SPL for all systems selected. Based on Power-In.

Driver Excur -Required excursion for applied wattage for each system.

Vent Air Speed -If all systems are vented or if only one

vented system is plotted.

PR Excur -If all systems are passive radiators or if only one

passive radiator system is plotted.

Step Response -Transient response of all selected systems.

Separate Resp -If only one system is plotted, this displays the

contribution of the box, the equalization and

the overall systems response.

Choosing Plots will cause a menu to drop down from which you

select how many systems you want to display simultaneously.

After this selection is  made there  will be a delay with the

message "Calculating" displayed.  This is when the extensive

floating point calculations are executed. Striking any key

will then return you to the plot type menu where you can make

another selection.  THE DEMO VERSION OF THIS PROGRAM DOES NOT

SUPPORT GRAPHICS DUMPS TO THE PRINTER, but the full version

provides that capability for over 300 printers.

When a single system is plotted, extra information is shown

in  some cases.  If the system is equalized, then three graphs

are shown in the small-signal magnitude plot: the box alone,

the equalizer alone and the combined response.

5. MISC

5.1 Driver Params

Any system parameter that may be of interest is calculated

by the program and displayed here.  The significance of most

of them is self-evident, but a few need comment.

In the list of DRIVER  parameters  QTS  is  the  parallel

combination of  QES  and  QMS  and  Sns is the calculated

sensitivity of the driver at 1 meter with input voltage

equal to the square root of RE. This is the usual 1 watt/

1  meter specification when the load impedance is R, and

it  usually is fairly close to manufacturers spec sheet

values.

5.2 Coefficients

Box Denominator Coefficients - Numerator coefficients of the

polynomial that describe the transfer function of the

box/driver combination of the current system.

Box Numerator Coefficients - Denominator coefficients of the

polynomial that describe the transfer function of the

box/driver combination of the current system.

Equalizer Denominator Coefficients - Denominator coefficients

of the polynomial that describe the transfer function of

the high-pass equalizer used in the current system.

5.3 Box Dim - Size

This menu option displays the internal box dimensions necess-

ary for the current box volume.  The dimensions are given to

the closest 1/16".  Several standard ratios are provided.

5.4 Fix Dim - Size

This option is provided for circumstances that require at least

one of the box dimensions to be fixed.  A typical case: An

enclosure you are designing for your car subwoofer.  You can

enter the distance you have available between the wheelwells

and the package tray in the trunk of your car and the program

would calculate the length the box has to be.

5.5 Ratios - Size

This option provides for the entry by the user of any dimens-

ion ratio they would prefer, over those calculated in Box Dim

above.

5.6 Show Alignment

alpha - Compliance ratio Vas/Vb.

h     - Tuning ratio Fb/Fs.

delta - Compliance ratio Vap/Vas

k1    - Cr1/Fs 1st order Equalization section.

dmp   - Reciprocal of filter Q. ( 1.4142 = -3.01 dB at Corner)

Cut Filter -> 20.0 * Log10(1.0/1.4142) = -3.01 dB

Boost Filter -> 20.0 * Log10(1.0/0.5)  = +6.02 dB

k2    - Cr2/Fs 2nd order Equalization section.

5.7 Vent Length

The vent length for vented-boxes is calculated  from  the

standard formula for one flanged and one unflanged end using

the speed of sound as determined from the ambient atmospheric

conditions given in the Temp & Pr window and the Vd (vent

diameter) in the Data->Box menu.

6. ALIGN

Choose this option to create a flat alignment for the current

selected system.

7. HELP

General comments and instructions on the operation of

this software.