5

Microsoft Windows and The Plug and Play Framework

Architecture

Corporate Backgrounder

Microsoftr WindowsTM and the Plug and Play Framework

Architecture

March, 1994

0394 Part No. 098-55723

Contents

Overview. . . .. . . . . . . . . . . . . .  . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . 2

PC hardware and software configuration difficulties

2

The Plug and Play solution

2

The Plug and Play Framework. . . . . . . . . . . . . . . . .

. . . . . . .. . . . . . . . . . . . . . . . . . . . . 3

Components and requirements of a Plug and Play system

3

Plug and Play: progress to date

4

The transition to Plug and Play systems

4

Functional aspects of a Plug and Play solution

5

The Next Release of Microsoft Windows: A Plug and Play

Operating System . . . . . . .     6

Plug and Play Devices. . . . . . . . . . . . . . . . . . . .

. . . . . . . . . .  . . . . . . . . . . . . . . . . . . . .

.    7

Plug and Play: Summary. . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

9

Conclusion. . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

. . . . . 10

Overview

Personal computers have revolutionized the way people work.

Today, PCs are being used in a wide range of settings and

have become the primary business tool.  Small and large

companies are using PCs to automate routine tasks through

productivity applications, and to solve complex business

problems through customized, vertical market applications.

Personal computer networks have enabled companies to work

together and communicate more efficiently, bringing office

productivity to new levels.  Furthermore, as hardware

technology has advanced, PCs have become both more powerful

and less expensive, accelerating their adoption on a

worldwide basis.

The wide-spread adoption of the Microsoftr WindowsTM

operating system has gone hand-in-hand with the advances in

hardware technology.  This corresponding advance in the

personal computer operating system has enabled a new class

of graphical applications that are easier to use and offer

greater benefits to users.  Additionally, new computer

peripherals including CD-ROM, sound and full-motion video

devices have further enhanced the potential of PCs, both in

the office and in the home.

PC hardware and software configuration difficulties

With these innovations, however, have come new industry

challenges.  Specifically, configuring PC hardware and

operating systems to work with different networks and with

different peripheral devices can be a significant problem.

Changing the hardware configuration of a machine is a task

that few end-users attempt, and even trained technicians can

find difficult, time-consuming and frustrating.  With the

growing use of mobile computers, such as notebooks and

laptops, this problem is compounded because users of these

computers typically need to change their PC configurations

on a more frequent basis.  These mobile devices are often

unplugged from corporate networks and peripheral devices in

the office, and re-configured to enable computing and remote

communication from the home or while on the road.

Whether changing the configuration of a notebook computer

from an office to a mobile setting, or simply adding a CD-

ROM or other device to an existing desktop computer, common

PC configuration difficulties have resulted in lower

customer satisfaction and increased support costs.

The Plug and Play solution

A broad base of companies within the computer industry,

however, is addressing these problems with a technology

known as the Plug and Play Framework Architecture.  As the

leading developer of personal computer operating systems,

Microsoft has played a key role in organizing support for

the Plug and Play framework.  To realize the goals of Plug

and Play, a fully functional Plug and Play operating system

is a critical requirement.  Microsoft is working closely

with the many participating hardware manufacturers, and will

release a complete Plug and Play operating system as the

next version of Microsoft Windows.

The Plug and Play Framework

Plug and Play is both a design philosophy and a set of PC

architecture specifications.  The ultimate goal of Plug and

Play is to design intelligence into the PC itself to handle

installation and configuration tasks without user

intervention. A Plug and Play system has a number of

characteristics.  First, any installation is a simple, fail-

safe operation.  For devices the installation is automatic:

plug the device in, turn on the system, and it works.  With

a Plug and Play system, the user can insert and remove

devices, or connect to or disconnect from a docking station

or network, without restarting the system or fiddling with

configuration parameters.  The system determines the optimal

configuration, and applications automatically adjust to take

full advantage of the new configuration.  Users do not need

to modify expansion card jumper settings, or even modify

operating system configuration files.  The benefits to both

users and computer industry members will be substantial, as

PC ease-of-use will be enhanced while support costs will be

substantially lowered.

The components and requirements of a Plug and Play system

The three core components of a Plug and Play system are:

Plug and Play hardware devices

Identification of installed devices

Creation of a complete system configuration, eliminating

all resource conflicts

Notification of configuration changes

As devices are added and removed, this process is repeated.

The operating system must centrally manage this

configuration process to ensure smooth coordination.  A

certain amount of configuration must first be performed by

the system BIOS during the power-up phase.  In order for the

system to boot, the BIOS must at a minimum configure a

display device, input device and initial program load

device. Then, it must pass the information about each of

these devices to the operating system for additional

configuration of the system.

The operating system then continues the configuration

process by identifying every device in the system and their

respective resource requirements.  Each non-boot device must

be inactive upon power-up so that the operating system can

identify any conflicts between the resource requirements of

different devices before configuring them.  In the event

that different devices require the same resources, the

devices must be able to provide information to the operating

system about alternative resource requirements, which the

operating system uses to identify a working system

configuration.  Once any resource conflicts have been

resolved, the operating system automatically programs each

hardware device with its working configuration, and then

stores all configuration information in the central

database.  Finally, the operating system loads the device

drivers for each device, and notifies these drivers of the

resource assignments.

If a change occurs to the system configuration during

operation, the hardware must be able to notify the operating

system of the event so that the operating system can

configure the new device.  Additionally, applications must

be able to respond to configuration changes to take

advantage of new devices and to cease calling devices that

have been removed.   Such dynamic configuration events might

include the insertion of a PCMCIA card, the addition or

removal of a peripheral such as a mouse, CD-ROM drive or

printer, or a docking event for a notebook computer.

The Next Release of Microsoft Windows: A Plug and Play

Operating System

The next release of the Windows operating system will

include several new components to provide complete Plug and

Play functionality.

These components include the following:

The Plug and Play Components of Microsoft Windows

The Hardware Tree

Resource Arbitrators

In addition, a number of components will be modified,

including device drivers, the operating system setup program

and portions of the user interface.

The Configuration Manager

The next version of Microsoft Windows will be the first Plug

and Play operating system for IBM-compatible personal

computers, making PCs easier to configure and maintain.

The Configuration Manager is the central software component

handling all phases of the configuration process.  It

orchestrates the entire flow of operations performed by all

the components involved in the configuration process, and

accepts and responds to communications from the BIOS and

hardware devices during the configuration process.  It also

responds to dynamic events during operation including the

insertion or removal of devices, and the docking or

undocking of mobile computers.  As these events occur, the

Configuration Manager communicates the information to

applications.

The Hardware Tree

The hardware tree is a record of the current system

configuration.  The tree information is drawn from a central

database of configuration information for all devices,

called the Registry.  The Registry is stored locally for

each computer, and holds information about all device types,

whether they are currently installed or not.  The hardware

tree is created by the Configuration Manager every time the

system boots or a run-time change occurs to the system

configuration.  The existence of the Registry will eliminate

the need for most of the device-specific initialization

files used today.

Bus Enumerators

Bus enumerators are responsible for building ("enumerating")

the hardware tree on a Plug and Play system.  The bus

enumerators are a new type of driver.  The different

enumerators are based on specific bus architectures, and

understand the implementation details of their bus type.

Thus, an ISA enumerator can identify the devices on an ISA

bus, read their resource requirements, and configure them as

instructed by the Configuration Manager.  Similarly, there

will be enumerators for Microchannel, VESA Local and PCI bus

types; SCSI and PCMCIA devices; and serial and parallel

ports.  During installation, Windows will automatically

determine which bus enumerators are applicable for a given

computer.

Resource Arbitrator

The Resource Arbitrator allocates specific types of

resources to devices, and resolves conflicts between devices

that request identical resource assignments.  The functional

separation of the Resource Arbitrator and the Configuration

Manager provides for future extensibility of the Windows

operating system to address new types of resources.

Changes to other operating system components

A new operating system setup program will create the central

configuration database during initial system setup.

Although under normal circumstances the system will not

require intervention by the user to perform any initial

setup configuration operations, there are some exceptions.

For example, if the user is installing a non-Plug and Play

device and the system fails to detect that device, the user

can force an installation by pushing a button in the control

panel or dragging an installation icon into the system

folder from a floppy disk.  Thus, the next version of

Microsoft Windows will not only automate the process of

installing and configuring Plug and Play devices, it will

also standardize the system interface for installing and

configuring non-Plug and Play devices.

At times the system may be unable to generate a non-

conflicting configuration for a non-Plug and Play device.

In this case, a new Windows user-interface component will

communicate the event to the user, and present the user with

several options to resolve the problem.  The Plug and Play

Windows operating system will also have an interface

component that will allow more advanced users to edit and

change the system configuration.

Plug and Play Devices

ISA

A specification for design of Plug and Play ISA devices has

been published, reviewed and refined through the Open

Process, and released to the industry.  This specification

defines a hardware and software design to be incorporated

into the next generation of ISA devices, referred to as Plug

and Play ISA devices.  Plug and Play ISA devices can run on

existing PCs, and are available today.  Over the coming

months, Plug and Play ISA devices will replace standard ISA

devices as the predominant device type.

PCI

The current PCI bus architecture meets most of the

requirements for providing Plug and Play functionality.  To

become fully Plug and Play capable, a bus enumerator for PCI

is being written by Intel to be incorporated into the next

version of Microsoft Windows.

Microchannel

Microchannel devices already provide a standard

identification mechanism and a mechanism for configuring

resources through software.  To integrate with the Plug and

Play architecture, a bus enumerator for the Microchannel bus

is being written by IBM to be incorporated into the next

version of Microsoft Windows.

VESA Local Bus

The VESA Committee is adopting the Plug and Play ISA

specification in order to adapt it to the VL-bus type.  An

enumerator will be provided in the next version of Microsoft

Windows.

IDE

The IDE Plug and Play specification has been refined through

the Open Process and released to the industry.  IDE

controllers already provide a way to support multiple disk

drives, and the IDE standard will also be able to provide a

low-cost solution for adding Plug and Play CD-ROM drives.

Full IDE Plug and Play capabilities will be incorporated

into the next version of Microsoft Windows.

SCSI

SCSI devices require both design changes and general ease-of-

use enhancements to deliver Plug and Play functionality.

The Plug and Play SCSI specification, refined through the

Open Process, specifies these requirements.  Full SCSI Plug

and Play capabilities will be incorporated into the next

version of Microsoft Windows.

PCMCIA

In order for PCMCIA devices to be supported under the Plug

and Play framework, Microsoft is writing a bus enumerator to

be incorporated into the next version of Microsoft Windows.

The approach to supporting PCMCIA under the Plug and Play

framework calls for the PCMCIA device to be treated like any

other Plug and Play device during the configuration process.

Thus, resources can be allocated and reclaimed dynamically

by the operating system, allowing for easy addition of

devices through hot-swapping, and support for hot or warm

docking of mobile computers.

Serial and Parallel Ports

Specifications for peripherals connecting through serial and

parallel ports, including Plug and Play printers, have also

been refined through the Open Process, and released to the

industry.  The next version of Microsoft Windows will

support Plug and Play automatic configuration and device

driver loading capabilities for these peripherals.

Plug and Play: Summary

Plug and Play technology will affect the entire personal

computer industry.  The technology will increase customer

satisfaction by making PCs even easier to use and maintain;

and will lower support and maintenance costs for both

vendors and users.  Plug and Play will provide a flexible,

robust platform for increased functionality and industry-

wide innovation.

Systems incorporating the next release of Microsoft Windows

as well as the Plug and Play BIOS and Plug and Play devices

will achieve the full benefits of Plug and Play.  These

systems will be able to automatically configure hardware

devices without any user intervention, and respond to

dynamic configuration events.  Excellent docking system

solutions will finally be feasible, because the system will

be able to automatically load and unload device drivers to

reflect the different devices attached to the system when it

is docked or undocked.  Also, applications will be able to

automatically adjust their configurations to reflect the

insertion or removal of devices, such as network cards and

fax-modem cards.

Partial Plug and Play functionality can also be achieved on

systems that will include only one or two of  the three Plug

and Play components.  Hardware vendors can begin to deliver

some of the benefits of Plug and Play prior to the release

of the next version of Microsoft Windows by incorporating

the ability to assign device resources through special

software drivers and a standard device driver interface,

eliminating the need to modify jumper settings on the

hardware devices themselves.  Also, the next release of

Windows will provide substantial Plug and Play benefits to

users of existing systems.  In such a system, resource

requirements for non-Plug and Play devices can be

accommodated through an improved configuration file format.

This will allow all existing devices to coexist with newer

Plug and Play devices.

Conclusion

The Plug and Play framework advances the PC architecture in

ways that will greatly enhance the ease-of-use of existing

PCs, and enable users to work with PCs in new ways.  The

results will not only be more satisfied customers, but also

lower support costs and increased demand for PC products.

In recognition of the benefits of Plug and Play, a variety

of companies representing all major areas of the industry

are working together to define the architecture and to

implement that architecture in their products.  Microsoft is

one of the leading members of this industry coalition, and

is including full Plug and Play functionality in the next

version of Microsoft Windows.  This new version of Windows

will bring substantial benefits to millions of existing

systems, as well as future systems based on the Plug and

Play hardware specifications.  Through Plug and Play,

personal computers will have the potential to even further

enhance business productivity.

For more information

Copies of Plug and Play specifications can be obtained from

the libraries in the "plugplay" forum on CompuServer.  This

forum also serves as the focal point for technical

discussions, industry feedback and technical support of the

Plug and Play specifications.

Plug and Play ISA Specification

Plug and Play SCSI specification

Plug and Play IDE Specification

Plug and Play COM Specification

This document is provided for informational purposes only.

The information contained in this document represents the

current view of Microsoft Corporation on the issues

discussed as of the date of publication.  Because Microsoft

must respond to change in market conditions, it should not

be interpreted to be a commitment on the part of Microsoft

and Microsoft cannot guarantee the accuracy of any

information presented after the date of publication.

INFORMATION PROVIDED IN THIS DOCUMENT IS PROVIDED "AS IS"

WITHOUT WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF

MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND

FREEDOM FROM INFRINGEMENT. The user assumes the entire risk

as to the accuracy and the use of this document. This

document may be copied and distributed subject to the

following conditions: 1) All text must be copied without

modification and all pages must be included; 2) All copies

must contain Microsoft's copyright notice and any other

notices provided therein; and 3) This document may not be

distributed for profit.

Copyright c Microsoft Corporation. All Rights Reserved.

Microsoft and MS-DOS are registered trademarks and Windows

is a trademark of Microsoft Corporation.

IBM is a registered trademark of International Business

Machines Corporation.

CompuServe is a registered trademark of CompuServe, Inc.