****  TECHTIPS.DOC as of Feb 3rd,1993  ****

�����������������������������������������ͻ

�            C X R  Telcom                �

�   521 Charcot Ave, San Jose, CA 95131   �

�                                         �

�       Fax         (408)955-0502         �

�       Sales Dept. (408)435-8520 x 297   �

�       BBS         (408)955-0803         �

���ͼ       Repair      (408)435-8520 x 311   ���ͻ

�  MOST COMMON QUESTIONS, PROBLEMS, AND ANSWERS.  �

�  ���������������������������������������������  �

�                                                 �

�  SECTION1:  Fax96              ("fax-only card")�

�  SECTION2:  Fax96 with 1-Liner ("fax-only card")�

�  SECTION3:  Fax96 with Modem                    �

�  SECTION4:  Fax96 with Modem and 1-Liner        �

�  SECTION5:  Windows software                    �

�  SECTION6:  Modem Information                   �

�  SECTION7:  General Information                 �

�  SECTION8:  Winfast 1445 & 1446                 �

������������������������������������������������������������������������������

SECTION1:  Fax96 ("fax-only card")

__________________________________

1. Q:SOFTWARE RELEASE. What is the latest software release?

A:      Release 4.15 is the latest fax software for the Fax96.

2. Q:PRINTING. Why is my printer not printing correctly?

A:   a. Run FAXCONFG and make sure the correct printer is selected.

b. If the EPSON-LQ printer giving you problems, run FAXCONFG and select

FORTIS-DQ (quality.)

c. HPII-LaserJet - select HPIIP in FAXCONFG.

d. HPIIIp-LaserJet - Make sure printer is set to Portrait Mode.

e. Post-Script printers - Only our Windows s/w will print to it.

f. Cannon Bubble Jet 300 - select IBM Proprinter X24E in FAXCONFG.

g. HPII-LaserJet - If TTI header (From: ... Pages 1of2) is not

printing, then select HPIII in FAXCONFG.

h. HP LaserJet IV - only our Windows Software will print to it.

Our DOS Fax software does not have a printer driver for LJ IV.

*****************************************************************************

Note: The Dos based Fax software will not have any additional printer drivers

added in the future.

*****************************************************************************

3. Q:DEMO MODE. Why do I keep getting "No fax card - Demonstration Mode Only?"

A:   a. Make sure Jumpers on card match settings in FAXCONFG.

b. Try a different IRQ and/or I/O settings on card and in FAXCONFG.

c. Power-off computer and re-seat fax card into computer.

4. Q:CAN'T RECEIVE. Why can't I receive a fax?

A:   a. Is this problem from any fax machine or just one fax? If only one,

then make sure you have the latest software and re-test. If

problem persists, call tech. support and give them the make/model,

and fax phone# of that problem fax machine for us to test.

b. Put FRECOMRF /ON in your AUTOEXEC.BAT file so that is loads before

and other program. Best place is usually one line below the PATH

statement. This will auto-load the receive program when you turn

on you computer. When sending, displaying, or printing a fax,

use only FCCPRG, not FRECOM.

5. Q:NO PHONE LINE. Why does the screen says "No Phone Line Detected" when I

try to dial out, and how can I correct it?

A:    a. Power-off your computer, wait a few seconds,then power back on.

b. Check that if the phone cable is plugged from the wall to the Fax96

bottom jack.

c. Exchange the phone cable going from the fax card to the wall.

d. Try connecting a phone to the top jack of the fax card and listen

for a dial tone.  If you can hear a dial tone, then the board has

a hardware problem, please call our Service department at

408-435-8520, extension 311 for repair or replacement.

Business hours from M-F 8-4 pst.

6. Q:QEMM MEMORY MANAGEMENT. How do I load my receive program using QEMM?

A:      From a DOS prompt, type  LOADHI /R:2 FRECOMRF /ON

7. Q:Not Working in Windows.  Why doesn't my Fax96 work with Windows?

A:      The Fax96 was not built to work under Windows; it is not compatible

with windows.

8. Q:Different Software.  Does Fax96 support other manufacturers' faxing

software?

A:      The answer is NO, Fax96 only works with the software packaged with

it.

SECTION2:    Fax96 1-Liner ("fax-only")

_______________________________________

1. Q:ANSWERING MACHINE. My ansering machine answers the phone, but

the 1-Liner will not detect a fax. Why?

A:   a: Make sure to connect your answering machine (a/m) to the TOP jack

of the 1-Liner card; cannot have it on another phone jack or

another room. Also set the answering machine to pick up the line

first.

Example: Set a/m to 4-rings and the 1-Liner to 6-rings. After

4-rings, the a/m will answer, THEN the 1-Liner can "listen" for a

fax.

b: Try putting a two second pause before starting and after ending

your greeting message, that should give the fax enough time to

acknowledge the tones.

c: Do you have a combination Telephone/Answering machine?  If you do

then we are aware of the problem, and are currently in the process

of conducting tests; the Fax96 does not seem to work with the new

digital technology Telephone/Answering machine combination.

d: Check to see if you have the latest software releases.

e: Check to see if you have any other memory resident programs(TSR's)

that might conflict with the one-liner not being to detect an incoming

fax.

SECTION3:    Fax96 with Modem

_____________________________

NOTE:

5.20 Software Release on this BBS. 5.20 is the latest

DOS faxing software update.

1. Q: ERROR:"NO FAX CARD FOUND-DEMO ONLY." I keep getting this when I start

the fax software?

A:   a. Check switches on the card to make sure they match the settings on

FAXCONFG.

b. BIG HELP!!! Disable the existing COM port in your computer.

Example: If card is set to COM2, then disable COM2 in your

computer. See your computer operators manual for instructions.

c. Test the card using COMIT software diagnostics.

Follow the keystrokes below:

/ "brings up the comit menu"

Diagnostic

Check all comm ports

The modem ID is 249. If the test found the modem on the com port.

2. Q: DISPLAY. Can't display some faxes?

A:      Bug in 5.17. Download 520-1.ZIP from this BBS.

520-2.ZIP

520-3.ZIP

3. Q: PRINTER setup. Everytime I enter FAXCONFG, my printer is de-selected?

A:      Download FAXCONFG.EXE from the BBS.

SECTION4:    Fax96 with Modem and 1-Liner

_________________________________________

1. See SECTION 2 for more 1-Liner tech. tips. See Fax96 with Modem section

for more fax modem tips.

2. Q: 1-LINER. My 1-Liner detects fax calls but does not detect modem calls?

A:      FAXDATSR is not working, and is currently in the testing stage

3. Q: 1-LINER. The 1-Liner does not pickup fax calls?

A: Check to see if you have the latest Software Releases and/or

latest Firmware Revisions.  Please refer to SECTION 2 for more

1-Liner troubleshooting tips.

4. Q: 1-LINER. How can I test the 1-Liner function of the fax card?

A:      The test below is used to test the Fax/Modem/1-Liner circuit.

I. Load COMIT software.

II. Type the following commands to put COMIT into manual control

mode:

/

O

M

III. Type the following from the COMIT manual control screen:

AT+FCLASS=0  hit the <Enter> key. Will see "OK"

AT+FAA=0             <Enter>

AT*F1                <Enter>

IV. Now have someone call you using a regular phone.

V. Pick up the phone attached to your Fax96 1-Liner. You should

see +HKLO appear on the screen.

VI. Have the caller hit touch-tones on their telephone key pad. You

SHOULD see the numbers they are hitting (i.e. +DTMF5 +DTMF4)

on your screen. If you do not see anything appearing then the

1-Liner circuit is not working and should be sent back for repair.

VII. If you DO see numbers appearing, then the fax 1-Liner  card is

working.

VIII. Type the following to get the 1-Liner card out of test mode:

AT*F0                <Enter>

6. Q: Why does my release 5.20 software schedule a delayed send when I want

the fax to go out immediately?

A: This is a bug, and the fix is currently being tested.  Will be on this

BBS soon.

7. Q: One-Liner & Call Waiting.

A: Please see section 5 for further details.

SECTION5:    Windows software

_____________________________

1. Q:SOFTWARE RELEASE. What is the latest software release?

A:      Release 2.05 is the latest software release for Windows.

Known fixed bugs:

Better Send/Receive.

Display Log bugs.

Display Fax bugs.

Support for Windows 3.1.

Option to turn off Auto-receive.

COM 1 thru 4 support AND Alt IRQ's

Flips in display

Auto print

1. Q: WINDOWS. Can I run my Fax96 card in Microsoft Windows?

A:      Only Fax96 with Modem cards will run in Windows.

See SECTION 7 for more info.

2. Q: WINDOWS.  What if my Windows is not installed on its default directory,

which is c:\WINDOWS?

A: The Fax Window will not install unless Windows is in the default

directory   C:\WINDOWS

3. Q: WINDOWS.  Why do I get several hundred pages when I attach files to be faxed

in Windows?

A: Make sure that the attached files are in the fax, pcx, ascii, or tiff format.

If you would like to send MS-Word 2.0 file, simply print to the Fax

Window; make sure that the Fax Window is initialized and minimized.

4. Q: WINDOWS.  Why do I get a "Can't open Comport Message" when I try to

load the Fax Window?

A:  a: Make sure that no other communications program is loaded.

For example if you have Terminal loaded, make sure to exit completely

out of it before loading the Fax Window.  The reason is the comport

cannot be shared by the two software applications.

b: Another thing you can do is change the IRQ settings for COM 3 or 4.

Please read your Windows manual on how to setup a different IRQ

than the default one that Windows comes with.  Also make sure that

the Fax/Modem (hardware) match the Windows (software) settings in

all aspects.  Example to change the IRQ for COM4, you load windows

then control panel, then ports, then DOUBLE CLICK ON COM4, that

opens up another window with different baud rates, bits, parity,...etc.

Push the advanced option button, and the window for the IRQ and comport

addressing pops-up.  Make the necessary changes so that the settings match

your fax/modem settings.

5. Q: WINDOWS & 1 LINER.  Will CALL WAITING work with my Fax Board in Windows?

A: No, if you have call waiting and you are speaking to someone on the line,

and you get another call; you click over and if it is a fax

IT WILL NOT GO THROUGH, one-liner will not be able to detect it.

Our suggestion is to have a telephone line that does not have a

CALL WAITING FUNCTION.

[THE SAME IS TRUE FOR THE 1 LINER FUNCTION IN THE DOS ENVIRONMENT]

6. Q: SCANNERS. Do you have scanner support for the Windows fax software?

A:      Only scanner support is with our DOS-based software at this time;

Chinon, Canon, and HP scanners.

7. Q:FEATURES. What features are in the DOS-based software but not in the

The Fax Window software?

A:      Some features not YET in The Fax Window Rel. 2.05 include:

-Custom Cover Letter

-Direct Scanner Support

-Able to save/rename LASTSENT fax file.

-Able to print the log.

SECTION6:    Modem Information [9624 FAX/MODEM]

______________________________

1. Q: MODEMS. Will it work with my communications programs like

Prodigy, Compuserve, Procom, Comit,...etc.

A:      You can re-set the fax/modem card using AT commands. Best way is

to turn off your computer, turn it back on, get into the Comit

program then follow the key strokes below:

/                to bring up the Menu

O                Go Online...

M                Manual control of modem

You will now be in a blank screen that will allow you to type in modem

commands. Type the following:

AT               then hit the <Enter> key. The modem will come back

with OK. If you do not see what you are typing, change

the ECHO setting  (Alt-E)

AT&F&C1&D2&W     then hit the <Enter> key. Once again, you will see OK.

Note: the W will write these commands into the modem

so you will not need to do this procedure again.

Now you can use Prodigy, Compuserve, ...etc.

2. Q: EPROM Revision. How can I find my Eprom Rev. without opening my PC ?

A1:     Use the AT command below using COMIT, PROCOMM, etc.:

AT+FREV?     then hit the <Enter> key. You will see something like:

1.29

A2:     You can also use CARDTEST /G. (See next Question below.)

3. Q: CARDTEST. Another easy way to find information about my fax card?

A:      Use the cardtest command from a dos prompt:

CARDTEST /G

4. Q: GARBAGE CHARACTERS.

A:      If you dial into this (or any other BBS) and get garbage

characters on the screen, then check the following:

a. Turn off MNP compression.

b. Make sure baud rate matches host computer; i.e. 2400

5. Q: DIALTONE. Getting "No dial tone" message using fax and modem.

A:   a. Check that bottom jack on fax card connects to phone jack in wall.

b. Using COMIT, get into manual mode and type:

ATDT

You should hear a dial tone coming from the fax card speaker. If

you do not hear anything, then the card might be at fault and needs

to be serviced or swapped for another Fax96 fax/modem.

SECTION7:    General Information

________________________________

1. Q: SERIAL NUMBERS. How can I tell which Fax96 card I am using?

A:      By the first letter in your serial number. The serial number is on

a small, white, bar-coded sticker located on the card as well as in

the operators manual.

FAXCARD                         SERIAL NUMBER#

Fax96                           A00...

Fax96 with 1-Liner              L00...

Fax96 with Modem                M00...

Fax96 with Modem and 1-Liner    B00...

1445 external Fax/modem V32bis  D00...

1446 internal Fax/modem V32bis  C00...

2. Q: MONOCHROME MONITORS. How do I put s/w in monochrome mode?

A:      Start software adding the command "/m" to the end command.

(Example: 1)fax /m  2)fccprg /m  3)frecom /m)

This switch is needed if running:

VGA Monochrome monitors

Laptop LCD computers

Some CGA monitors

3. Q: PHONE BOOK. Can I import my Dbase phone list into my Fax96 phone book?

A:      Not yet. Hope to have a utility that will take an ASCII file and

port it to TELBOOK.DAT. Will be on this BBS when ready!

4. Q: WINDOWS. Can I operate my Fax96 card in Microsoft-Windows?

A:      Fax96 "fax-only" cards run in DOS.

Fax96 with Modem cards run in DOS.

To run in Windows you must have:

a. The Fax Window program ($30 + shipping upgrade from Sales Dept.)

b. MS-Windows 3.0 only understands COM1 or COM2.

MS-Windows 3.1 only understands COM1,COM2,COM3,COM4

5. Q: AUTO-PRINTING. I see "Auto-Print Disabled" in the print menu.

How do I enable it?

A:      Hit the "A" from the Print Menu screen.

6. Q:POST-SCRIPT FILES. How can I covert Post-Script files so they can be

faxes by Fax96?

A:      Call your local software vendor and ask about one of these

programs; QMS Ultrascript, LaserGO, or GoScript.

7. Q:TANDY COMPUTER. I get "Unable to write to Drive D:" when installing the

fax software.

A:      Tandy computers have a ROM-drive on drive D: (read-only). Before

installing our software, get to a DOS prompt and type:

SET COMSPEC=C:\COMMAND.COM

next, put our install disk#1 in drive A: and from the A:\> prompt,

type: INSTALL A: C:

after the fax program is installed, re-boot you computer.

8. Q:SPEED. Why does my Fax96 send at 9600 baud, but receive at 4800 baud?

A:       Non-turbo XT, or PC class computers are too slow to accommodate

speeds greater than 4800 in the receive mode.

9. Q:GRAPHIC PROGRAMS. My CAD proram does not support .PCX,.TIF formats. How

do I convert them into a "faxable" format?

A:      Look into getting a file conversion program like "Hijaak," by

Inset Systems (800)828-8088.

10.Q:PCX FILES. How can I convert a received fax into .PCX ?

A:      You can use the FAX2PRN.EXE utility to convert a fax96 fax file

into a PCX file by typing the command below. NOTE: This PCX file

will only be read by MS-Windows Paint program. Other programs seem

to have problems reading it.

FAX2PRN faxname.000 newname.PCX /PCX

Example: C:\FAX>FAX2PRN C:\FAX\FAXES\ROSE.000 FLOWER.PCX /PCX

11.Q:WORD PROCESSING. Can't fax Word Perfect files using Faxography.

A:      Make sure to use the EPSON print driver inside WP51. If you have

WP 5.0 then use their EPSON LQ-500 driver (it works just fine)

You MUST print to a file using this driver and the file name MUST

have the extention of .ECL (example SAMPLE.ECL)

*The above note applies to ANY Word Processing program.

12. Q:FAX96 FAX FORMAT?

A:     We are following the Industry Standard CCITT Group III fax format

as noted in the "Blue Book Volume VIII" To obtain a copy, call:

Omnicom    (703)281-1135

fax    (703)281-1505

13. Q:MEMORY. How does your program run in memory?

A:     You can run our background program in EXPANDED, not EXTENDED memory

using a memory managment program like QEMM.

Memory usage for background programs are:

FAX96 fax-only: frecomrf /on = about 50k

FAX96/Modem   : faxsr /on    = about 65k

Your computer should have at least 640K of base memory.

Our foreground programs ONLY run in base memory.

14. Q: OPERATING SYSTEMS. Which ones do you support?

A:     PC-DOS           YES

MS-DOS           YES

CONCURRENT DOS   NO

UNIX             NO

ZENIX            NO

OS/2             NO

DR DOS 6.0       NO "Intermittent Problems"

15. Q: LOCAL AREA NETWORKS's. Can I run my fax card on a LAN?

A:     We have ONLY tested it on a Banyan Ethernet network (Only on a

workstation, not the file server.)

If running Novell, Make sure to have IPX 3.0 or higher and make sure

you do not have an IRQ conflict with your netwrk card. Printing was

tested using a local HPII laserjet printer.

(Novell info. was supplied by a Frecom customer.)

16. Q: PBX PHONE SYSTEM. Can I run my fax card through my phone system?

A:     Digital PBX phone system  NO

Analog phone system       YES

17. Q: VOICE CARDS. Do you work with them?

A:     Watson Card              NO

Complete PC Voice card   NO

They seem to compete with our receive TSR when a call comes in.

18. Q: SCANNERS. Which ones do you support?

A:     We support HP Scan Jet, Canon IX-8, IX-12, Chinon DS-2000, DS-3000.

All other vendor's scanners still work but you must scan an image

into a .PCX or a .TIF file, set scanner resolution to 200dpi or

below, then enter Fax96 program to send it.

*Will work with HP Scanners NOT HP compatibles.

19. Q: MENU SOFTWARE.

A:     Fax96 has been tested and works with

DIRECT ACCESS  Version 4.1

Delta Tech. International

P.O. Box 1104 Eau Claire, WI 54702

(715) 832-0958

Thank You Gary!

20. Q: FAXOGRAPHY.

A:     If your line drawings are not lining up, make sure to download

our latest software from this BBS. Older releases had this bug.

21. Q: SPEAKER. How can I increase the speaker volume on my Fax96/Modem?

A:     The volume is not as high as we would like, however there is

no means to boost its output at this time.

22. Q: INTEL CAS COMPATIBLE.

A:     Fax96 is not CAS compatible. Our fax/modem cards are CLASS 2.

SECTION8:       Winfast 1445 & 1446

-----------------------------------

************************************************************************************

IMPORTANT NOTICE TO WINFAST USERS:  1445 & 1446 FAX/MODEMS

************************************************************************************

IF YOU HAVE RECENTLY UPGRADED YOUR FIRMWARE REVISIONS AND ARE UNABLE TO OPERATE THE

FAX/MODEM, THEN YOU NEED TO EXECUTE ANY COMMUNICATIONS SOFTWARE, SUCH AS, COMIT

PROCOMM, TELIX, ETC...

ONCE THE COMMUNICATIONS SOFTWARE IS LOADED YOU NEED TO GO TO A SCREEN WHERE YOU

ARE ABLE TO ENTER AT COMMAND SETS.

AT THE SCREEN WHICH ENABLES YOU TO ENTER "AT" COMMAND SETS; FOLLOW THE INSTRUCTIONS

BELOW:

AT&W255

An 'OK' response should appear, if not check to see that the comport settings

match both the hardware, as well as, the software settings.

************************************************************************************

1.  Q:  Why won't my Winfast fax/modem connect to some fax machines?

A:  The fax machine you are trying to transmit to, might be out of spec.

Please check to see if you have the latest Firmware revisions, as well

as, the latest faxing software release.

HOW TO CHECK FOR LATEST FIRMWARE:

LOAD ANY COMMUNICATIONS PROGRAM COMIT, PROCOMM, ETC...

GO TO WHERE YOU CAN ENTER MANUAL AT COMMANDS.  ONCE AT SCREEN FOLLOW

THE INSTRUCTIONS BELOW:

ATI3

You should get a value depending on which Winfast you have.  If you have

the

1446:  Then something like

1446 FXD1 69-51446-../..

For the 1445 external unit you should get

1445-FXD1 69-51445-.../...

THE DOTS AT THE END ARE THE REVISION NUMBERS, CHECK WITH TECH SUPPORT

IF THE ONES YOU HAVE ARE THE LATEST.

2. Q:  Will my Winfast work with any other Fax Software packages in Windows?

A:  Yes it will work with Eclipse Fax Software, by Eclipse Systems

ECLIPSE SYSTEMS

SUITE 1121

33 W. MONROE STREET

CHICAGO, ILLINOIS 60603

312-541-0260

800-452-0120

The Winfast will not work with any other software in the DOS environment

except for the one packaged with it.

1445/1446 have compatability problems with Z-Soft's Ultrafax software,

Delrina's Winfax Pro, BitFax, and FaxMaster.

Q:  Will the one-liner work with any other Manufacturers' fax software?

A:  No, the 1-liner only works with the FAX WINDOW, none of the other fax

software producers can activate the one-liner option.

SECTION9:  FarFetch Fax for Fax96 card only

-------------------------------------------

1. Q:  FarFetch.  Why do I get beeeep beeeep beeeep instead of beep beep when

I enter my FarFetch code?

A:  The Fax96 program needs to be running for the FarFetch Fax feature to work.

The three long beeeeeps mean that you have new faxes but that the Fax96

foreground is not running on your PC.  This is because even though the

background receive can take your call, the Fax96 foreground needs to be

running in order to send your faxes.  We considered building a background

send routine, but believe that the memory required would be more than you

would be willing to sacrifice.

**********************************END****************************************