-----------------------------------------------------------------------------

GROUND PROXIMITY WARNING SYSTEM / FMA / AP MODES / RADIO UTILITIES  V4.0beta

You need FlightShop and FS5.x to use these files.

-----------------------------------------------------------------------------

CONTENTS:

1)  GROUND PROXIMITY WARNING SYSTEM

A GPWS provides warnings when flying in abnormal or dangerous situations

close to the ground. It also gives altitude call-outs during a landing.

2)  OTHER WARNINGS

Altitude Alert, Autopilot Disconnect, Take-Off Configuration, Overspeed,

Stall Warning and Trim sounds.

3)  FLIGHT MODE ANNUNCIATOR

The FMA shows you the autopilot modes.

4)  WAYPOINT NAVIGATION MODE

You can enter the coordinates of maximum 20 waypoints.

5)  HEADING AND ALTITUDE MODES

You can select an AP heading and altitude without using the FS5 menu.

6)  VERTICAL SPEED MODE

To climb/descent with a fixed vertical speed.

7)  AUTOLAND MODE

To make automatic landings.

8)  RADIO UTILITIES

Freqencies can be entered in numbers, each radio set has a memory for a

second frequency and some advanced utilities.

9)  BACKGROUND ATC AND ATIS

Random ATC messages in the background and automatic weather reporting.

MAIN CHANGES FROM V3.2 TO V4.0:

- Altidude Acquire/Hold annunciation on the FMA.

- Altitude Alert Warning.

- Elevator Trim Wheel rotation sound.

- New sounds for Take-Off Configuration Warning (intemittent horn),

Overspeed warning (clacker) and Stall Warning (stick shaker).

- Systems Setup has been changed.

- Waypoint Navigation Mode (max.20 waypoints).

- Improved key response and options for compatibility with FMS.

-----------------------------------------------------------------------------

GPWS_V40.ZIP CONTAINS THESE FILES:

gpws40.adv    ->  Move this file to your \ADV dir and start it up within FS5

gpws40.doc    ->  This file

gpws40.txt    ->  Source file of gpws40.adv (you don't need it)

gpws_mod.zip  ->  Unzip this file for the Custom Module of gpws (to add gpws

to an ATC Workshop adventure). It also contains a gpws only

version without any other utilities.

-10.wav        -gs.wav        ->  Move these files to your \ADVWAV dir

-20.wav        -min.wav

-30.wav        -snkrate.wav

-40.wav        -snkrtpu.wav

-50.wav        -ss1.wav

-100.wav       -ss2.wav

-200.wav       -stkshkr.wav

-500.wav       -terrnpu.wav

-alarm.wav     -toolowt.wav

-alert.wav     -towarn.wav

-altalrt.wav   -trim1.wav

-apdisco.wav   -trim2.wav

-clacker.wav   -warning.wav

-dontsnk.wav   -windshr.wav

-flaps.wav     -wptshft.wav

-gear.wav

Notes: - This program will run smoother if you increase the adventure speed.

Edit your \FLTSIM5\fsfs.ini file by changing the first line to:

"adventure_exec_speed = 6"  (6 is the maximum speed, if you enter a

higher value the speed will drop back to 3!).

- If you use the Dual-Player option (programs like FMS), make sure

that you start the gpws40.adv before making the connection,

otherwise the 0 (zero) key won't function in this adventure.

- If you've got an updated or patched version of FlightShop you could

modify gpws40.txt for a small improvement for the g/s warning and

autoland. It only gives an improvement on airports where the ILS g/s

isn't 3 degrees (only on very few airports), so it's not really

necessary. If you do want to make the modification you can find some

instructions (concerning 'xVOR_1_GLIDE_SLOPE') somewhere at the

beginning of gpws40.txt (also for the files in gpws_mod.zip).

-----------------------------------------------------------------------------

KEY SUMMARY:

Systems Setup:

Shift-S   1. GPWS:      on / alt / off

Shift-S   2. GPWS:      normal / g/s-inh / flap-inh / gear-inh

Shift-S   3. FMA:       auto+ / auto / manual

Shift-S   4. FMA-key:   Space / Del / off

Shift-S   5. RD-keys:   Shift / normal / off / swap

Shift-SS  1. WPT-NAV:   on / off

Shift-SS  2. HDG/ALT:   all / keypad / normal / off

Shift-SS  3. V/S-mode:  on / off

Shift-SS  4. LAND:      on / off

Shift-SS  5. CAT:       A / B / C / D

Shift-D    DECISION HEIGHT: 200 / 100 / 50 / 20 / OFF

Spacebar   FMA/RA

key ` ~    HDG SELECT/HOLD/OFF    ->  use  keys 0-9 - + [ ]

key \ |    ALT SELECT/HOLD/OFF    ->       Enter/Tab

key ' "    V/S MODE               ->       Backspace/Del/Esc

key , <    WPT NAV SELECT/ON/OFF  ->

Ctrl-KeyPad7    Ctrl-KeyPad8    Ctrl-KeyPad9

HDG/HOLD/OFF    ALT UP 500ft    ALT/HOLD/OFF

Ctrl-KeyPad4    Ctrl-KeyPad5    Ctrl-KeyPad6

HDG <- 10deg    CRS TO/FROM     HDG -> 10deg

Ctrl-KeyPad1    Ctrl-KeyPad2    Ctrl-KeyPad3

HDG <- 1deg     ALT DN 500ft    HDG -> 1deg

(Shift)N   NAV1 frequency+CRS   ->

(Shift)M   NAV2 frequency+CRS   ->  use  keys 0-9

(Shift)A   ADF frequency        ->       Enter/Tab

(Shift)C   COM frequency        ->       Backspace/Del/Esc

(Shift)T   XPDR code  -> (not available by default because all keys are

in use, by changing the Systems Setup this key

can become active.)

(Shift)NN  swap NAV1 memory

(Shift)MM  swap NAV2 memory

(Shift)AA  swap ADF memory

(Shift)CC  swap COM memory

(Shift)NM  swap NAV1 <-> NAV2

-----------------------------------------------------------------------------

1)  GROUND PROXIMITY WARNING SYSTEM

A GPWS consists of the following modes:

MODE 1  - EXCESSIVE DESCENT RATE

(you'll hear:"SINK RATE" and when things get worse:"PULL UP")

This mode is armed below 2450ft RA, and becomes more sensitive when

flying closer to the ground.

I think many flightsimmers will hear this warning a lot, when

making an approach. Avoid it! You have to make a go-around each

time you hear it (that's the policy of the safer airlines).

You can avoid it by scanning your vertical speed, especially during

the last part of the approach. Never descend with more than

1000ft/min during the approach! If your ground speed is 130kts,

you should descend with 700ft/min on the glide path, try to keep

the needle there, if you get too high or low, just change it a

little. (If the pitch is too sensitive, try to decrease the

Elevator Sensitivity in Options-Preferences-Joystick)

MODE 2  - EXCESSIVE TERRAIN CLOSURE RATE

("TERRAIN TERRAIN - PULL UP")

You'll hear this warning when approaching the ground with a really

high vertical speed. It's armed below 2450ft RA, and more sensitive

when flying low (3200ft/min at 1000ft RA).

MODE 3 -  ALTITUDE LOSS AFTER TAKE-OFF OR GO-AROUND

("DON'T SINK")

This mode is armed after take-off or when you raise your gear or

flaps below 245ft RA during a go-around. If you loose too much

height you'll get this warning.

The mode is disarmed above 700ft(low speed) / 1333ft RA, or when

selecting gear down and landing flaps.

MODE 4a - UNSAFE TERRAIN CLEARANCE WITH LANDING GEAR NOT DOWN

("TOO LOW TERRAIN" / "TOO LOW GEAR")

This mode is armed below 1000ft RA with high speeds, and always

below 500ft RA (below 500ft with low speeds: "TOO LOW GEAR").

You can inhibit this mode (Shift-S  2.GPWS:gear-inh) if you have to

make a gear-up landing.

MODE 4b - UNSAFE TERRAIN CLEARANCE WITH FLAPS NOT IN LANDING POSITION

("TOO LOW TERRAIN" / "TOO LOW FLAPS")

This mode is armed below 1000ft RA with high speeds, and always

below 245ft RA (below 245ft with low speeds: "TOO LOW FLAPS").

You can inhibit this mode (Shift-S  2.GPWS:flap-inh) for a landing

with flaps less than 20, for instance with an engine failure or if

the flaps are damaged.

MODE 5  - BELOW GLIDE SLOPE DEVIATION

("GLIDE SLOPE")

If you sink more than 0.5 degrees below the ILS glide slope

(1.3 dots on the nav1 g/s indicator) and you're below 1000ft RA

you should hear "GLIDE SLOPE". This mode is almost impossible to

program with FlightShop but I made the following solution:

If you follow the g/s very precisely for a while (below 1500ft RA)

the g/s mode gets armed and you'll hear a double click.

Thereafter if you sink more than 1.3 dots below the g/s (and you're

below 1000ft RA) you'll hear the warning.

If it gives a false warning you can reset it by mistuning the NAV1

ILS frequency for a few seconds.

You can inhibit this mode (Shift-S  2.GPWS:g/s-inh) for a visual or

circling approach with the ILS frequency tuned.

(Below 100ft this mode is inactive, because the g/s signal

becomes less reliable at low altitudes.)

MODE 6  - BELOW SELECTED MINIMUM RADIO ALTITUDE and RADIO ALTITUDE CALL-OUTS

You'll hear these call-outs everytime you make a landing:

"FIVE HUNDRED"  - descending through 500ft RA

"TWO HUNDRED"   - descending through 200ft RA

"ONE HUNDRED"   - descending through 100ft RA

"FIFTY"         - descending through 50ft RA

"FOURTY"        - descending through 40ft RA

"THIRTY"        - descending through 30ft RA

"TWENTY"        - descending through 20ft RA

"TEN"           - descending through 10ft RA

This is a very handy mode in FS5 because we still don't have a

Radio Altimeter (altitude above ground). If you want to know your

Radio Altitude press the Spacebar twice.

Note: GPWS calculates the height of the landing gear for the

selected Aircraft while you are on the ground, and uses this

info to calculate the Radio Altitude more precisely.

So when you start GPWS while flying, or when you choose

another Aircraft while flying, the altitude call-outs are

less precise (until you've made one touchdown with this

aircraft). So it's better start GPWS on the ground, and to

change Aircraft on the ground.

You can select the DECISION HEIGHT (DH) by pressing Shift-D until

the desired DH appears. You can select 200, 100, 50, or 20ft.

When descending through the selected altitude you'll hear

"MINIMUMS", at this point you have to make a go-around if you don't

see the runway or the approach lights.

Background info:

The DH you select for an ILS approach depends on:

- the published DH for an ILS approach procedure (somewhere at the

bottom of an approach chart)

- the weather; ILS approaches are devided in three Catagories:

CAT I:    DH=200ft  the visibility or RVR (Runway Visual Range)

has to be around 600m or better (3/8mi)

CAT II:   DH=100ft  the RVR has to be around 300m or better

(1/4mi=400m)

CAT IIIA: DH=50ft   the RVR has to be around 200m or better

or less   (1/8mi)

CAT IIIB: DH=20ft   the RVR has to be around 100m or better

or less   (1/16mi)

- the aircraft; CAT II and III approaches may only be flown in

Autoland. You can now try autolands with this program, make an

approach with minimum CAT III visibility, you don't see a lot,

but that's the fun of it.

MODE 7  - WINDSHEAR WARNING

("WINDSHEAR WINDSHEAR WINDSHEAR")

You'll get this warning below 1500ft RA when you encounter a

performance decreasing windshear (when your airspeed suddenly drops

10 kts or more due to a decrease in headwind or an increase in

tailwind). If you want to hear it in FS5 you have to select a

wind layer (between surface and 1500ft) with a strong speed and

climb or descend through the top or base of the layer with an

airspeed below 180kts.

GPWS SETUP:

You can change the GPWS system setup in special cases, normally you don't use

these options, so don't touch it if you're a beginner.

Select: Shift-S  1.GPWS: on / alt / off

In 'alt' only MODE 6 is active (altitude callouts / minimums call).

In 'off' all GPWS modes are inhibited, the other warnings can't be

switched off.

Select: Shift-S  2.GPWS: normal / g/s-inh / flap-inh / gear-inh

'g/s-inh' inhibits the MODE 5.

'flap-inh' inhibits MODE 4b (and 5).

'gear-inh' inhibits MODE 4a (and 4b and 5).

-----------------------------------------------------------------------------

2)  OTHER WARNING SYSTEMS:

ALTITUDE ALERT          - When you have selected an AP Altitude, you will

get an alert tone and a flashing ALT ALERT "light"

(simulated in the gray message window) when:

- approaching the selected altitude within 750ft,

this is the normal 'entry' alert.

(In some aircraft you'll get no alert tone when

the AP is on, only the ALT ALERT light.)

- deviating more than 250ft from the selected

altitude, this is the more serious 'exit' alert.

Note that the AP doesn't have to be on, you can

also use it in manual flight. Just switch on the

ALTITUDE MODE and select an altitude, while the AP

is off.

Altitude Alert is inhibited with gear down or with

flaps 20 or more. You can switch off the altitude

alert "light", select: Shift-S  3.FMA:auto (see 3

FMA SETUP). The alert tone can't be switched off.

AUTOPILOT DISCONNECT    - When you switch off the AP you'll hear a warning

sound, this is normal (to wake you up).

TAKE-OFF CONFIGURATION  - Normally this mode is armed when you move the

throttle lever forward for take-off, but I couldn't

program this with FlightShop, so now this mode is

armed when accelerating through 30kts (above safe

taxi speed). You'll hear a warning sound when:

- parking brakes are set

- flaps are not in take-off position

(flaps 0 to 20 are OK for take-off)

- elevator trim is outside limits for take-off.

OVERSPEED                 - You'll hear a clacker.

STALL WARNING             - In big aircraft a STICK SHAKER starts to vibrate

both control columns. (Unfortunately the old

stall warning sound can't be switched off.)

ELEVATOR TRIM WHEEL       - Each time you push the Elevator (Stabelizer) Trim

switches (Home/End), you can hear the big trim

wheels rotating in your cockpit (on each side of

the pedestal next to the thrust levers).

You will also hear this sound when the AP trims

the elevator, this is fully realistic.

-----------------------------------------------------------------------------

3)  FLIGHT MODE ANNUNCIATOR

The FMA is an indicator which shows you the AP modes. In allmost all aircraft

it's located directly above the Attitude Indicator, in FS5 we have to use the

gray message window (until we have a better panel).

You can also select/check the AP heading/altitude in this window (in real

aircraft this is done with the MODE CONTROL PANEL (MCP)).

Each time when the AP mode changes, the FMA will show up automatically.

You can always check the FMA by pressing the SPACEBAR.

The information is presented in the following order:

ROLL MODE  -  PITCH MODE  -  armed mode

ROLL MODES:                         PITCH MODES:

___ ___    -> no mode engaged       ___ ___     -> no mode engaged

HDG ...    -> heading select        ALT ....    -> altitude select

HDG HOLD   -> heading hold          ALT ACQ     -> altitude acquire

LVL        -> wing leveler          ALT HOLD    -> altitude hold

LOC        -> localizer             V/S ....    -> vertical speed

LBC        -> loc back course

NAV1 VOR   -> vor1

WPT NAV    -> waypoint navigation

combined ROLL & PITCH modes:

ATT ROLL  ATT PITCH    -> attitude hold

APP LOC   APP G/S      -> approach

LAND                   -> autoland

LAND      FLARE        -> autoland flare

armed modes:

g/s armed        app armed    -> when both loc & g/s are armed

loc armed        land armed   -> autoland test succesfull

lbc armed        app only     -> final autoland test failed

vor armed        flare armed  -> final autoland test succesfull

(AP OFF  -> autopilot is off but a mode is engaged)

Example:

- before you intercept an ILS, you'll fly in heading select and altitude hold,

the FMA(MCP) looks like this:      HDG 030   ALT 02000

- when you're "cleared approach" by ATC you press CTRL-A to arm the approach

mode, the FMA looks like this:     HDG 030   ALT 02000   app armed

- when you're approaching the localizer, the AP changes from heading select

to localizer, the FMA will show:   APP LOC   ALT 02000   g/s armed

- and when you're on the glide slope, the AP changes from altitude hold to

glide slope, the FMA will show:    APP LOC   APP G/S

FMA SETUP:

Select: Shift-S  3.FMA: auto+ / auto / manual

In 'auto+' (default) the FMA will also show:

- automatic AP hdg/alt changes. For instance when you are using a

second computer with GPSNAV or FMS (to avoid a constant display of

hdg/alt changes select 'auto').

- ALT ACQ and ALT HOLD when levelling off at the selected altitude.

- In 'auto+' you'll also see the flashing ALT ALERT "light", when

approaching/deviating from the selected altitude (see 2).

In 'manual' the FMA will only be displayed if you press the SPACEBAR.

Select: Shift-S  4.FMA-key: Space / Del / off

Normally you use the Spacebar to check the FMA/RA, however users of

FMS may need the Spacebar if they type a command on the FS5 computer,

therefore they can select the Del key for the FMA/RA.

Notes: - When decending below 200ft RA the FMA won't display automatically,

also when disconnecting the AP you won't see the FMA.

- RADIO ALTITUDE

If you press the SPACEBAR twice you can check your Radio Altitude,

that is your altitude above ground level (you can do this at all

altitudes, although the range of a real Radio Altimeter is limited

to around 2500ft). If you've selected a Decision Height (Shift-D) it

will also be displayed.

-----------------------------------------------------------------------------

4)  WAYPOINT NAVIGATION MODE

With a Waypoint Navigation System (or INS Inertial Navigation System) you can

navigate without using radio beacons. You just have to know the coordinates

(lattitude and longitude) of a point, which you can find on the (Jeppesen)

charts/maps. This program uses the same waypoint format as on the maps/charts

and as you can find in all aircraft with a INS/FMS.

You can enter max.20 waypoints.

Simple use:

Press key < or ,

You will see: WPT01  >N00 00.0  E000 00.0

Here you can enter the first waypoint, just type the

coordinates with keys 0-9 (and N/S E/W if required).

For Instance: N48 51.5  E002 17.7  (Eiffel Tower in Paris)

(you have to type ALL the digits, for instance not:2 but:002)

and press Enter, you can now enter a second waypoint (WPT02),

or just press Enter again if you don't want to insert more

waypoints.

If the AP is ON the aircraft will fly to WPT01 (ACTIVE WPT).

You will now see waypoint information like:

WPT01  hdg302  trk305  83.2nm  00:21

showing the heading and the track to waypoint (the diffence

between hdg and trk is the windcorrection), the distance(nm),

and the time to go (with the actual speed) in hours and minutes.

This info will be displayed as long as WPT NAV is active.

When approaching WPT01, the AP will automatically change to

WPT02 if you've entered it, etc.

Full options:

key < or ,

1 time:  WPT SELECT to view/enter waypoints, you can now use these keys:

-/+   to select a waypoint (WPT01-WPT20)

Tab   to select the active waypoint directly, press Tab again to

selects the next wpt.

N/S   (if required)           ->  DON'T TYPE TOO FAST!

0-9   to enter the lattitude  ->  Type ALL digits!

E/W   (if required)           ->  Press Enter when you are finished,

0-9   to enter the longitude  ->  or press Tab/-/+/Backspace/Del/Esc

Enter - Normally if you press Enter you will leave WPT SELECT (and

turn WPT NAV ON if it was off).

- After you entered a waypoint, press Enter, now you can enter

the next waypoint etc, or just press Enter twice if you are

finished.

>     DIRECT TO WPTXX (key > or .)

the program will search for the next waypoint, thereafter

press Enter to fly direct to this wpt (this wpt becomes the

ACTIVE wpt), or press > again to continue the search.

I     WPT INFO OFF/ON

By default the WPT INFO (hdg/trk/dist/time) will be displayed

constantly when WPT NAV is on. Press I to switch the gray

message window off. With WPT INFO OFF you can still view the

WPT INFO by pressing key < twice.

O     POSITION

To view your present position, thereafter press Esc or press

Enter to copy this position to the selected wpt.

T     TRACK, DISTANCE AND TIME INFO

To view the track, distance(nm) and time to go, if you would

fly direct to this waypoint from your present postion.

IMPORTANT: Don't stay in WPT SELECT forever, because the aircraft

doesn't make WPT NAV corrections, and can't do automatic

waypoint shifts, so you'll risk flying past your ACTIVE

wpt.

Also you can't use a some keys in the FS5 menus, if you

accidently are stuck in the menus: USE THE MOUSE.

key < or ,

2 times: WPT NAV ON / WPT INFO

- if a waypoint was entered this will also switch WPT NAV ON

- with WPT INFO OFF, use it to view the WPT INFO.

3 times: WPT NAV OFF

the AP changes to HDG HOLD

(or select HDG HOLD to switch WPT NAV OFF).

WPT SHIFT:

When you are approaching the active wpt, the program will automatically

change to the next wpt, you'll get an alert tone and a message: [WPTXX next].

If there is no next wpt, you'll get an alert tone and message: [no next wpt]

if you don't enter a new waypoint within about 20 seconds, WPT NAV will

disengage and the AP will change to HDG HOLD.

Note: If the active wpt is WPT20, the next wpt will be WPT01 again.

WPT WARNING:

If the track to the active wpt changes too much for some reason, you'll get

an alert tone and a [check wpt] message. If you don't check the waypoint

within about 20 seconds, WPT NAV will disengage and the AP will change to

HDG HOLD (you can press < to check the active waypoint, or press < twice to

ignore the message).

Notes: - N00 000.0  E000 00.0 is no wpt (empty), if you have to go to there

enter N00 00.1 and go a bit to the south.

- For those of you who have some waypoints in the old format, you have

to divide the seconds(") by 60.

For instance old: N48 51'30"       E002 17'43"

|                 |

30/60=.5          43/60=.716

|                 |

new:        N48 51.5         E002 17.7

If you get a headache, buy some new maps.

- You can also use the WPT NAV mode with the AP off, just follow the

WPT heading info.

- With WPT NAV on, sometimes a keypress doesn't function (you can hear

a double click), you have to press the key again if this happens.

-----------------------------------------------------------------------------

5a)  HEADING AND ALTITUDE MODES

Use the normal keys to select an AP mode, except for heading and altitude you

can use these keys:

key ` ~  (left of the 1 on most keyboards)

1 time:  HEADING SELECT

you can now select a heading in the following ways:

- Type a heading with keys 0-9 (not on the keypad) and press Enter.

You can also use the Backspace/Del/Esc keys.

(remember: don't type too fast.)

- Use keys - and + to change heading by ten degrees or [ and ] for

one degree steps. Press Enter when your finished.

- Tab changes the heading by 180 degrees.

Note: If you enter a heading which requires a 180 turn (+/-15 degr),

the AP will turn over left indicated by: HDG <-

If you want to turn over right press Enter again or press Tab.

(If you were turning over right already it will show: HDG ->.)

You'll have to wait until the FMA disappears before you can

make new selections.

2 times: HEADING HOLD

the AP will maintain the actual aircraft heading.

3 times: HEADING OFF

the heading mode will be disengaged.

key \ |

1 time:  ALTITUDE SELECT

you can now select an altitude in the following ways:

- Type an altitude with keys 0-9 (not on the keypad) and press Enter.

You can also use the Backspace/Del/Esc keys.

IMPORTANT: Type the altitude in FLIGHT LEVEL format (that's

without the last two zeros). For instance: if you want

to fly at 4500ft or FL45 type: 45

- Use keys - and + to change altitude by one thousand feet or

[ and ] for one hundred feet steps. Press Enter when your finished.

2 times: ALTITUDE HOLD

the AP will maintain the actual altitude

3 times: ALTITUDE OFF

the altitude mode will be disengaged.

Note: When you choose HDG/ALT SELECT a "<" appears after the hdg/alt to

remind you that you have to enter something, in the meantime the rest

of the adventure is bypassed, you won't hear any warnings etc, so don't

forget to press Enter.

Also in the meantime some keys won't function in the FS5 menus. If you

accidently open a menu in this case you have to use the MOUSE or press

SHIFT with the desired key.

For instance: while you're selecting a heading or altitude, the Enter,

Esc and other keys won't function with the FS5 menus. If you open a

menu in this case you have to use the MOUSE or use SHIFT-Enter or

SHIFT-Esc to leave the menu.

-----------------------------------------------------------------------------

5b)  EXTRA KEYS FOR THE HEADING AND ALTITUDE MODE

Extra keys are included to control the AP HDG & ALT modes directly (so you

don't have to press Enter), you can program these keys on devices like the

CH (Pro) Throttle, Thrustmaster WCS and such.

Ctrl-KeyPad7    Ctrl-KeyPad8    Ctrl-KeyPad9

HDG/HOLD/OFF    ALT UP 500ft    ALT/HOLD/OFF

Ctrl-KeyPad4    Ctrl-KeyPad5    Ctrl-KeyPad6

HDG <- 10deg    CRS TO/FROM     HDG -> 10deg

Ctrl-KeyPad1    Ctrl-KeyPad2    Ctrl-KeyPad3

HDG <- 1deg     ALT DN 500ft    HDG -> 1deg

- Press 'Ctrl-KeyPad7': 1 time to engage the AP HDG mode

2 times for HDG HOLD

3 tree times to switch off the HDG mode.

- Press 'Ctrl-KeyPad9': 1 time to engage the AP ALT mode

2 times for ALT HOLD

3 times to switch off the ALT mode.

- Press 'Ctrl-KeyPad5': 1 time to set the NAV1 CRS(OBS) TO the VOR beacon

2 times to set the course FROM the VOR beacon

If an ILS or LOC is recieved on NAV1 this CRS

will be set.

3 times to set the NAV2 CRS TO

4 times for FROM.

If you press 'Ctrl-KeyPad7'(HDG) within a few

seconds the TO/FROM CRS will be entered as AP HDG

(not with an ILS/LOC CRS).

- While in V/S MODE you can now change AP HDG and the preset ALT with

these keys. The response is a lot slower because the Vertical Speed has

to be calculated all the time. (You can't switch off the HDG/ALT modes

and 'Ctrl-KeyPad5'(CRS) doesn't function at all in the V/S MODE.)

HDG/ALT SETUP:

Select: Shift-SS  2.HDG/ALT: all / keypad / normal / off

You can choose which keys to use:

'all' (default) to use both the Ctrl-KeyPad keys and the ` and \ keys

'keypad' to use the Ctrl-KeyPad keys only

'normal' to use the ` and \ keys only.

-----------------------------------------------------------------------------

6)  VERTICAL SPEED MODE

key ' "  V/S MODE  (the AP must be on)

you can adjust the vertical speed in the following ways:

-  Press - and + to adjust v/s by 200ft/min or [ and ] to adjust

v/s by 50ft/min.

-  If you press 0-9 you can select a v/s of 0-900ft/min directly,

use Tab to change sign.

-  Press Enter/Esc or key ' to leave the V/S mode.

IMPORTANT:

As long as this mode is active you'll see the FMA with V/S mode on screen.

Other systems (GPWS etc.) are inactive and you cannot make other selections

without leaving the V/S mode! (except for the 'extra keys' see 5b).

WPT NAV corrections are also inactive with V/S mode.

The V/S mode is usefull for making a VOR approach or a LOC approach or for

making a slow climb/descent. You can also intercept a g/s with this mode, if

you're near the g/s (with appoach mode selected) the V/S mode will switch off.

You control the speed with the throttle.

The min/max v/s you can reach in this mode is around +/-1800ft/min for jet

aircraft (unfortunately this is a limitation of the FS5 AP).

Note that the scale on the Vertical Speed Indicators of most panels in FS5 is

not accurate, you can see differences of +/-100ft/min.

With heavy aircraft the V/S mode will be less stable.

V/S MODE -> ALT HOLD

If you enter an altitude with ALTITUDE SELECT and thereafter select V/S mode

and fly towards this altitude, the AP will level off at this altitude.

The FMA will show the selected altitude after a few seconds and when

approaching it the AP changes from V/S MODE to ALT ACQ (altitude acquire)

followed by ALT HOLD. (If you adjust the v/s during ALT ACQ, the AP changes

back to V/S mode.)

Notes: - While you're in V/S mode, the some keys won't function with the

FS5 menus. If you open a menu in this case you have to use the MOUSE

or press SHIFT with the desired key.

- When you activate the Map with Num Lock, the Map window sometimes

stays hidden. You could then press the ' key to bring the Map window

op top. But the ' key is used now by V/S, so you have to press

SHIFT ' now to bring the Map window on top.

- The AP response in the V/S mode also depends on the selected

aircraft category. If the V/S mode is unstable for an aircraft,

select a higher aircraft category (Shift-SS  5.CAT-A/B/C/D)

(for more info on categories see below).

-----------------------------------------------------------------------------

7)  AUTOLAND MODE

If the aircraft can make a stabilized AP ILS approach until 200 to 500ft RA

(depending on the selected aircraft category) the AP will make an AUTOLAND.

The landing is not fully automatic because you have to control the speed.

After touchdown you have to do the rollout.

You have to select the aircraft category first: (Shift-SS  5.CAT-A/B/C/D)

(Aircraft types are divided in categories, based on speed. Examples:

Cessna 172: CAT-A, Fokker 50: CAT-B, Boeing 737: CAT-C, Boeing 747: CAT-D.

To make a stabilized AP approach is easy with the learjet (you can use CAT-B

or higher) but gets more difficult with bigger aircraft; it probably won't

work with aircraft heavier than a 737, so don't bother to try a 747 or a 777.

(Many flight models for FlightShop are too sluggish (often exaggerated) for

the FS5 AP.)

Start the approach from 2000ft RA or higher (Ctrl-A arms the approach mode),

if the approach becomes unstable during the last part, select a higher

category, try to increase the speed a bit next time or use a lighter aircraft.

- Below 1000ft RA the AP will start autoland tests, if the aircraft is stable

on the ILS, the FMA will show "land armed" indicating that the aircraft

will try make an autoland.

- Between 200 to 500ft RA (depending on selected category) the AP will do a

final autoland test, if something is wrong the FMA will show "app only" and

you have to disconnect the AP and make a manual landing or a go-around.

If the final autoland test is succesfull, you'll see "LAND  flare armed"

for a brief moment, and the AP will make an autoland.

- At 30-50ft RA (depending on selected category) the AP will start to flare

and you have to close the throttle. The "LAND  FLARE" annunciation doesn't

appear automatically this is to give you a good view on the runway in

minimum visibility.

FMA annunciations during a autoland:

APP LOC   APP G/S   land armed    -> approach (autoland test succesfull)

APP LOC   APP G/S   app only      -> approach (final autoland test failed)

LAND                flare armed   -> autoland (final autoland test succesfull)

LAND      FLARE                   -> autoland flare (not displayed

automatically)

As with autolands in real aircraft you have to watch the performance of the

AP, and take over manually if necessary (autolands can't be made in every

wind condition or sometimes the ILS isn't suitable).

You have to control the speed with the throttle (because we can't program an

autothrottle with FlightShop). Find out the proper Final Approach Speed for

the aircraft; for most jet aircraft between 120-150kts.

If the speed is too low the approach becomes unstable or you'll make a hard

landing. If it's too high the touchdown will be too late.

Around 30ft RA you have to close the throttle!

If you make a crosswind landing you have to align the aircraft with the

runway centerline at touchdown.

After touchdown you have to keep the aircraft on the runway centerline

yourself (some aircraft also have an automatic rollout mode but this can't be

programmed) and apply thrust reversers and brakes. (Officially you have to

disconnect the AP after touchdown and push the nose down a bit during rollout,

but you may also leave the AP on during rollout as long as you steer it on

the centerline yourself).

Notes: - If the LAND mode is engaged, you cannot make any selections until

the AP is disconnected except for changing the DH (Shift-D) and

checking the FMA/RA (Spacebar).

- Sometimes the aircraft deviates a bit from the g/s while passing

200-500ft RA, when it deviates a lot check if your approach speed.

Note that the AP trims your elevator when you change speed, find the

approach speed at which the trim becomes neutral.

-----------------------------------------------------------------------------

8)  RADIO UTILITIES

- Freqencies can be entered with keys 0-9

- Each radio set has a memory for a second frequency (standby frequency)

- You can swap the NAV1 and NAV2 active freqencies and the OBS courses

(this is handy because the AP can only track NAV1).

keys:

(Shift)N   NAV1

- Tab swaps the active freq <-> standby freq.

- Use keys 0-9 to enter a frequency and press Enter to make it

active (or press Backspace/Del/Esc); thereafter you have to

enter the CRS (see below).

CRS

- Enter the CRS (OBS course), if you don't want to change it just

press Enter again. (Note that the CRS doesn't have a memory; you

can only enter one CRS.) Keys - + [ ] are also active now.

- If NAV1 recieves an ILS or LOC signal, the CRS will

automatically be displayed after two seconds (If you don't want

this to happen you have to type a course within two seconds).

(The CRS sometimes differs one degree from the published CRS.)

- If VOR1 is active you can press Tab to select the CRS TO the VOR,

press Tab again to select the CRS FROM (not always accurate).

(Shift)NN  Swaps NAV1 active freq <-> standby freq right away.

(Shift)M   NAV2  (you can also use (Shift)N2)

- Tab swaps the active freq <-> standby freq.

- Use keys 0-9 to enter a frequency and press Enter to make it

active; thereafter you have to enter the CRS (see below).

CRS

- Enter the CRS.

- If VOR2 is active you can press Tab to select the CRS TO the VOR,

press Tab again to select the CRS FROM.

(Shift)MM  Swaps NAV2 active freq <-> standby freq right away.

(Shift)A   ADF

- Tab swaps the active freq <-> standby freq.

- Use keys 0-9 to enter a freq and press Enter to make it active.

(Shift)AA  Swaps ADF active freq <-> standby freq right away.

(Shift)C   COM

- Tab swaps the active freq <-> standby freq.

- Use keys 0-9 to enter a freq and press Enter to make it active.

- keys - and + control the RFGAIN that is how much background ATC

you'll hear (see 7 BACKGROUND ATC).

(Shift)CC  Swaps COM active freq <-> standby freq right away.

(Shift)T   XPDR Transponder

- Use keys 0-7 to enter a transpoder code and press Enter.

Note: This function is not available by default because all keys

are in use, if you change the System Setup this key can

become active (for instance if you deselect the V/S-mode:

Shift-SS 3.V/S-mode:off, the (Shift)T will function).

(Shift)NM  Swaps NAV1 <-> NAV2 active freq & CRS.

(You can also press (Shift)MN or (Shift)N2N (all the same))

You can choose to use the Shift-N,M,A,etc. keys or to use the N,M,A,etc. keys

without Shift. The last option is easier but has the disadvantage that you

can't use those keys in the FS5 menus anymore.

Press Shift-S thereafter press 6 until you see the desired option:

Shift-S  6.RD-keys:shift  -> use Shift-N,M,A,etc. (default)

6.RD-keys:normal -> use N,M,A,etc.

6.RD-keys:off    -> radio utilities off

6.RD-keys:swap   -> to use the memory swap utility only:

use Shift-N,M,A,C to swap memory

use Shift-NM to swap NAV1 <-> NAV2

use Shift-CC and - or + for RFGAIN

Note: While you're selecting a freqency or course, a "<" appears to remind

you that you have to enter something, in the meantime some keys won't

function in the FS5 menus. If you open a menu in this case you have to

use the MOUSE or press SHIFT with the desired key.

ADVANCED RADIO UTILITIES:

- After selecting a NAV CRS you can press HDG SELECT (key ` or Ctrl-KeyPad7)

instead of Enter. The CRS will be displayed in HDG SELECT (this heading is

not corrected for drift).

- After selecting an ADF frequency you can also press HDG SELECT instead of

Enter. If the ADF is active, the QDM (course to the beacon) will be

displayed in HDG SELECT after two seconds. If you want the QDR (course from

the beacon) you have to press Tab.

- While you're in HDG SELECT you can press (Shift)N or (Shift)M to copy the

OBS CRS to the HDG. If you press (Shift)A the QDM will be displayed as HDG.

- For the NAV CRS utilities with the 'Ctrl-KeyPad5' key, see 3b.

-----------------------------------------------------------------------------

9)  BACKGROUND ATC AND ATIS

If you tune your COM Radio to 118.00 you can hear some random ATC messages

in the background. The messages have no meaning they just add some atmosphere.

Press (Shift)C - or +  to adjust the RFGAIN, that is how much ATC your radio

lets through (with 'swap' only: Shift-CC).

(You can expand the background ATC to other frequencies if you have new

*.wav files in your \ADVWAV subdirectory. Edit the 'atc'-subroutine in

gpws40.txt and recompile it.)

If you tune your COM Radio to 122.20 you will hear a simple ATIS weather

report, this is a continuously transmitted message which contains the surface

wind, clouds and the QNH (altimeter setting) in mb and inches of an airport.

Note: When decending below 500ft you won't hear ATC or ATIS to avoid

interference with the GPWS.

-----------------------------------------------------------------------------

This program is based on the systems of modern jet aircraft. I took the

B737, Fokker100, A310 and MD11 as examples, these systems are basically the

same in all types. I've used the gpws sounds of a Fokker100 (from a file by

Mathijs Kok, sounds recorded by Arjan de Jong) and the other warning sounds

were sampled by me in a B737 simulator. The idea for the swap freq came from

a program written by Stephen S. Odgaard. Thanks to all testers and mailers.

You may edit or use parts of gpws for your own use only, don't distribute or

upload changed versions without permission. Please report any bugs to:

Wilco van Deijl (on the FsFan net) or puk_muk@knoware.nl (email)

puk en muk freeware - the netherlands - jul96

-----------------------------------------------------------------------------