~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

~              README OF Mustek TWAIN Data Source                                                        ~

~                                                                                                                                          ~

~              Dec 20, 1994                                                                                                       ~

~                                                                                                                                          ~

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

This file contains the information for how to change the scanner configuration setting.

If you are a PCMCIA scanner user, please read the file PCMCIA.DOC included

in the same disk.

* INTRODUCTION

During the installation of Mustek TWAIN Data Source, a device line that specifies the scanner driver and the used configuration (i.e.,  IRQ, DMA and I/O address settings) will be automatically updated in the config.sys  file.

However, there are instances when you may need to change the IRQ, DMA or I/O address setting to avoid a hardware conflict.

* HOW TO CHANGE THE CONFIGURATION SETTING

To change the settings in the config.sys file, you can run the Scantest module which can be found in the Mustek Twain group icon by the following steps.

Double click the Scantest icon.

Reset the settings

Note: To change the I/O address setting, you should change it by changing the I/O address switch setting on the scanner interface card and the Scantest module will detect the new setting automatically. Please refer to the included manual for how to change the I/O address.

Click the Scantest button to ensure the updated setting does not conflict.

Click OK after passing the scantest and follow the screen message to reboot the system.

After rebooting the system, the config.sys file will be updated automatically according to the new setting. If you have a problem with changing the setting by running the Scantest module please read the following information for another solution by editing the scanner device line manually in the config.sys file.

1. TOOLS FOR EDITING THE SCANNER

To edit the CONFIG.SYS file, you will need a text editor.

If you have DOS 5.0, you should use EDIT.EXE. If you have a DOS

version below 5.0, you should use EDLIN.COM. Under Windows, you

could also use the Windows Notepad. Please refer to your DOS

(or Windows) manual for steps on how to use these text editors.

2.  THE SCANNER DEVICE LINE

The scanner device line should include the following items. These are:

*  Device driver

*  DMA setting (only included when using 1904 interface card )

*  IRQ setting

*  Scanner type setting

*  I/O address setting

In the CONFIG.SYS file, the correct syntax for the scanner

device line is as follows:

DEVICE=[drive:\][path-directory\]driver-name  In An Sn

where:

[drive:\][path-directory\]    is the drive and subdirectory

where the scanner driver is located

driver-name                   is the name of the scanner driver

(e.g., M1904.SYS, PRINSCAN.SYS)

Dn                           is the DMA channel

(e.g. D3 for DMA3)

In                            is the IRQ setting

(e.g., I7 for IRQ7)

An                            is the I/O address

(e.g., A2AB for I/O address 2AB, A378 for I/O address LPT1)

Sn                            is the scanner type

(e.g., S2 for HT-105 scanner, S5 for GS-800 scanner)

3.  SETTING THE SCANNER DEVICE LINE PARAMETERS

When you edit the scanner device line, there are certain

precautions that you should take note of especially when

choosing the IRQ and I/O address settings for the

PrinScan adapter.

Avoid hardware and software conflicts that can adversely

affect the performance of the scanner by selecting only the

available and unused DMA, IRQ, and I/O address settings.

Use the following options for setting the scanner device line

parameters.

3.1  The Mustek scanner device driver  [driver-name]

The Mustek scanner device driver is found in the WINDOWS\TWAIN\MHSC2  subdirectory.

For the scanner to work, it is important that the scanner driver is loaded

properly when you boot your computer. In the CONFIG.SYS file, make

sure the driver in the scanner device line has the correct path.

Driver name              Interface type

----------------            ------------------

M1904.SYS              Mustek Interface card with GI1904(A) chip

M1906.SYS              Mustek Interface card with GI1906(A) chip

PRINSCAN.SYS      Mustek PrinScan printer port adapter

PRINSCAN3.SYS    Mustek PrinScan III printer port adapter

PRINSCAN4.SYS    Mustek PrinScan IV printer port adapter

3.2  The DMA channel  [Dn]

When you assign the DMA setting for the scanner 1904 interface card,

make sure it is not used by any other device connected to

your computer. The available DMA channels for the

interface card are:

D1        for  DMA 1

D3        for  DMA 3 (recommended setting)

The devices that might have DMA settings are sound cards,

SCSI devices, anti-virus cards, CD-ROM drives, multimedia

cards, etc. If you have any of these devices in the computer,

please select the DMA channel not being used.

3.3  The IRQ level  [In]

When you assign the IRQ setting for the scanner interface,

make sure it is not used by any other device connected to

your computer.

The available IRQ levels for the interface card are:

I3         for  IRQ 3

I5         for  IRQ 5  (recommended setting for XT bus)

I10       for  IRQ 10 (recommended setting for AT bus)

The available IRQ levels for the Mustek PrinScan adapter are:

I5         for  IRQ 5

I7         for  IRQ 7 (recommended setting)

I10       for  IRQ 10

I11       for  IRQ 11

I12       for  IRQ 12

I15       for  IRQ 15

3.4  The Scanner type  [Sn]

S2        for  HT-105 / Matador 105

S7        for  GS-400L

S6        for  HT-800 TURBO

S5        for  GS-800

S4        for  CG-6000

S9        for  CG-8000

SA        for  CG-8400

3.5  The I/O Address  [An]

For the Mustek interface card:

The scanner interface card has eight (8) available I/O

addresses and is equipped with a switch for selecting the

address settings.

Inspect the interface card closely. Printed on the circuit

board are the 8 available addresses.  As with the DMA and

IRQ settings, select an I/O address that is not used by any

other device connected to your computer.

It is very important that the I/O address selection in the

scanner device  line should  match  the scanner  interface

card's I/O address setting.

The 8 available I/O addresses are:

*  22B                          *  32B

*  26B                          *  36B

*  2AB (recommended setting)    *  3AB

*  2EB                          *  3EB

The I/O address  3EB will conflict with  COM3  port  and

Western Digital's IDE hard disk controller card. If you are

using the I/O address 3EB, the I/O address on the scanner

interface card (DIP switch) and in the device driver should

be changed to another address (e.g. - 26B)

For the PrinScan adapter

The PRINSCAN adapter has three  available I/O addresses and is

equipped with a switch for selecting the address setting.

The three available I/O addresses normally are:

*  378 (LPT1)

*  278 (LPT2)

*  3BC (LPT3)

Note: The address which is actually used for LPT(1,2 or 3) depends

upon your system configuration. After installing the PrinScan

adapter, you can run the ScanTest program to find out what

address your system actually can use in the scanner device

line of the CONFIG.SYS file.

3.6  Examples of Scanner Device Lines

DEVICE=C:\WINDOWS\TWAIN\MHSC2\M1906.SYS D3 I5 S9 A2AB

In the example above, the scanner driver M1906.SYS is located

in subdirectory WINDOWS\TWAIN\MHSC2 of drive C. The parameters used are

DMA 3, IRQ 5, and I/O address 2AB. The scanner model is the

CG-8000 color scanner.

DEVICE=C:\WINDOWS\TWAIN\MHSC2\M1904.SYS D3 I5 SA A2AB

In the example above, the scanner driver M1904.SYS is located

in subdirectory WINDOWS\TWAIN\MHSC2 of drive C. The parameters

used are DMA 3, IRQ 5, and I/O address 2AB. The scanner model is the

CG-8400 color scanner.

DEVICE=C:\WINDOWS\TWAIN\MHSC2\PRINSCAN4.SYS  I7 S7 A378

In the example above, the scanner driver PRINSCAN4.SYS is

located in subdirectory WINDOWS\TWAIN\MHSC2 of drive C. The parameters

used are  IRQ 7, and I/O address 378. The scanner model is the GS-400L.

DEVICE=D:\WINDOWS\TWAIN\MHSC2\PRINSCAN.SYS  I5 S2 A278

In the example above, the scanner driver PRINSCAN.SYS is

located in subdirectory WINDOWS\TWAIN\MHSC2 of drive D. The parameters

used are  IRQ 5, and I/O address 278. The scanner

model is either the HT-105 or the Matador 105.

AFTER YOU HAVE EDITED THE SCANNER DEVICE LINE IN THE

CONFIG.SYS FILE, REBOOT YOUR COMPUTER TO MAKE SURE THE

CONFIG.SYS FILE IS READ.

SPECIAL NOTICE

~~~~~~~~~~~~~~

1. ABOUT OmniPage OCR (TWAIN-Compliant Version)

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Please modify the SCANMGR.INI under the Windows directory as below:

1.1 Set

...

TWAINDS=Mustek Hand Scanner II

...

1.2 Add

...

[Mustek Hand Scanner II]

SHOW_UI=1    --------------> Show User Interface

INVERT=1     --------------> Invert image

AUTO_BRI=0   --------------> Text(LineArt) Scan Mode

DPI=1        --------------> Resolution = 300 dpi

DOCTYPE=0    --------------> Document Type = Black & White

ORIENT=4     --------------> Orientation = Landscape(L to R)

1.3 Switch scanner to TEXT scan mode, 300 DPI.

2. ABOUT GO-CR V1.3 & V2.1

~~~~~~~~~~~~~~~~~~~~~~~~~~

Merge function is not supported for this application.