Introduction

Welcome to CacheCDFS, a CD-Rom control solution for the

Amiga family of computers. The CacheCDFS package consists of

a high-performance FileSystem, an emulation package that

allows you to play most of the popular CD-Rom games designed

for the AmigaCD32 game console and other support programs to

further control your CD-Rom drive.

In this package you will find: this manual and a 3.5 inch master

disk. You are strongly advised to fill in the registration card at the

end of this manual and send it to us to achieve technical support

and updates.

What is a FileSystem?

Data is written to a storage media in a precise manner. With

Amiga hard drives, this data layout is called FastFileSystem.

Data on CD-Roms is stored differently. This is why you need a

different filesystem. In simple terms, the FileSystem is a "translator"

between the physical layout of a disc and AmigaDOS.

It must be noted, that a FileSystem only provides access to data.

No processing or converting of the actual data takes place. This

job is left to other applications. A good example for such an

application would be a PhotoCD reader (like PhotoWorx from

Corporate Media).

The FileSystem gives AmigaDOS access to the physical disc layout,

PhotoWorx uses AmigaDOS to read the images and displays

them.

There are different common ways the data on a CD-Rom may be

organized:

1. HighSierra

High Sierra was the first attempt to define a standard data layout

for CD-Roms. It has been quickly replaced by its successor, the

ISO9660 standard.

Today this format can be considered obsolete.

The CacheCDFS does NOT support the High Sierra protocol. High

Sierra formatted disks can't be read by the CacheCDFS.

However, if there is enough public demand High Sierra might find

its way into the FileSystem in a future release.

2. ISO9660, Level 1

This is the most popular data layout for CD-Roms and can be

found mostly in the MS-Dos world. The CacheCDFS is able to read

such discs.

3. ISO9660, Level 2

This is an extension to ISO9660, Level 1. It allows longer filenames. It

is the most popular format for dedicated Amiga CD-Roms. The

CacheCDFS is able to read such discs.

4. Rock Ridge

Rock Ridge is a further extension to ISO9660. The "trick" with Rock

Ridge is, that an ISO9660, Level 1 formatted disc, readable with

MS-DOS machines, "magically" gets extended file names on more

"sophisticated" machines, like your Amiga. The CacheCDFS is able

to process Rock Ridge extensions correctly.

5. Macintosh HFS

As the name implies, this format can only be found on Apple

Macintosh machines. It is the same layout as used on Macintosh

harddisks. The CacheCDFS will try its best to read such disks.

System Requirements

This package requires the following to operate:

-	a Commodore Amiga computer

-	AmigaDOS 2.04 or greater

-	a CD-Rom drive with controller

-	at least 1 MegaByte of memory

-	any ISO9660, RockRidge or HFS formatted CD-Rom discs

The CD32 Emulator requires the following to operate:

-	a Commodore Amiga computer with AA/AGA chipset and

an MC68EC020 or better processor (A1200/A4000)

-	AmigaDOS 3.0 or greater (for better compatibility with CD32

games, AmigaDOS 3.1 is recommended)

-	a SCSI-2 compatible double-speed CD-Rom drive (Some

games will work on single speed drives, too) or a

TandemCD/CD1200 from bsc.

-	at least 2 MegaByte of chip memory and 1 MegaByte of 32-

bit fast memory.

-	an AmigaCD32 game on compact disc.

Backing-Up the CacheCDFS Master Disk

It is always advisable to make a backup copy of your master disk.

Since CacheCDFS is not copy-protected, the Commodore

diskcopy utility will suffice for this purpose. If you are unfamiliar with

this program, consult the AmigaDOS manual for further

instructions.

Important: The backup will usually be called "Copy_of_InstallCDFS"

After making the backup, you MUST rename the copy to

"InstallCDFS", otherwise the installation utility will not work with the

copy.

By not installing a copy-protection method on the master disk, we

are relying on your honesty to curb piracy. We want you to spread

the word about CacheCDFS, not the disk!

Installation

For a painless installation procedure, CacheCDFS utilizes the

Commodore Installer utility.

Important Note: Before starting to install CacheCDFS, your CD-

Rom drive MUST be connected to your Amiga and must be

operational (switched on), otherwise the installation will not work!

A step by step guide through the installation

Important Note: Read this carefully while doing the first time

installation!

Double-click on the Install-CDFS icon to invoke the installation

progress. The Install-CDFS window will appear.

Here you should click Proceed with Install.

Now you are asked for Installation Options. Simply click Proceed.

Now the FindCD program is launched to help you selecting the

correct device and unit for the CD-Rom drive you wish to install.

You may manually select the device driver by clicking its name, or

you may click the Scan gadget. If you click Scan, FindCD will

search for any CD-Rom drive connected to your machine. If

FindCD finds a CD-Rom drive, it will be displayed.

Note for users with more than one CD-Rom drive: If more than one

CD-Rom drive is found, you have to select the one you wish to

install. To install multiple CD-Rom drives or CD-Rom changers, you

simply have to start the Installation utility again and again, until all

of your drives are installed.

After you selected the drive to install, click Use.

Note for TandemCD users: If you are using the Tandem/CD1200

package from bsc, the device driver will now automatically be

updated, if necessary.

If everything went well, you are now presented with the Filesystem

-> Controller Configuration window. If not, FindCD was not able to

locate your CD-Rom drive. In this the case, you'll have to check

your hardware installation.

Now you have to configurate CacheCDFS to your CD-Rom or SCSI

hostadapter that you are going to use. The installation utility will

present you reasonable defaults that should always work. You

may click on Help to get a detailed explanation about the

different options.

Here are some example configurations:

TandemCD/CD1200:

Use Diskchange Interrupt

A2091/Hardframe:

Use SCSI Direct

Use 24 Bit-DMA

A3000/A4091/FastLane/ALF2/Supra/Emplant/Progressive

Peripherals/Golem/Oktagon500/others:

Use SCSI Direct

ALF3/Oktagon508/Oktagon2008:

Use Diskchange Interrupt

GVP Series2 (with FaaaastRom driver):

Use Diskchange Interrupt

Use Motor off

If you are done, click Proceed.

Now the Installation utility asks you, if you would like the CD-Rom

drive automatically mounted upon bootup. Usually you should

select Yes.

Important Note: If you are not sure if your CD-Rom or hostadaptor

works with CacheCDFS, it was wise to select No. Otherwise you

might not be able to boot your Amiga, as the machine might

freeze or crash while trying to activate your CD-Rom! If you are in

doubt, select No! After testing the proper operation of

CacheCDFS with your hostadaptor and CD-Rom, you can simply

start the installation utility again and select Yes at this point of the

installation procedure.

Now you are asked which AmigaDOS device name your CD-Rom

should get. The default is CD0. You may insert any valid

AmigaDOS device name if you like. Be sure that you choose a

unique name. Do not enter trailing colons, eg. enter CD0 and NOT

CD0:!

Note for users with more than one CD-Rom drive: Your CD-Roms

must be named differently, of course!

After you chose the name, click Proceed.

Now you should specify where the installation utility shall place the

CacheCDFS support programs, like the JukeBox audio player or

the CD32-Emulator. It is advised that you create a new drawer for

the CacheCDFS stuff on your harddrive by clicking Make New

Drawer. If you are not running the installation utility for the first

time, you should simply select the drawer created from the last

installation procedure. Than click Proceed.

Should the CDFSprefs utility be installed?

With CDFSprefs you can easily change the operational

parameters of CacheCDFS "on the fly". You really should click Yes.

The CDFSprefs utility is installed in your SYS:Prefs directory, as it is a

classic "Preferences" program!

Should the KillDev utility be installed?

With KillDev you can completely "unmount" a CD-Rom drive after

it has been mounted. This is very handy if you plan to use more

than one CD-Rom filesystem (e.g. the Xetec CDx FileSystem for

CDTV emulation).

Should the FindCD utility be installed?

You already used FindCD during the installation. It is of no real use

for normal operation. However, it might be handy for diagnostic

purposes.

Should the PlayCD utility be installed?

PlayCD is small but powerful Audio Player for SCSI-2, Mitsumi and

ATAPI drives.  The documentation is accompanied with the

program and will be copied, too. Please refer to the

documentation that comes with it.

Should the JukeBox Audio Player be installed?

JukeBox is a very comfortable Audio Player, written by Franz-Josef

Reichert. Included with CacheCDFS is the free distributable

version of this wonderful program. The documentation is

accompanied with the program and will be copied, too. An

enhanced version of JukeBox is available directly from the author.

Please refer to the documentation that comes with it.

Should the CD32-Emulator be installed?

This question will only appear, if you are running Kickstart /

Workbench version 3.0 or better on your machine, as the CD32-

Emulator will not work without it!

Should the Installer utility be installed?

This question will only appear, if you installed the CD32-Emulator.

The CD32-Emulator needs the Installer utility. If you have the

Installer utility already somewhere on your harddisk, you may

select No, otherwise you should select Yes.

Now the installation of CacheCDFS (and the CD32-Emulator) has

been completed.

Using CacheCDFS

Activation

CacheCDFS can be activated in a number of ways depending

on your version of AmigaDOS and whether you specified that

CacheCDFS be automounted in the Installation utility.

If you have AmigaDOS 2.04 installed on your Amiga, you will need

to use the mount command as follows:

mount <DEVICE> from devs:MountList.<DEVICE>

where <DEVICE> represents the appropriate device name as

specified in the install procedure, usually CD0.

For example, to manually activate CacheCDFS installed as CD0:,

you would have to enter the following command:

mount CD0: from devs:MountList.CD0

If you chose AutoMount during installation, this command will be

inserted into your s:user-startup file and automatically executed

upon bootup.

With AmigaDOS 2.1 and above, Commodore has introduced the

concept of Storage and DOSDrivers directories. In this case, a file

needs to be created in either the SYS:Storage/DosDrivers directory or

the DEVS:DosDrivers directory. This file will contain a conventional

mountlist, with the mounted device assuming the filename. All files

in the DEVS:DosDrivers directory are automatically mounted upon

startup, while files in the SYS:Storage/DosDrivers directoy are not. Both

directories are searched when a mount command is issued.

For example, if you want to mount CacheCDFS installed as CD0:,

you would enter the following command:

mount CD0:

Further you can activate CacheCDFS without entering the mount

command from the shell by double clicking the device icon in

either the DEVS:DosDrivers or SYS:Storage/DosDrivers directory.

Use of AmigaDOS Commands

The comfort of a FileSystem is that its operation is totally

transparent to the user. All AmigaDOS commands, such as dir,

copy, list work exactly as if they were executed on a floppy or

hard drive. You may use your favourite directory utilities such as

SID, DirWorks or Directory Opus with CacheCDFS. This can simplify

the traversal of the disc, especially where Macintosh HFS discs are

concerned, as many non-standard characters are present in HFS

filenames.

Due to the read-only nature of CD-Rom discs, it is impossible to

write on the disc. Therefore, AmigaDOS will regard the CD-Rom

disc as a large, write-protected volume. All attempts to write to

the disc will result in a standard "write-protected" requester, which

has to be cancelled.

Using CacheCDFS with Macintosh HFS

The Macintosh HFS file system is a bit tricky to use. Any HFS file

consists of two "forks" - a data and a resource fork. Unlike other

filesystems CacheCDFS will always show you the data and

resource fork as two different files. To distinguish these two files you

may add an extension to either the resource filename, the data

filename or both with the CDFSprefs program.

Some HFS filenames contain a lot of spaces. This makes typing

their names from the shell a tedious task. The "convert spaces"

option of CDFSprefs will convert spaces in HFS filenames to

underscores.

(See also the chapter "Setting CacheCDFS Preferences")

Theory of Cache Operation

Before continuing, there are a few things about cache

organization that you must know. In order to make cache access

as fast and efficient as possible, the main cache "buffer" must be

organized in a special way. We talk about "lines", "prefetch", "data

buffer" and "minimal direct read".

These values all relate very closely to sector numbers. All devices

that are used to store files on have sectors. One sector is a tiny bit

of information - in case of a CDROM - 2048  bytes in size.

All devices have a controlling task (a running program) called a

device driver, whose primary task it is to read and write these

sectors.

This might be the scsi.device for an A3000 with a SCSI-CD Rom

drive, or the tandemcd.device for the wonderful TandemCD

package from bsc. If a file system (like CacheCDFS) wishes to

read sectors off a drive, it must ask the device driver to fetch

those sectors. This device driver is sometimes called an "Exec"

device driver.

The file system is a kind of protocol that describes how data is

organized on the physical device. The CacheCDFS can read

ISO9660, RockRidge and MAC HFS formatted disks.

As the filesystem is a kind of interface between the exec device

driver controlling the hardware and AmigaDOS, it is sometimes

called a "DOS" device driver.

The term "prefetch" is an indication of how many sectors the

cache will read or write for each physical device access. If

prefetch is set to 4, then this means, that even if the DOS only asks

for two sectors, four will be read. Since each drive access is a

lengthy and time consuming procedure, much time can be

saved by "guessing" what sectors will be needed next. Since the

best guess is often that "the next sector will be needed next",

prefetching sectors in clusters is indeed a good thing. Keep in

mind that it only takes a bit longer to read four sectors than to

read one. It would take much longer to read four sectors

individually than to read them all in one go.

Furthermore, as the FileSystem really knows how large a file

actually is and how the disc is organized, those guesses have a

very good chance to be correct.

Prefetch, or "read-ahead" if you want, can boost performance a

great deal, providing you set the prefetch value not too big. If

you use too much prefetch, it will not help you one bit. In fact, it

will probably work against you, slowing everything down. This is

because the next sector in sequence probably is NOT the next

sector that will be needed, and so the cache will be reading

much more than it should, on each access.

The morale is to use prefetch with reason. Generally a prefetch of

2 or 4 gave very good results with ISO9660 formatted disks.

Moving on to the term "lines". Lines is a means of grouping

together sectors  in "blocks" of a certain length. If a cache buffer

uses 32 sets of lines, this means that the cache can remember up

to 32 different entries, with each entry consisting of 'prefetch'

number of sectors. To illustrate this, consider the following sketch

of a 32 line cache, with a prefetch of 4:

Set number       Line contents

0                [sector][sector][sector][sector] (4*2048=8k)

1                [sector][sector][sector][sector] (4*2048=8k)

2                [sector][sector][sector][sector] (4*2048=8k)

3                [sector][sector][sector][sector] (4*2048=8k)

4                [sector][sector][sector][sector] (4*2048=8k)

etc....

31               [sector][sector][sector][sector] (4*2048=8k)

32*8k=256k

Each line can hold any four consequtive disk sectors with this

prefetch configuration. If the file system asks for sector number 8,

the cache will read sectors 8, 9, 10 and 11 into one of the lines.

By now it must be obvious that more lines also mean more space

available for sector data, and therefore a higher chance that a

read operation will be able to complete with no or only little

physical disk access. Unfortunately, it also means you have to set

aside more memory for the buffers. A compromise you often

encounter with computers.

A typical cache configuration is a 50-set, cache, with a prefetch

(line size) of 4. The memory required by such a cache buffer will

be:

50 sets x (4 x 2048 bytes) = 400k bytes

The CacheCDFS has also a special buffer, called "data buffer".

This buffer has always the multiple size of one line.

If the line size is 4, and the data buffer has a size of 4, it is

organized like this:

[line][line][line][line] (4*4*2048=32k)

If the FileSystem does caching (it does not always, see the term

"direct read" later), it checks the actual file length. In case the file

is longer as the amount actually requested, the rest is read

immediatly into the data buffer, and then copied to the

appropriate cache line.

The main difference between the Data Buffer "read ahead" and

the normal "read ahead" is as follows:

The cache lines are always filled (a "classic" read ahead cache),

so even different directory sectors or sectors from different files are

cached. The data buffer is a more "intelligent" special read-

ahead cache line, existing only once and caching only files, if the

filesystem "believes" they are worth it.

So you can do nice caching even on machines with not much

free memory. Look at this (which is the default setting after first

time installation):

50 sets with a line size of 1 (no "directory" prefetch!), but a data

buffer of 8 (8 sectors "file" prefetch).

50 sets x (1 x 2048 bytes)

= 100k bytes

+ 8 x ( 1 x 2048 bytes) (data cache)

= 116k bytes

This gives pretty good results, even on a machine with not much

memory available.

Now moving to the term "direct read". If you read large files (e.g.

"off the disk animations") it is pretty useless to cache all these

blocks. The file will purge all other sectors out of the cache, even

such blocks like directory entries. To avoid this, you can specify a

"minimum direct read" value. If DOS requests more sectors than

specified here, the filesystem will bypass the cache and the

device driver will copy its data directly into the callers buffer (if

any of these sectors are already present in cache memory, these

will be used).

Specifying a value of zero (0) will give you a sophisticated default.

A very high value (e.g. 2000) will only do requests greater than

4MB directly, what is very unlikely to happen and the cache will

always be used.

Specifying a value smaller or equal to the data buffer size is not

very useful, as you would in fact waste memory.

For best performance, use the default (0).

Before doing direct read, the Memory Type and Memory Mask of

the callers buffer will be checked, if it fits to the device

requirements (specified in the mountlist as BufMemType and

Mask). If it doesn't, the data buffer will be used as a cache.

It's simply not possible to give you a recipy on how to create a

perfect cache. The range of possible combinations of all the

above values and the memory you wish to spend for caching is

simply too vast.

You will have to experiment to find the combination that best suit

your needs.

You can adjust all these values in the filesystem's mountlist entry

using a text editor, but this method is strongly discouraged - using

the CDFSprefs program for this purpose is much easier and secure.

Setting CacheCDFS Preferences

All options for CacheCDFS can be set via an Intuition-based

preferences editor. This will eliminate the need to change cryptic

mountlist entries.

The CDFSprefs program is located in your SYS:Prefs drawer and can

be activated by double-clicking its icon.

Important Note: To edit the preferences for a particular CD-Rom

drive, the drive must be activated (mounted)!

The window of the CDFSprefs program is splitted into 3 sections:

-	The upper half of the window defines overall preferences

-	The lower left part are ISO9660 preferences

-	The lower right part are Macintosh HFS preferences

Simply adjust the values as you like, and click the SAVE gadget.

The changes will be stored in your mountlist entry and the

filesystem will be informed of these changes immediately.

There is no need to reboot for making the changes active!

The USE gadget will perform the changes, but will not store them

in the mountlist entry. This is handy if you want to experiment with

different cache settings.

GADGETS

DEVICE:

Select the CDROM you want to edit. If only one CDROM is

mounted, it will be selected automatically.

BUFFERS:

Same is "Buffers" entry in mountfile. Sets the number of cache lines.

The default is 50.

DATA CACHE:

Same as DC in control entry in mountfile. Sets the size of the Data

Cache. The default is 8. Note that the size of the Data Cache is

defined in "multiples of the Line Cache size".

LINE CACHE:

Same as LC in control entry in mountfile. Sets the size of the Line

Cache (prefetch). Default is 1 - this means that prefetch is off.

MIN DIRECT:

Same as MD in control entry in mountfile. Specifies the amount of

sectors a read request must have to bypass the caching system.

Default is 0 (automatic).

RAM (Read only gadget):

Shows you the total amount of RAM use of your buffer settings in

kBytes.

PRO BITS:

This will specify the protection bits all files on a CD-Rom should

have, as the SetProtection command of AmigaDOS will not work

because of the read-only nature of CD-Rom media.

SCAN FIRST FOR:

Select, if the FileSystem should look first for an ISO9660 or a MAC-

HFS identification on a CD inserted. This is useful, if you have "Dual-

system" discs with both an ISO9660 and a Macintosh HFS

"partition".

SCSI DIRECT:

Same as S in control entry in mountfile. Uses SCSI direct calls

instead of trackdisk like commands. Default has been set by the

Installation utility.

NO CHANGEINT:

Same as NC in control entry in mountfile. If set, CacheCDFS will

not use TD_ADDCHANGEINT and TD_REMCHANGEINT. Default has

been set by the Installation utility.

MOTOR OFF:

Same as M in control entry in mountfile. Will send a TD_MOTOR off

after CMD_READ. Default has been set by the Installation utility.

FILES TO LOWERCASE:

Same as L in control entry in mountfile. All file names will be

converted to lowercase. This option only applies to ISO9660 discs.

VOLUMES TO LOWERCASE:

Same as LV in control entry in mountfile. All Volume names will be

converted to lowercase. This option only applies to ISO9660 discs.

CONVERT FIRST CHAR:

Same as LFC in control entry in mountfile. This option only applies

to ISO9660 discs and specifies the behaviour of FILES TO

LOWERCASE and VOLUMES TO LOWERCASE.

If selected, all characters in names will be converted to

lowercase. If not, the first character will be untouched.

AUTO LOWER

Same as AL in control entry in mountfile.This flag will advise the

FileSystem, that only file and volume names of non-Amiga CDs

should be converted. CacheCDFS will try its best to determine, if

mixed case file names exist on a CD. However, this may not

always work.

NO ROCKRIDGE NAMES

Same as NRR in control entry in mountfile. This flag will disable

RockRidge filenames. If it is selected, only ISO file names will be

shown.

ROCK RIDGE CASE SENSITIVE

Same as RRCASE in control entry in mountfile.

RockRidge file names are defined to be case sensitve, e.g. the file

"Olli" is different from the file "oLLI". With Amiga DOS these two

filenames describe the same file. If this flag is set, the CacheCDFS

will handle RockRidge file names case sensitive. This may be

usefule, if you want to read a RockRidge CD where different files

can only be distinguished by the case of their file names. Weird.

CONVERT SPACES:

Same as HCS in control entry in mountfile. This option only applies

to MAC-HFS discs. Will convert spaces in MAC-HFS filenames to

underscores, if selected.

RESOURCE:

Same as HR in control entry in mountfile. This option only applies to

MAC-HFS discs. Defines the name extension of the resource fork of

MAC-HFS files.

DATA:

Same as HD in control entry in mountfile. This option only applies to

MAC-HFS discs. Defines the name extension of the data fork of

MAC-HFS files.

SAVE:

Make changes active and save them to mountfile.

USE:

Make changes active.

CANCEL:

Exit without making changes.

MENUS

PROJECT - ABOUT:

Gives you informations about the CDFSprefs program and the

CacheCDFS in use.

PROJECT - QUIT:

Leaves the program.

EDIT - RESET TO DEFAULTS:

Get the default presets.

EDIT - LAST SAVED:

Get the values saved in the mountfile.

EDIT - RESTORE:

Get the current active values.

AUDIO - ON

Check this Menu Item, if you want the CacheCDFS to display an

Icon for Audio CDs on the Workbench screen.

AUDIO - EXTERNAL AUDIO PLAYER

Check this Menu Item, if you want the CacheCDFS to start an

external audio play program, if you double click the Audio Icon.

AUDIO - PATH

Here you can choose the external Audio player to be started by

CacheCDFS. We recommend to use the PlayCD program

(included in this package) for this purpose.

Playing Audio CDs

To play audio tracks on a CD you can either use the capabilities

embedded in the CacheCDFS filesystem, or you may use an

external audio player program.

Embedded Audio Capabilities

If you have enabled Audio support for the CacheCDFS file system

with the CDFSprefs program and if you have a SCSI-2 compatible

CD-Rom drive the filesystem will show an audio icon on your

Workbench, if a CD with audio tracks is inserted.

The first track will be played, if you double click this icon.

If you double click again, audio play will pause.

If you double click again, audio play will resume.

If you double - double click the icon (yes, this is four times!) the

next track will be played.

If you have defined the use of an external audio player program

with CDFSprefs, double clicking the audio icon will start this

external player.

With this package three different CD audio player programs are

included; for use with this feature we recommend Elaborate

Bytes' "PlayCD".

Defining a custom Audio Player Icon

You may use an own icon that will be displayed if a CD

containing audio tracks is inserted.

CacheCDFS will look for the file ENV:CDDA.info if it is started.

If this file exists, it will be used instead of the built in design.

Let us try this: Copy the file Trashcan.info to ENVARC:CDDA.info

and reboot your machine. If you insert an Audio CD, the Trashcan

icon will show up.

Introduction to the CD32-Emulator

The CD32-Emulator is a seperate part of the CacheCDFS

package. If you have at least Kickstart 3.0 and if you have

installed the CD32-Emulator with the Installation utility, you will find

a drawer named "CD32" on your harddrive.

The CD32-Emulator tries to emulate all of the special system

resources that can be found in an Amiga-CD32 game console.

This includes the emulation of the battery backed up RAM for

storing Highscores and the setting of the preferred user language.

Embedded in the CD32-Emulator is a system degrader which will

disable features of your Amiga - like processor caches or fast

memory - to make your machine more "CD32" like, as some badly

programmed games need this. You can control this "degration"

easily from the CD32-Emulator's preference window and save

your preferred settings.

Great effort has been taken to make the CD32-Emulator as

compatible as possible, but the usual restrictions that can be

found with any kind of emulation apply:

We cannot guarantee that all games that work on an Amiga CD32

game console will work with the CD32-Emulator!

The Amiga CD32 has an in-built CDTV emulation. The CD32-

Emulator has NOT!

CDTV titles WILL NOT WORK with the CD32-Emulator, even if they

work with the Amiga CD32 console!

Emulating a PAL/NTSC console

On a PAL Amiga, the CD32-Emulator will emulate a PAL CD32, on

an NTSC Amiga the CD32-Emulator will emulate an NTSC CD32.

You may change the PAL/NTSC state of your Amiga from the

Kickstart Boot Menu. This will change the type of the CD32

emulated (PAL/NTSC), too.

Installing the CD32-Emulator

The CD32-Emulator may be used immediately from your

harddrive. But it is very unlikely that many games will run correctly

if you try this.

The preferred method is to create a dedicated "CD32-Emulator

Boot Disk" and start the emulation from this disk. The "Make CD32

Floppy" utility will create such a disk for you.

Another utility is the "Make HD Startup" utility. It will create a new

Startup-Sequence on your harddisk, that allows you to boot

directly into CD32-Emulation, if you press a function key during

startup. Your originial startup-sequence will be renamed to

"Startup-Sequence.std" and will be executed if you boot without

pressing the dedicated function key. This method is faster than

booting from floppy, but it is advised that only experienced users

try this option. Your original startup-sequence will be changed

and there is a slight possibility that this may cause problems!

Important Note: The CacheCDFS must already be installed and

the CD-Rom drive you wish to use MUST be mounted! The cache

settings should be set to its default values and SAVEd before

starting the "Make CD32 Floppy" utility!

Double-click the "Make CD32 Floppy" icon and insert an empty

disk into your internal drive (DF0:). The disk doesn't need to be

already formatted as the program allows you to format the disk if

you wish. All data on this disk will be erased!

After the installation is completed you should wait a few seconds

for all disk activity to finish. Now you may reboot your Amiga and

your machine will boot from the created CD32-Emulation disk and

the CD32-Emulator preferences window will appear.

Using the CD32-Emulator

The CD32-Emulator window offers you some detailed control

possibilites of the emulation's behavior.

Two gadgets exist to activate CD32 emulation: Boot and Initialize.

Boot will set up the emulation and will immediately start any

Games CD that is inserted in your CD-Rom drive CD0:

Important Note: Clicking Boot is a one-way-ticket! Once the boot

process from a Games CD has been started, there is no way to

resume normal Amiga operations. The only way to back out is to

reset your machine! This is important to know if you have started

the CD32-Emulator from your harddrive!

Initialize will NOT start the Games CD, but will initialize all of the

emulation modules embedded in the CD32-Emulator. Its purpose

is for games that can be started from Workbench.

Important Note:

To get the most fun out of CD32 games, a CD32 game controller is

recommended. Please note that the originial Commodore

controller may not work correctly with an A1200! The Competition

PRO SuperCD32 controller works with the A1200. Both controllers

work with the A4000.

Setting the CD32-Emulator preferences

Gadgets

NoFastmem

Enabling NoFastMem will disable all Fast Memory in your Amiga.

You should only enable this switch, if the game you want to play

requires this. If the graphic and/or sound of a game is corrupted,

try this option.

NoInstCache

This will disable the instruction cache of the processor. This option

should rarely be neccessary. If you encounter that a game is

running too fast, you should try this option.

NoDataCache

This will disable the data cache of the processor. If you encounter

that a game is running too fast, you should try this option.

VectorBase0

This option is only neccessary if you started the game from

Workbench. Some games don't work at all if this option isn't

enabled.

NoVolumeControl

This will disable the attenuation control of the CD32-Emulator. If

the digital audio produced by a game CD cannot be heard or

the game "hangs", try enabling this flag.

Emulate Controller

If this is enabled, the CD32 Game Controller will be emulated with

the keyboard. You shouldn't select this, if you have a CD32 Game

Controller connected to your Amiga. You may connect a one or

two button joystick and use the keyboard to access the

additional keys. Emulation keys are:

Numbers 1, 2, 3, 4, 6, 7, 8, 9 on numeric Keypad and cursor keys:

Joypad direction.

[ and / on numeric keypad: Reverse.

] and * on numeric keypad: Forward.

F1 and F6: Green button.

F2 and F7: Yellow button.

F3, F8 ,SPACE , ENTER and 0 on numeric keypad: Red button.

F4, F9 and DEL on numeric keypad: Blue button.

F5 and F10: Play/Pause button.

Note: It is possible that some games do not work if controller

emulation is enabled!

Use 68040.library

Most games will not work if Commodores 68040.library is used. But

if it is possible to use this library, system performance will be greatly

increased. This only applies to machines with 68040 processors, of

course.

Load Workbench

Some games will probably not run correctly if there is no

Workbench process running. We know only of the game "Morph"

which requires this flag to be selected. Usually you should leave

this off.

DOS Device

Here you define the CDRom drive, where the CD32 emulation

should happen.

Blocks Buffer

The CD32-Emulator has a built in prefetch cache. Here you can

set the size of this cache (1 Block equals 2048 Bytes). The optimum

value differs depending on your configuration (Amiga model,

hostadapter, CD-Rom drive). You have to experiment to find a

good value. The rule is: Keep the value as small as possible, but

big enough, that off-the-disk-animations run smoothly. For

TandemCD/CD1200 the optimum value is 4, most SCSI drives work

best with a value of 2.

Delay p. Block

If you specify a value other than zero, a delay will be added

between reading different sectors

This is important, if you have a CD-Rom drive, which is faster than

the original CD32.

If you have a single or double speed drive, leave this value to

zero. If you have a faster (triple, - quad speed) drive, you should

enter a value between 50 and 100 here. You must experiment for

best results, as the correct timing will be affected by your system

performance.

Max Volume

Here you can specify the maximum level the CD sound should

have during gameplay.

Language

Here you can select your preferred language for multi-language

games.

Menus

Project - Save Settings

This allows you to save your CD32-Emulator prefences. The settings

are stored as ToolTypes in the program's icon file.

Project - About

Tells you about the version and copyright of the CD32-Emulator.

Project - Quit

Will leave the program.

Game Store - Select Path

Some CD32 games allow you to save highscores or other data in

the battery backed up RAM of the Amiga CD32 game console for

later retrival.

The CD32-Emulator allows you to save this data to a dedicated

place either on floppy disk or your harddisk.

If you select this menu item, a standard ASL requester will appear.

Choose the directory where you want the CD32-Emulator to save

the data. Every Game saving data will get its own drawer. You

may easily manipulate (copy, delete, etc) files in these drawers

from the Workbench.

Remember: After adjusting your preferred settings, you should

select Save Settings.

You should ONLY degarde your system, if the game you want to

play refuses to run otherwise. Most of the games run fine without

degration.

Some Example settings:

Microcosm,  Pirates Gold, D-Generation, Sleepwalker, Clou and

Frontier - Elite II:

All off.

Pinball Fantasies, James Pond II - Robocod:

All off, but NoVolumeControl might be required on some

hostadapters.

Diggers:

All off, but the intro sequence will probably run too fast.

For the correct speed of the intro sequence you should select

NoFastmem.

Oscar, Dangerous Streets:

NoFastMem

NoDataCache

VBR0 (only if started from Workbench on 68040 machines)

Morph

NoFastMem

Load Workbench

Running the CD32-Emulator in the Background

You may specify the ToolTypes NoWinInst or NoWinBoot in the

CD32-Emulator's icon file.

NoWinInst will immediately install the CD32-Emulator's modules

without openening its preferences window.

If you wish to have the CD32 Emulation present all the time, you

may specify NoWinInst and place the CD32-Emulator in your

SYS:WBStartUp drawer.

NoWinBoot is very similar, but it will immediately try to boot from

any disc present in CD0:.

Questions & Answers - Troubleshooting

Q: The AmigaCD32 only has 2 MegaByte chip memory. Why do

some games only work on my Amiga 1200 if I add 32-bit-wide fast

memory?

A: CacheCDFS and the CD32-Emulator are loaded from disk.. This

will use the memory a game probably needs. If an Amiga1200

doesn't have fast memory, the CD32-Emulator will be loaded in

chip memory. As chip memory is slower than fast memory the

speed of the emulation will probably be to slow for some games.

Q: A game doesn't run on a plain A1200, but it works if I add fast

memory. But I have to select NoFastMem in the emulator's

preference window! Isn't that weird?

A: No, it isn't. The trick behind NoFastMem of the CD32-Emulator is,

that CacheCDFS and the CD32-Emulator are loaded and

initialized BEFORE the fast memory will be disabled.

This leaves all the chip memory for the game to run.

Q: I have a NEC-SCSI-CD-Rom drive. It works fine with CacheCDFS.

But the CD32-Emulator doesn't work correctly. Why?

A.: CacheCDFS works with any kind of CD-Rom drive, because it

only needs to access plain data. The CD32-Emulator needs to

access Audio playback functions and other more sophisticated

stuff. The SCSI-2 standard defines the methods how this should be

done and the CD32-Emulator uses SCSI-2 commands to talk to the

drive. Only drives conforming to SCSI-2 will work correctly with the

CD32-Emulator. Unfortunately most of the NEC drives do not

comply to SCSI-2 and therefore won't work, sorry.

Q: What drives do work with the CD32-Emulator?

A: All SCSI-2 drives will work, e.g. Toshiba, Sony and Apple drives.

To get full functionality, they should be "Double Speed".

Q: May I use another filesystem with the CD32-Emulator?

A: Please don't. The results are unpredictable.

Q: Why didn't you add CDTV Emulation?

A: Some CDTV Titles don't run with other processors than the

68000. Most CDTV Titles don't work with Kickstart 2.0 or better. A lot

of CDTV titles don't work properly if the AA/AGA chipset is

enabled. The Amiga CD32 has built-in "compatibility hacks", that

make even those titles run on the console. Without these hacks

only a few titles would work, and this is simply not worth the effort.