INTRODUCTION

This file describes how to modify a Mouse Systems M3 serial mouse

so that it will directly replace the normal Amiga mouse.  Although this

requires a little tinkering with the mouse, I think it is preferable because

it it TOTALLY compatible with ALL software and does not tie up a serial port.

Of course, I discovered this AFTER writing the OptMouse serial mouse program.

Ed Hanway

149 Arborwood Crescent

Rochester, NY 14615

BIX, GEnie, PLink: jehanway

Usenet: uunet!sisd!jeh or jeh@sisd.kodak.com

DISCLAIMER

I take no responsibility for any damage the following procedure may cause

to either the mouse or the computer.  The procedure will probably not work

for other Mouse Systems mice, and may not even work with all revisions of

the M3.  The risk is entirely yours.

Finally, don't get in over your head.  The procedure is simple enough that

you don't have to be a Ph.D., but if you've never used a soldering iron

before, find someone who has.

MODIFYING AN M3 SERIAL MOUSE TO WORK ON AN AMIGA

Inside my Mouse Systems M3 serial mouse, on the circuit board is a row of

10 holes with pin 1 closer to the center of the mouse. Wired as follows,

it works perfectly on my Amiga:

Mouse	Amiga

1 ----- 9

2 ----- 5

3 ----- 6

4 ----- 1

5 ----- 3

6 ----- 4

7 ----- 8

8 ----- 7

9 ----- 2

10 NC

All you need is a length of nine-conductor wire, a DB-9S connector, and

minimal soldering skills and you're set.

MODIFYING OTHER MICE

If you have the newer M4 model serial mouse, you're not so lucky.  The

convenient row of holes is missing and practically the entire circuit is

replaced by one custom chip.

It is still possible, although unlikely, that the proper signals exist

somewhere inside other mice, and if you probe around the innards of the

mouse with a voltmeter, you might find them.  Here's what you'll need to

hook up:

Amiga

Pin # Description

----- -----------

1     V (Vertical Pulses)

2     H (Horizontal Pulses)

3     VQ (Vertical Quadrature Pulses)

4     HQ (Horizontal Quadrature Pulses)

See below

5     UBUT* (Middle Button -- Optional)

All buttons are active low.  When the switch is closed (button

depressed) the signal should be grounded.

The middle button can be left unconnected, but if you can find the

signals for the other two buttons, you should be able to find this

one, too.

6     LBUT* (Left Button)

Active low.

7     +5V

Power to the mouse from the Amiga.

8     GROUND

9     RBUT* (Right Button)

Active low.

Identifying the movement pulses:

Once you identify the +5V and GROUND lines, you can hook them up and look for

candidates for the movement pulses. These are all logic-level signals (roughly

+5 or ground) that should flip back and forth as you move the mouse on its pad.

As you (very slowly) move the mouse to the RIGHT, you should see the following

pattern

____      ____      ____

H  _/    \____/    \____/    \____

____      ____      ____

HQ ____/    \____/    \____/    \____

When you move to the LEFT, you should see

____      ____      ____

H  _/    \____/    \____/    \____

____      ____      ____      __

HQ     \____/    \____/    \____/

Similarly,

DOWN:

____      ____      ____

V  _/    \____/    \____/    \____

____      ____      ____

VQ ____/    \____/    \____/    \____

UP:

____      ____      ____

V  _/    \____/    \____/    \____

____      ____      ____      __

VQ     \____/    \____/    \____/

(Info from the Amiga Hardware Reference Manual)

Notes:

Serial Mice that draw their power from the computer are often wired with a

three wire cable (+5V, GROUND, and the serial data).  This should help in

identifying +5V and GROUND  (they should be connected to thicker traces on

the mouse PC board).